WD-104

Published by the

International Hydrographic Organization

4b quai Antoine 1er

Principauté de Monaco

Tel: (377) 93.10.81.00

Fax: (377) 93.10.81.40

info@iho.int

www.iho.int

Water Level Information for Surface Navigation

Product Specification

Edition 2.1.0 – October 2026

© Copyright International Hydrographic Organization 2026

This work is copyright. Apart from any use permitted in accordance with the Berne Convention for the Protection of Literary and Artistic Works (1886), and except in the circumstances described below, no part may be translated, reproduced by any process, adapted, communicated or commercially exploited without prior written permission from the International Hydrographic Organization (IHO). Copyright in some of the material in this publication may be owned by another party and permission for the translation and/or reproduction of that material must be obtained from the owner.

This document or partial material from this document may be translated, reproduced or distributed for general information, on no more than a cost recovery basis. Copies may not be sold or distributed for profit or gain without prior written agreement of the IHO Secretariat and any other copyright holders.

In the event that this document or partial material from this document is reproduced, translated or distributed under the terms described above, the following statements are to be included:

“Material from IHO publication [reference to extract: Title, Edition] is reproduced with the permission of the IHO Secretariat (Permission No ……./…) acting for the International Hydrographic Organization (IHO), which does not accept responsibility for the correctness of the material as reproduced: in case of doubt, the IHO’s authentic text shall prevail. The incorporation of material sourced from IHO shall not be construed as constituting an endorsement by IHO of this product.”

“This [document/publication] is a translation of IHO [document/publication] [name]. The IHO has not checked this translation and therefore takes no responsibility for its accuracy. In case of doubt the source version of [name] in [language] should be consulted.” The IHO Logo or other identifiers shall not be used in any derived product without prior written permission from the IHO Secretariat.

Document History

Changes to this Specification are coordinated by the Tides, Water Levels and Currents Working Group (TWCWG). New editions will be made available via the IHO web site. Maintenance of the Specification shall conform to IHO Resolution 2/2007 (as amended).

| Version Number | Date | Editor | Purpose |
| --- | --- | --- | --- |
| 0.0.0 | February 2015 | Z Jayaswal | Initial Draft |
| 0.0.1 | May 2016 | Z Jayaswal | TWCWG1 – Working group input incorporated from Brazil meeting. |
| 0.0.2 | August 2016 | Z Jayaswal | TWCWG – incorporate feedback on Portrayal and Attributes. |
| 0.0.3 | March 2017 | Z Jayaswal | Extract commonality from S-111 Ver. 0.1.10 to ensure consistency between standards. |
| 0.0.4 | May 2017 | Z Jayaswal | As edited during TWCWG2. |
| 0.0.5 | November 2017 | Z Jayaswal | Feedback from TWCWG and S-100WG. |
| 0.0.6 | September 2018 | Z Jayaswal | Feedback from TWCWG3 and S-129WG. |
| 0.0.7 | March 2019 | Z Jayaswal | Feedback from S100 Test Strategy Meeting Sep 2018. |
| 0.0.8 | April 2019 | Z Jayaswal, K Hess & G. Seroka | Feedback from NOAA and TWCWG4. |
| 0.0.9 | Aprril 2021 | Z Jayaswal, G. Seroka, R. Malyankar Review by TWCWG and S-104 project team | Feedback from TWCWG5. |
| 1.0.0 Draft 1 | June 2021 | R. Malyankar | Applied review comments on Ver. 0.0.9. Added carrier metadata attributes for coordinate systems approved by S-100WG. Added exchange set structure and use cases. |
| 1.0.0 Draft 2 | August 2021 | R. Malyankar. Review by ZJ, GS. | Applied comments on Draft 1. |
| 1.1.0-20220831 | August 2022 | R. Malyankar | Aligned with S-100 5.0.0; applied some of the held-over comments from Edition 1.0.0; new enumeration dictionary material |
| 1.1.0-20230131 | February 2023 | R. Malyankar, G. Seroka | Fill value for waterLevelHeight now has 2 zeroes after decimal point; specified datatype size for HDF5 attributes; clarification of attribute dimension in feature type metadata; updated references; updated rules for dataset and support file names; guidance on (optional) ISO metadata; adjusted language on use of Schematron and digital signatures; added emphasis for differences between S-100 and S-104 exchange catalogues; added productIdentifier metadata attribute; added Res. 3/1919 as a separate reference. Figures in Annex E removed. |
| 1.1.0 | April 2023 | R. Malyankar | Applied TWCWG review feedback; corrected S-100 error in digital signature types in metadata. |
| 2.0.0-20240131 | January 2024 | R. Malyankar | References updated; registry producer code URL replaced with main page URL; citation of S-100 WG7 paper removed and usage sentences updated (register is now active in the GI registry); multiple changes to reduce scope of S-104 reduced to regular grids per S-100 WG8; alignment with S-100 Edition 5.2; additional EPSG codes for horizontal reference systems; removed Annex B (Additional terms…); uncertainty attribute added; restricted maximum length of HDF5 string attributes (clause 12.3) |
| 2.0.0-20240208 | February 2024 | R. Malyankar | Added attribute verticalCoordinateBase (Table 12-1); sequencing rule must be linear (implicit in regular grid format); instance metadata constraints (Table 12-3) adjusted for WLA compatibility |
| 2.0.0 Draft 2 | May 2024 | R. Malyankar | Applied feedback from S-104 PT; provision for multiple vertical datums; added optional dataOffsetCode; updated data quality section for multiple vertical datums and DQWG review of Ed. 1.0.0; removed Annexes for validation checks, additional terms, and sample datasets; added terms for data quality and selections from deleted “Additional Terms” Annex; updated fileless cancellation; aligned with S-100 5.2.0; updated UML diagrams as per feedback and changes to metadata; expanded material on S-98 conformance; added EPSG codes for UTM zones and newer WGS84 epochs |
| 2.0.0 Draft 3 | July 2024 | R. Malyankar | Clarifications for multiple vertical datums; editorial changes in S-158 series references; further alignment with scope reduction to RD-only; clarification of CRS encoding and addition of new EPSG code for WGS84 G2296 |
| HSSC Draft 1 | September 2024 | R. Malyankar | Applied TWCWG review comments. Clarifications for datums, data quality, interpolation/extrapolation to fill gaps; added material for domain extent polygons; new metadata attribute for vertical datum epoch and guidance for epoch differences; updated language about optional attributes in Group_F; Group_F uom name “metres” changed to “metre” to conform to FC; locale constraints corrected (12.2.4). |
| MS Draft 1 | October 2024 | R. Malyankar | Applied HSSC comments (remove unused otherLocale (12.2.4); remove bilinear interpolation (12.3.2); updated S-98 and S-111 entries in references; updated PS date (1.7). |
| Edition 2.0.1 Draft 1 | February 2026 | G. Seroka | Added waterLevelTime to Feature Catalogue with remarks (Annex A, C) |
| WD 2.0.1 | July 2026 | R. Malyankar | Accumulated clarifications to Edition 2.0.0; aligned to S-100 5.2.1 |
| ED 2.0.1 | September 2026 | R. Malyankar | Apply S04 PT comments, Endorsement draft |
| WD 2.1.0 | October 2026 | R Malyankar | Working draft Ed. 2.1.0 |

Summary of Substantive Changes in Edition 2.1.0

Bold references in the Clauses Affected column indicate the principal sections/clauses that are affected by the described change.

| Change Summary | Clauses Affected |
| --- | --- |
| Substantive Changes in Edition 2.0.1 |  |
| Restore deleted attribute waterLevelTime to the Feature Catalogue, add remarks indicating waterLevelTime in the Feature Catalogue is for ECDIS use only and must not be present in the HDF5 dataset | Feature Catalogue, Annex A, Annex C |
| Relax specification of time threshold for trend calculation; clarify reporting of trend | 1.4.2, 7.1.1, |
| Remove EPSG 6498 as allowed vertical coordinate system | 5.2 |
| Only datums in the S-100 list or “National Low Water Datum” are permitted | 5.2, 12.3.1, 12.3.3, 12.3.5 |
| Scale ranges for datasets introduced | 7.7.3, |
| Construction of coverages clarified | 7.9 |
| Feature instances in the same data file must not overlap | 10.2.2 |
| Optional attributes included in Group_F only if they are actually present | 10.2.2.2 |
| Axis names fixed as latitude, longitude, northing, easting | 10.2.2.3, 10.2.2.6, 10.4 |
| Overrides of values clarified | 12.3, 12.3.4 |
| Positioning group not used in this edition of S-104 | 10.2.2.6 |
| Data types for coordinates | 10.3, 12.3.1, 12.3.2, 12.3.3, A.1 |
| Axis names and order | 10.4 |
| Size limit in terms of the number of value records | 11.1, 11.2.1 |
| File naming convention changed | 11.2.1, 11.2.3 |
| Adjustments to discovery metadata | 12.2.1, 12.2.4, 12.2.6 |
| Storage order clarification | 12.3.2 |
| Bounding box mandatory at instance level | 12.3.3, 10.2.2 |
| Substantive Changes in Edition 2.1.0 |  |
| Scale ranges and grid resolution | 7.10 (new), 7.7.3 |
| Bounding polygon guidance | 7.11 (new), 12.2.6 |
| Guidance for pick reports | 9 |

Contents	Page

1	Overview	1

1.1	Introduction	1

1.1.1	Data types	1

1.1.2	Display	1

1.1.3	Encoding	1

1.2	Scope	2

1.3	References	2

1.3.1	Normative	2

1.3.2	Informative	3

1.4	Terms, definitions and abbreviations	4

1.4.1	Terms and definitions	4

1.4.2	Abbreviations	7

1.4.3	Notation	8

1.5	Use of language	9

1.6	General data product description	9

1.7	Data Product Specification metadata	9

1.7.1	IHO Product Specification maintenance	10

2	Specification Scopes	11

3	Dataset Identification	11

4	Data Content and Structure	12

4.1	Introduction	12

4.2	Application Schema	13

4.3	Feature Catalogue	13

4.3.1	Introduction	13

4.3.2	Feature types	13

4.3.3	Feature relationship	13

4.3.4	Attributes	13

4.3.5	Spatial quality	13

4.4	Dataset types	14

4.5	Spatial Schema	14

4.5.1	Coverages	14

4.5.2	Regular grids	15

5	Coordinate Reference Systems (CRS)	16

5.1	Horizontal reference system	16

5.2	Vertical reference system	17

5.3	Temporal reference system	18

6	Data Quality	18

6.1	Introduction	18

6.1.1	Data quality indication within datasets	18

6.1.2	Data quality elements and data quality measures (informative)	18

6.1.3	Description of quality elements (informative)	19

6.1.4	Applicable quality measures	20

6.2	Additional components of data quality	23

6.3	Assessment of data quality	23

6.4	Validation checks	24

7	Data Capture and Classification	25

7.1	Data sources for water levels	25

7.1.1	Determination of trend	26

7.2	The production process	26

7.2.1	Metadata	26

7.2.2	Water Level Data	27

7.2.3	Validation	27

7.2.4	Digital Signatures	27

7.3	Guidance for chunking and compression (informative)	27

7.4	Datasets in a series	28

7.5	Data use purpose	28

7.5.1	Datum requirements	28

7.5.2	Spatial type	28

7.5.3	Suitability for navigation	28

7.5.4	Use purpose metadata	29

7.6	Compliance categories	29

7.7	Compliance with S-98	29

7.7.1	Requirements for visual interoperability	29

7.7.2	Requirements for harmonised user experience	29

7.7.3	Requirements for water level adjustment	29

7.8	Datasets with multiple vertical datums	30

7.9	Construction of coverages	31

7.9.1	Grid cell structure	31

7.10	Scale ranges	31

7.11	Bounding polygon guidelines	31

8	Maintenance	32

8.1	Overview of dataset maintenance	32

8.1.1	Update of harmonic constants (informative)	33

8.2	Metadata related to dataset maintenance	33

8.2.1	Elements used in S-104	33

8.2.2	New datasets	33

8.2.3	New Editions	34

8.2.4	Cancellations	34

8.2.5	Other S100_Purpose values (informative)	36

8.2.6	Maintenance of support files	36

8.2.7	Encoding update frequency	36

9	Portrayal	37

10	Data Product Format (Encoding)	37

10.1	Introduction	37

10.2	HDF5 product structure for time series and gridded data	37

10.2.1	Data type definition	37

10.2.2	Product structure	38

10.3	Datatypes for coordinates	43

10.4	Axis names and ordering	44

11	Data Product Delivery	44

11.1	Introduction	44

11.2	HDF5 dataset packaging	45

11.2.1	Exchange Sets	45

11.2.2	Exchange Catalogue	47

11.2.3	Dataset file naming	47

11.2.4	Support files	48

11.2.5	Support file naming	48

12	Metadata	49

12.1	Introduction	49

12.1.1	Realisation of Exchange Set components and metadata classes (informative)	49

12.1.2	Exchange Set components and related metadata	50

12.2	Discovery metadata	52

12.2.1	S100_ExchangeCatalogue	55

12.2.2	S100_ExchangeCatalogueIdentifier	56

12.2.3	S100_CataloguePointofContact	56

12.2.4	S100_DatasetDiscoveryMetadata	57

12.2.5	S100_NavigationPurpose	60

12.2.6	S100_DataCoverage	60

12.2.7	S100_Purpose	61

12.2.8	S100_TemporalExtent	62

12.2.9	S100_EncodingFormat	62

12.2.10	S100_ProductSpecification	62

12.2.11	S100_CompliancyCategory	63

12.2.12	S100_ProtectionScheme	63

12.2.13	S100_SupportFileDiscoveryMetadata	64

12.2.14	S100_SupportFileFormat	65

12.2.15	S100_SupportFileRevisionStatus	65

12.2.16	S100_SupportFileSpecification	65

12.2.17	S100_ResourcePurpose	66

12.2.18	S100_CatalogueDiscoveryMetadata	66

12.2.19	S100_CatalogueScope	67

12.2.20	MD_MaintenanceInformation	67

12.2.21	MD_MaintenanceFrequencyCode	68

12.2.22	PT_Locale	68

12.2.23	S100_SE_CertificateContainer	69

12.2.24	S100_SE_DigitalSignatureReference	69

12.2.25	S100_SE_DigitalSignature	69

12.2.26	S100_SE_SignatureOnData	70

12.2.27	S100_SE_SignatureOnSignature	70

12.2.28	DataStatus	71

12.2.29	EX_GeographicBoundingBox	71

12.2.30	EX_BoundingPolygon	72

12.3	Carrier Metadata	74

12.3.1	General metadata - details	80

12.3.2	Feature Type metadata - details	83

12.3.3	Feature Instance metadata - details	85

12.3.4	Values Group attributes - details	86

12.3.5	Additional enumerations used in carrier metadata	86

12.4	Language	91

ANNEX A – Data Classification and Encoding Guide	93

A-1	Features	93

A-2	Feature Attributes	93

ANNEX B – S-104 Comprehensive Model Including Application Schema and Carrier Metadata (UML Diagrams)	95

ANNEX C – Feature Catalogue	101

ANNEX D – Use Cases	105

D-1	German Water Level Data and Forecast	105

D-1.1	Summary	105

D-1.2	Additional details	105

D-1.2.1	Types of data used to create S-104 test datasets	105

D-1.2.2	Data processing	106

D-1.2.3	S-104 data transformation	106

D-1.2.4	Results	106

D-2	Depth Adjustment in ECDIS	107

D-2.1	Summary	107

D-2.2	Additional details	108

D-2.2.1	Types of data	108

D-2.2.2	Processing	108

FOREWORD

The International Hydrographic Organization Tides, Water Level and Currents Working Group (TWCWG) remembers Kurt Hess, and acknowledges his invaluable and significant contributions in developing this Product Specification within the TWCWG.

Page intentionally left blank

# Overview

S-104 is the Water Level Information for Surface Navigation Product Specification, produced by the IHO. This edition of S-104 is intended for use with the “water level adjustment” functionality described in IHO Publication S-98.

The development of electronic navigation with high resolution bathymetric data, and the drive to increase safety of navigation are now demanding time-sensitive data. IHO has identified the requirement for a Product Specification for dynamic tidal and water level data.

Tidal height information has traditionally been provided as high/low predictions; however, with increasing drafts and technology, there has been a move to hourly or more frequent predictions with major ports providing real-time water level information to their pilots and web-sites.

In order to allow applications to dynamically adjust depths, there is a requirement for S-104 Edition 2.0.0 to supply water level data as gridded surface time-series data compatible with the IHO S-102 (Bathymetric Surface) and S-101 (Electronic Navigation Chart) data products.

## Introduction

Tidal and water level predictions have been fundamental in route planning and entry to ports (SOLAS Chapter V). These have traditionally been supplied as a physical hard copy publication and recently as a separate software installation that may not be integrated with the ECDIS. New functionality is now being defined to integrate dynamic water level information into ECDIS. To improve safety of navigation, this product specification will ensure that tidal and water level data supplied for dynamic capability is consistent by all approved authorities.

Annex C of IHO Publication S-98 describes ECDIS functionality for combining dynamic water level information with bathymetric data to present ship’s officers with a more accurate picture of water depth over an interval. Given the technological ability to generate short-term dynamic forecasts of water level information and communicate them to vessels more frequently, this allows provision of more timely and accurate look-ahead depth information which can be used for more accurate estimation of navigability and hazard assessment by ship’s officers and watchstanders.

### Data types

This edition of S-104 describes one data type that can be delivered to recipients such as ECDIS or other applications, either on board a vessel or in a shore installation:

A time series of water level height relative to a vertical datum and water level trend. The data represent an array of points contained in a grid. Time and datum information are contained in the metadata. The primary purpose of this data type is to enable displays of dynamic depth information on ECDIS in combination with bathymetric data; another purpose is to update water depths for under-keel clearance management.

### Display

This edition of S-104 is designed for a single means of displaying water level data to support navigation, route planning, and route monitoring:

- Adjusted display of depths from datasets containing bathymetric data or electronic nautical chart information. This will be described in S-98 Annex C.

### Encoding

There is one encoding of water level data:

1. HDF5 (Hierarchical Data Format version 5) is used for encoding time series of water level heights and trends (and optionally, uncertainty) at an array of points in a grid. HDF5 promotes compatible data exchange due to its common neutral encoding format. HDF5 is object oriented and suitable for many types of data.

Table 1-1 summarises Clauses 1.1.1 through 1.1.3.

Table 1-1 – S-104 data variables, formats, encoding and display.

| Data Variable | Data Format | Encoding | Display |
| --- | --- | --- | --- |
| Water level height | Multiple Locations, Time Series (i.e., Gridded Forecast), as values records combining all attributes. | HDF5 | According to data product with which S-104 data is combined, with depth values and uncertainties adjusted. For example, bathymetry coverage or ENC feature display with shade according to adjusted depth. |
| Water level trend |  |  | Not included in S-98 Annex C water level adjustment. Should be included in pick report if pick reports are implemented. |
| Uncertainty |  |  | Combined with uncertainty in data product with which S-104 data is combined. |

## Scope

This document describes an S-100 compliant Product Specification for the encapsulation and data transfer of tidal and water level data for use in an Electronic Chart Display and Information System (ECDIS) or any proposed dynamic tide application. This Product Specification includes the content model, the encoding, the feature catalogue and metadata. The water level product may be used either alone or combined with other S-100 compatible data.

## References

### Normative

HDF5	Hierarchical Data Format version 5 – www.hdfgroup.org

M-3	Resolutions of the International Hydrographic Organization, IHO Publication M-3, 2nd Edition, 2010 (updated July 2023)

Res. 3/1919	IHO Resolution 3/1919 (as amended), IHO Publication M-3

S-44	IHO Standards for Hydrographic Surveys, Edition 6.1.0, October 2022

S-62	List of Data Producer Codes (online), IHO GI registry, URL: https://registry.iho.int

S-97	IHO Guidelines for Creating S-100 Product Specifications, Edition 2.0.0 (in preparation)

S-98	Data Product Interoperability in S-100 Navigation Systems, Edition 3.0.0. In preparation

S-100	IHO Universal Hydrographic Data Model, Edition 5.2.1, December 2025

S-158	Validation Checks – Introduction and Structure, Edition 1.0.0, February 2025

S-158:100	Validation Checks (Universal Hydrographic Data Model), Edition 1.0.0, February 2025

S-158:104	Validation Checks (Water Level Information for Surface Navigation), Edition 1.1.0. In preparation

S-158:98	Data Product Interoperability Validation Checks (Interoperability), Edition 1.0.0, February 2025

NOTE: Titles and numbers for publications in preparation are provisional pending publication by the IHO.

### Informative

IALA G1143 	Unique Identifiers for Maritime Resources, Edition 3.0. International Association of Marine Aids to Navigation and Lighthouse Authorities, June 2021

IOC 14-4 	Manual on Sea-level Measurements and Interpretation, Volume IV : An update to 2006. Paris, Intergovernmental Oceanographic Commission of UNESCO. (IOC Manuals and Guides No.14, vol. IV; JCOMM Technical Report No.31; WMO/TD. No. 1339)

ISO 3166-1:1997 	Country Codes

ISO 8601:2004 	Data elements and interchange formats - Information interchange - Representation of dates and times

ISO/TS 19103:2005 Geographic information – Conceptual schema language

ISO 19111:2003 	Geographic information – Spatial referencing by coordinates

ISO 19115-1 	Geographic information – Metadata – Part 1 – Fundamentals. As amended by Amendment 1, 2018

ISO 19115-2:2009 	Geographic information – Metadata: Extensions for imagery and gridded data

ISO 19115-3 	Geographic information – Metadata - XML schema implementation for fundamental concepts, 2016

ISO 19123:2005 	Geographic information – Schema for coverage geometry and functions

ISO 19129:2009 	Geographic information – Imagery gridded and coverage data framework

ISO 19131:2007 	Geographic information – Data product specifications

ISO 19157:2013 	Geographic information – Data Quality. As amended by Amendment 1, 2018

ISO/IEC 19501-1 and 19505-2 Information technology — Open Distributed Processing – Unified Modelling Language Version 2.4.1

netCDF	Network Common Data Form Unidata – URL: www.unidata.ucar.edu/software/netcdf

NGA 2021	Recent Update to WGS 84 Reference Frame and NGA Transition to IGS ANTEX, NGA Office of Geomatics / GNSS Division, St. Louis, National Geospatial-Intelligence Agency, 2021.

NGA 2023	WGS 84 (G2296) Terrestrial Reference Frame Realization, Office of Geomatics, National Geospatial-Intelligence Agency, NGA-U-2023-02846, 2023.

RFC 3986	Uniform Resource Identifier (URI): Generic Syntax. T. Berners-Lee, R. Fielding, L. Masinter. Internet Standard 66, IETF. URL: http://www.ietf.org/rfc/rfc3986.txt or http://www.rfc-editor.org/info/std66

RFC 2141	URN Syntax. R. Moats. IETF RFC 2141, May 1997. URL: http://www.rfc-editor.org/info/rfc2141

S-101	IHO Electronic Navigational Chart Product Specification, Edition 1.2.0 (in preparation)

S-32	International Hydrographic Dictionary. URL: http://iho-ohi.net/S32/index.php

S-102	IHO Bathymetric Surface Product Specification, Edition 3.0 (in preparation)

S-111	IHO Surface Currents Product Specification, Edition 2.0.0 (October 2024)

XML Schema Part 2: Datatypes, Second Edition, W3C Recommendation, 28 October 2004, URL: https://www.w3.org/TR/xmlschema-2/

## Terms, definitions and abbreviations

### Terms and definitions

Terms and definitions have been taken from the normative references cited in Clause 1.3. Only those which are specific to this document have been included and modified where necessary.

accuracy

closeness of agreement between an observed value and the true value or a reference value accepted as true [ISO 19157, ISO 19116]

NOTE 1: A test result can be observations or measurements

NOTE 2: For positioning services, the test result is a measured value or set of values

NOTE 3: For observations and measurements, true values are not obtainable. In their place reference values which are accepted as true values are used

application schema

conceptual schema for data required by one or more applications [ISO 19101]

computed

calculated; derived rather than measured or observed

continuous coverage

coverage that returns different values for the same feature attribute at different direct positions within  a single geometric object in its spatiotemporal domain [ISO 19123]

NOTE Although the spatiotemporal domain of a continuous coverage is ordinarily bounded in terms  of its spatial extent, it can be subdivided into an infinite number of direct positions.

coordinate

one of a sequence of numbers designating the position of a point in N-dimensional space [ISO 19111]

coordinate reference system

coordinate system which is related to the real world by a datum [ISO 19111]

coverage

feature that acts as a function to return values from its range for any direct position within its spatial, temporal, or spatiotemporal domain

EXAMPLE: Examples include a raster image, polygon overlay, or digital elevation matrix.

NOTE: In other words, a coverage is a feature that has multiple values for each attribute type, where each direct position within the geometric representation of the feature has a single value for each attribute type [ISO 19123].

coverage geometry

configuration of the domain of a coverage described in terms of coordinates [ISO 19123].

data product

dataset or dataset series that conforms to a data product specification

NOTE: The S-104 data product consists of metadata and one or more sets of water level height and trend [ISO 19131]

data quality element

quantitative component documenting the quality of a dataset [ISO 19101:2002]

NOTE: The applicability of a data quality element to a dataset depends on both the dataset’s content and its product specification, the result being that all data quality elements may not be applicable to all datasets

data quality evaluation procedure

the whole of operations used in applying and reporting quality evaluation methods and their results [ISO 19113]

data quality measure

an evaluation of a data quality sub-element [ISO 19113]

data quality overview element

the non-quantitative component documenting the quality of a dataset. Information about the purpose, usage, and lineage of a dataset is non-quantitative quality information

NOTE Information about the purpose, usage and lineage of a dataset is non-quantitative quality information. [ISO 19101]

data quality result

a value or set of values resulting from applying a data quality measure or the outcome of evaluating the obtained value or set of values against a specified conformance quality level

EXAMPLE A data quality result of “90” with a data quality value type of “percentage” reported for the data quality element and its data quality subelement “completeness, commission” is an example of a value resulting from applying a data quality measure to a data specified by a data quality scope. A data quality result of “true” with a data quality value type of “Boolean variable” is an example of comparing the value (90) against a specified acceptable conformance quality level (85) and reporting an evaluation of a kind, pass or fail. [ISO 19113]

data quality scope

the extent or characteristic(s) of the data for which quality information is reported [ISO 19113]

NOTE: A data quality scope for a dataset can comprise a dataset series to which the dataset belongs, the dataset itself, or a smaller grouping of data located physically within the dataset sharing common characteristics. Common characteristics can be an identified feature type, feature attribute, or feature relationship; data collection criteria; original source; or a specified geographic or temporal extent. [S-100 Annex A]

data quality sub-element

a component of a data quality element describing a certain aspect of that data quality element [ISO 19103]

dataset

identifiable collection of data [ISO 19115]

NOTE A dataset may be a smaller grouping of data which, though limited by some constraint such  as spatial extent or feature type, is located physically within a larger dataset. Theoretically, a dataset  may be as small as a single feature or feature attribute contained within a larger dataset. A hardcopy  map or chart may be considered a dataset.

dataset series

collection of datasets sharing the same product specification [ISO 19115]; collection of datasets with data for the same geographical area at different times or for successive time periods

NOTE: This Product Specification uses the second sense.

datum

parameter or set of parameters that define the position of the origin, the scale, and the orientation of a coordinate system [ISO 19111, ISO 19116]

NOTE 1: A datum defines the position of the origin, the scale, and the orientation of the axes of a coordinate system

NOTE 2: A datum may be a geodetic datum, a vertical datum, an engineering datum, an image datum, or a temporal datum

direct position

position described by a single set of coordinates within a coordinate reference system [ISO 19107]

domain

well-defined set [ISO 19103]

NOTE: Domains are used to define the domain set and range set of operators and functions.

elevation

the altitude of the ground level of an object, measured from a specified vertical datum. [IHO S100 GFM]

feature

abstraction of real world phenomena [ISO 19101]

NOTE: A feature may occur as a type or an instance. Feature type or feature instance should be used when only one is meant.

feature attribute

characteristic of a feature

EXAMPLE 1: A feature attribute named colour may have an attribute value green which belongs to the data type text

EXAMPLE 2: A feature attribute named length may have an attribute value 82.4 which belongs to the data type real

NOTE 1: A feature attribute may occur as a type or an instance. Feature attribute type or feature attribute instance is used when only one is meant.

NOTE 2: A feature attribute type has a name, a data type, and a domain associated to it. A feature attribute instance has an attribute value taken from the domain of the feature attribute type.

NOTE 3: In a Feature Catalogue, a feature attribute may include a value domain but does not specify attribute values for feature instances.

[ISO 19101, ISO 19109, ISO 19110, ISO 19117]

height

distance of a point from a chosen reference surface measured upward along a line perpendicular to that surface. [ISO 19111:2006]

NOTE: Height is distinguished from elevation in that it is a directional measurement.

georectified

corrected for positional displacement with respect to the surface of the Earth [ISO 19115-2]

georeferenced Grid

grid for which cells can be located by the use of specific algorithms. See ungeorectified grid.

grid

network composed of a set of elements, or cells, whose vertices, or nodes, have defined positions within a coordinate system. See also georeferenced grid, regular grid, ungeorectified grid, node, and grid point. [ISO 19123]

NOTE 1: A rectangular grid has axes perpendicular to each other.

NOTE 2: A uniform rectangular grid has constant spacing in the X-direction and constant spacing in the Y-direction, although the two spacing values are not necessarily equal.

grid cell

element of a grid defined by its vertices, or nodes

grid coordinates

sequence of two or more numbers specifying a position with respect to its location on a grid [S-100 Annex A]

grid point

point located at the intersection of two or more grid cells in a grid. Also called a node. [ISO 19123]

gridded data

data whose attribute values are associated with positions on a grid coordinate system [ISO 19115-2]

measurement

the (detailed) dimensions of a physical quantity [S-32]

observation

the act or practice of noting and recording facts and events as for some scientific study. The measure of a quantity whose value is desired. The DATA so noted and recorded. A single measure, at a single setting of an apparatus [S-32]

positional accuracy

closeness of coordinate value to the true or accepted value in a specified reference system

NOTE: The term absolute accuracy is sometimes used for this concept to distinguish it from relative positional accuracy. Where the true coordinate value may not be perfectly known, accuracy is normally tested by comparison with available values that can best be accepted as true  [ISO 19116]

record

finite, named collection of related items (objects or values) [ISO 19107]

NOTE: Logically, a record is a set of pairs <name,item>.

result scope

scope of the (data quality) result

NOTE: Result scope is a subset of the data quality scope. [S-97 ed. 1.1]

sequence

finite, ordered collection of related items (objects or values) that may be repeated

NOTE 1: Logically, a sequence is a set of pairs <item, offset>. LISP syntax, which delimits sequences with parentheses and separates elements in the sequence with commas, is used in this international standard [ISO 19107]

standalone quality report

free text document providing fully detailed information about data quality evaluations, results and measures used [ISO 19157:2013]

temporal series

collection of datasets with data for the same geographical area at different times or for successive periods

timestamp

value of time at which an object’s state is measured and recorded [ISO 19132]

uncertainty

the interval (about a given value) that will contain the true value of the measurement at a specific confidence level [IHO S-44]

NOTE: Errors exist and are the differences between the measured value and the true value. Since the true value is never known it follows that the error itself cannot be known. Uncertainty is a statistical assessment of the likely magnitude of this error.

water level trend

change of water level at a given time, such as ‘increasing’, ’decreasing’, or ‘steady’.

When the average change of the water level over a defined period is greater than or equal to a specified threshold value it is considered “increasing”. When it is less than or equal to -(threshold value), it is “decreasing”. When it is between the positive and negative thresholds, it is “steady”. The Producing Authority determines the duration of the period and the threshold value.

In areas of small water level range, for example Baltic Sea, use of “not available” is optional

ungeorectified grid

grid with non-uniform point spacing in any coordinate system. Includes triangular irregular networks (TINs) and those curvilinear coordinate grids whose node positions cannot be calculated analytically

vertical coordinate system

one-dimensional coordinate system used for gravity-related height or depth measurements [ISO 19111]

vertical datum

datum describing the relation of gravity-related heights or depths to the Earth

NOTE: In most cases the vertical datum will be related to mean sea level. Ellipsoidal heights are treated as related to a three-dimensional ellipsoidal coordinate system referenced to a geodetic datum. Vertical datums include sounding datums (used for hydrographic purposes), in which case the heights may be negative heights or depths [ISO 19111]

### Abbreviations

API	Application Programming Interface

CRS	Coordinate Reference System

DQWG	Data Quality Working Group

ECDIS	Electronic Chart Display Information System

EPSG	European Petroleum Survey Group

ENC	Electronic Navigational Chart

FC	Feature Catalogue

GIS	Geographic Information Systems

HDF	Hierarchical Data Format

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

IHO	International Hydrographic Organization

IMO	International Maritime Organization

ISO	International Organization for Standardization

MRN	Maritime Resource Name

NetCDF	Network Common Data Form

PC	Portrayal Catalogue

SOLAS	International Convention for the Safety of Life at Sea

TIN	Triangulated Irregular Network

TWCWG	Tides, Water Level and Currents Working Group

UML	Unified Modelling Language

URN	Uniform Resource Name

UTC	Coordinated Universal Time

W3C	World Wide Web Consortium

XML	eXtensible Markup Language

### Notation

In this document conceptual schemas are presented in the Unified Modelling Language (UML). Several model elements used in this schema are defined in ISO Standards developed by ISO TC 211, or in IHO S-100. In order to ensure that class names in the model are unique ISO TC/211 has adopted a convention of establishing a prefix to the names of classes that define the TC/211 defined UML package in which the UML class is defined. Since the IHO Standards and this Product Specification make use of classes derived directly from the ISO Standards this convention is also followed here. In the IHO Standards the class names are identified by the name of the Standard, such as “S100” as the prefix optionally followed by the bialpha prefix derived from ISO. For the classes defined in this Product Specification the prefix is “S104”. In order to avoid having multiple classes instantiating the same root classes, the ISO classes and S-100 classes have been used where possible; however, a new instantiated class is required if there is a need to alter a class or relationship to prevent a reverse coupling between the model elements introduced in this document and those defined in S-100 or the ISO model.

Table 1.2 – Sources of externally defined UML classes

| Prefix | Standard | Package |
| --- | --- | --- |
| CI | ISO 19115-1 | Citation and Responsible Party |
| CV | ISO 19123 | Coverage Core & Discrete Coverages |
| DQ | ISO 19157 | Data Quality Information |
| DS | ISO 19115-1 | Metadata Application Information |
| EX | ISO 19115-1 | Metadata Extent information |
| IF | ISO 19129 | Imagery Gridded and Coverage Data Framework |
| LI | ISO 19115-1 | Linage Information |
| MD | ISO 19115-1 | Metadata entity set information |
| MI | ISO 19115-2 | Metadata entity set imagery |
| S100 | IHO S-100 | IHO Standard for Hydrographic Data |
| SC | ISO 19111 | Spatial Referencing by Coordinates |
| SD | ISO 19130 | Sensor Data |

## Use of language

Within this document:

- “Must” indicates a mandatory requirement.

- “Should” indicates an optional requirement, that is the recommended process to be followed, but is not mandatory.

- “May” means “allowed to” or “could possibly”, and is not mandatory.

## General data product description

NOTE: This clause provides general information regarding the data product.

Title:	Water Level Information for Surface Navigation

Abstract: 	Encodes information and parameters for use in making a tidal and water level product.

Content: 	Describes the tidal and water level data contained in the product. The specific content is defined by the Feature Catalogue and Schema.

Spatial Extent:	Description: Areas where tidal and water level information is available.

East Bounding Longitude: 180

West Bounding Longitude: -180

North Bounding Latitude: 90

South Bounding Latitude: -90

Purpose: 	The data shall be used to produce a dataset to be used for dynamic water level applications, including an ECDIS.

## Data Product Specification metadata

NOTE: This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see clause 12.

Title:	Water Level Information for Surface Navigation

S-100 Version:	5.2.1

S-104 Version: 	2.1.0

Date:	October 2026

Language: 	English

Classification: 	Unclassified

Contact: 	International Hydrographic Organization.

4 quai Antoine 1er

B.P. 445 MC 98011 MONACO CEDEX

Telephone: +377 93 10 81 00

Fax: + 377 93 10 81 40

Email: info@iho.int

Role:	Owner

URL: 	https://registry.iho.int

Identifier: 	S-104

Maintenance:		Changes to the Product Specification S-104 are coordinated by Tides, Water Level and Currents Working Group (TWCWG) of the IHO and made available via the IHO Publications web site. Maintenance of the Product Specification must conform to IHO Technical Resolution 2/2007 (revised 2010). This Specification will be a standing agenda item for TWCWG meetings with clarifications, revisions and new editions released as required.

### IHO Product Specification maintenance

#### Introduction

Changes to S-104 will be released by the IHO as a New Edition, revision, or clarification.

#### New Edition

New Editions of S-104 introduce significant changes. New Editions enable new concepts, such as the ability to support new functions or applications, or the introduction of new constructs or data types. New Editions are likely to have a significant impact on either existing users or future users of S-104. All cumulative revisions and clarifications must be included with the release of approved New Editions.

#### Revision

Revisions are defined as substantive semantic changes to S-104. Typically, revisions will change S-104 to correct factual errors; introduce necessary changes that have become evident as a result of practical experience or changing circumstances. A revision must not be classified as a clarification. Revisions could have an impact on either existing users or future users of S-104. All cumulative clarifications must be included with the release of approved revisions.

Changes in a revision are minor and ensure backward compatibility with the previous versions within the same Edition. Newer revisions, for example, introduce new features and attributes. Within the same Edition, a dataset of one version could always be processed with a later version of the Feature and Portrayal Catalogues.

In most cases a new Feature Catalogue will result in a revision of S-104.

#### Clarification

Clarifications are non-substantive changes to S-104. Typically, clarifications: remove ambiguity; correct grammatical and spelling errors; amend or update cross references; and insert improved graphics. A clarification must not cause any substantive semantic change to S-104.

Changes in a clarification are minor and ensure backward compatibility with the previous versions within the same Edition.

#### Version numbers

The associated version control numbering to identify changes (n) to S-104 must be as follows:

New Editions denoted as n.0.0

Revisions denoted as n.n.0

Clarifications denoted as n.n.n

# Specification Scopes

This Product Specification outlines the types of water level products from a national authority or authorised producer, to the end user. The data may be real-time observation, astronomical prediction, analysis or hybrid method, or forecast models. Requirements for data and metadata are provided. The data product is a time series product, including series of water level heights relative to a vertical datum and the water level trend (rising, falling, etc.). The data product is a gridded data product– provision of water level information for a defined region as a surface, allowing any grid point to be queried as per a traditional single point.

Scope ID:		Global

Level:		006- series

Level name:	Water Level Dataset

# Dataset Identification

Title: 	Water Level Information for Surface Navigation

Alternate Title:	None

Abstract: 	This data product is a file containing water level data for a particular geographic region and set of times, along with the accompanying metadata describing the content, variables, applicable times, locations and structure of the data product. Water level data is the height of the water observed or mathematically-predicted. The data may consist of water level at a small set of points where observations or predictions are available or may consist of numerous points organised in a grid as from a hydrodynamic model forecast.

Topic Category:	Producing Authority to choose the most appropriate from the list below:

| Concept Name | ISO 19115-1 Topic Category Number | ISO 19115-1 Topic Category Code | Definition | Remarks |
| --- | --- | --- | --- | --- |
| Inland Waters | 012 | inlandWaters | Inland water features, drainage systems and their characteristics Examples: rivers and glaciers, salt lakes, water utilization plans, dams, currents, floods, water quality, hydrologic information | Use for datasets covering navigation on inland waterways |
| Oceans | 014 | oceans | Features and characteristics of salt water bodies (excluding inland waters) Examples: tides, tsunamis, coastal information, reefs | Use for datasets intended for coastal, offshore, or ocean navigation |
| Transportation | 018 | transportation | Means and aids for conveying persons and/or goods Examples: roads, airports/airstrips, shipping routes, tunnels, nautical charts, vehicle or vessel location, aeronautical charts, railways | Use for datasets intended for navigation (inland or maritime) |

Geographic Description:	Areas specific to water navigation

Spatial Resolution:	The spatial resolution, or the spatial dimension of the earth covered by the size of a grid matrix cell (nominal ground sample distance), varies according to the model adopted by the producer.

Purpose:	Water level data in this product is intended to be used for dynamic adjustment of depths in bathymetry and ENC layers.

Language: 	English

Classification: 	Data can be classified as one of the following:

1) Unclassified;

2) Restricted;

3) Confidential;

4) Secret;

5) Top Secret;

6) Sensitive but Unclassified;

7) For Official Use Only;

8) Protected; or

9) Limited Distribution.

Spatial Representation Type: 	Coverage

Point of Contact: 	Producing Authority.

Use Limitation:	Invalid over land. Some datasets may be designated as not for navigation.

# Data Content and Structure

## Introduction

This clause discusses the Application Schema, which is described in UML; the Feature Catalogue; dataset types, in which there is an extensive discussion of the water level data; and geometry.

Water level data consist of one basic geographic feature type:

A time series of water level height and trend relative to a vertical datum. An optional time attribute is also provided for use with certain types of water level information. The data can be represented for an array of points contained in a grid. Time and datum information are contained in the metadata.

Figure 4-1 – Water level feature

## Application Schema

The Application Schema is expressed in UML. The single feature type, WaterLevel, is depicted in Figure 4-1. The details of the Application Schema are given in Annex B, which also describes its relation to the conceptual model of coverage data described in ISO 19123 and S-100 Part 8.

## Feature Catalogue

### Introduction

The S-104 Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product. See Annex C – Feature Catalogue

The S-104 Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema and can be downloaded from the IHO GI Registry website.

### Feature types

#### Geographic

Geographic (geo) feature types form the principal content of S-104 and are fully defined by their associated attributes.

#### Meta

Meta features contain information about other features within a data set. Information defined by meta features override the default metadata values defined by the data set descriptive records. Meta attribution on individual features overrides attribution on meta features.

### Feature relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type. There are three common types of feature relationship: Association, Aggregation and Composition. In S-104 there are no relationships used.

### Attributes

S-100 defines attributes as either simple or complex. S-104 uses three types of simple attributes, listed in Table 4-1. There are no complex attributes.

Table 4-1 – Simple feature attribute types.

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |

### Spatial quality

Spatial quality attributes (Figure 4-2) are encoded as horizontal and vertical uncertainty values. In S-100 Edition 5.0.0 they are encoded at the feature type level (see Figure 4-2 and Table 12-2), which means they apply uniformly to all Water Level feature instances in the dataset and uniformly to all locations (grid points or station locations).

Figure 4-2 – Spatial quality

Note that uncertainty in water levels pertains to the quality of data values, not to spatial quality as that term is used in S-100, and is encoded differently and at the instance level (see Table 10-2 and Clause 10.2.2.4).

Water levels are usually defined at one or more individual locations, so spatial quality applies to these locations.

NOTE: The Spatial Quality information type used in S-101 and other products is not used in this Edition of S-104.

## Dataset types

Datasets for S-104 include one basic type of dataset: HDF5 files, which may contain gridded water level data.

## Spatial Schema

### Coverages

For an ECDIS, water level data are formatted as arrays of points contained in a regular grid. Further details on the data products are given in clause 10 – Data Product Format.

Water level data are categorised as follows, based on the data source:

Computed, observed, predicted, or forecasted values (for example forecast data from hydrodynamic models) arranged in a regular grid.

The spatial structure used by S-104 is a regular grid, described by S100_IF_Grid (S-100 Edition 5.2.1, clause 8-7.5). The class S100_IF_Grid is a realization of CV_RectifiedGrid and CV_GridValuesMatrix from ISO 19123 and a component of ISO 19123 continuous quadrilateral grid coverages as realized by S100_IF_GridCoverage.

The types of water level data and their corresponding coverages are shown in Table 4-2.

Table 4-2 – Water level data types and their coverages

| Type of Data | Spatial Structure | Coverage Type |
| --- | --- | --- |
| Regularly-gridded data at one or more times | S100_IF_Grid | S100_IF_GridCoverage |

The spatial representations in S-104 are encoded using the implementation specification in S-100 Part 10c, which realises S-100 Part 8 and ISO 19123 conceptual models. The relationships are depicted in Figure 4-3 below.

Figure 4-3 – Coverages and their realisation from S-100 Part 8 and ISO 19123

### Regular grids

S-104 regular grid geometry is an implementation of S100_IF_Grid (S-100 Part 8 – Imagery and Gridded Data). The spatial grids for the regular grid type are two dimensional, orthogonal, and georeferenced (with the X or longitudinal axis directed toward the east), and are defined by several attributes, including grid origin, spacing, and grid indexing. Water level values apply at the vertices of the grid; that is, the intersections of the row and column lines. These parameters are explained in more detail below. A typical regular grid and some of its parameters are shown in Figure 4-4.

Figure 4-4 – Schematic of the regular grid and some of its attributes

Vertices are shown as the filled squares at the intersections of the rows and columns. The offsetVectors are shown as the Latitudinal Spacing and Longitudinal Spacing. The origin is shown at the lower left corner of the grid.

The grid is oriented to the Earth by the Coordinate Reference System (CRS), with the variable coordinateReferenceSystem. The origin contains the latitude and longitude as a DirectPosition and is located at the point at the lower left (southwest) extent of the grid. The upper corner is the north easternmost point in the grid. The attribute dimension is 2, and the variable interpolationType has one of the allowed values for interpolation methods specified in S-100 Part 8.

S-104 grids allow for different spacing of points along the X (longitudinal) axis and the Y (latitudinal) axis. For rectangular grids the offset vector establishes the cell size. The attribute offsetVectors carries the two vectors for grid spacing (Latitudinal Spacing and Longitudinal Spacing). The first vector is 90 degrees clockwise from CRS north, and represents the distance between grid values on the X axis. The second vector is 0 degrees clockwise from CRS north, and represents the distance between the values on the Y axis. The distances are given in degrees.

The attribute extent effectively defines a bounding rectangle describing where data is provided. The attribute extent carries two sub-attributes; low and high. The sub-attribute low carries the value “0, 0” to indicate the index values at the start of the extent is the southwest (lower left) corner of the grid. The sub-attribute high, carries the value of the highest position along the X axis and the highest position along the Y axis. For example, if the number of rows is numROWS and the number of columns is numCOLS, then the index values for high would be ‘numCOLS-1,numROWS-1’. Together they form the grid coordinate of the upper right corner.

The sequence rule for a regular cell size grid is straightforward. When the cells all have the same dimensions, the cell index can be derived from the position of the Record within the sequence of Records. The attribute sequencingRule has two sub-attributes; type and scanDirection. The sub-attribute type carries the value “linear”, and the sub-attribute scanDirection carries a comma-separated list of axis names for the coordinate reference system. Together with the value “0,0” stored in the attribute startSequence, they indicate that for a hypothetical S-104 grid with scanDirection=”X,Y”, grid values along the X axis at the lowest Y axis position are stored first, starting with the left most value going right, followed by the values along the X axis at the next increment upward along the Y axis, and so on till the top of the Y axis. The last value in the value sequence of the grid will be at the top rightmost position in the grid. In Figure 4-4, first all columns in row 1 are selected, then all columns in row 2, and so on.

NOTE: Since the origin is at i_index and j_index value 0, the location of any longitude and latitude in the grid is computed by:

Longitude = GridOriginLongitude + (i_index)×(gridSpacingLongitudinal).     [Eqn 4.1]

Latitude = GridOriginLatitude + (j_index)×(gridSpacingLatitudinal).              [Eqn 4.2]

# Coordinate Reference Systems (CRS)

To define the location of features using the S-100 Framework, one first needs to define a Coordinate Reference System (CRS). A Coordinate Reference System in two dimensions uses a coordinate pair, either X and Y for a Cartesian system or latitude and longitude for a geodetic/geographic system to define the location of a feature on a 2-D grid. However, if one wants to plot features in a 3-dimensional Coordinate Reference System, where we now want to include depths on a nautical chart or elevations on a map, one needs to assign the depth or elevation as the third component. For Cartesian systems, one would use X, Y, Z as the triplet or for geodetic/geographic systems, one would use latitude, longitude and height. The height can be the ellipsoid height or any of the other vertical references (see Vertical Reference System below). Geodetic/geographic coordinates are more intuitive for positioning and navigation applications on or near the Earth’s surface while Cartesian coordinates are more appropriate if vectors are needed to accurately illustrate a graphical relationship between two or more points. If geodetic/geographic coordinates are specified, then the IHO recommends using the latest realisation of the World Geodetic System of 1984 (WGS 84).

## Horizontal reference system

For products based on the S-100 Framework, including this Standard for S-104 products, the geodetic/geographic Coordinate Reference System must be of the form EPSG:xxxx (with WGS 84 as base datum). The generic form/code for the WGS 84 frame is EPSG:4326 while the latest and most widely adopted realisation of the WGS 84 reference frame as of 2022 was EPSG:9057. The full reference to EPSG can be found at https://epsg.org and other EPSG references for recent WGS 84 realisations are given below:

WGS 84 (generic)				ESPG:4326

WGS 84(G2296)				EPSG:10606

WGS 84(G2139)				EPSG:9755		Valid epoch 2016:0

WGS 84(G1762)				EPSG:9057		Valid epoch 2005.0

WGS 84(G1674)				EPSG:9056		Valid epoch 2005.0

WGS 84(G1150)				EPSG:9055		Valid epoch 2001.0

WGS 84 / UTM Zone 1N to Zone 60N	EPSG:32601 – EPSG:32660

WGS 84 / UTM Zone 1S to Zone 60S	EPSG:32701 – EPSG:32760

WGS 84 / UPS North (E,N)			EPSG:5041

WGS 84 / UPS South (E,N)			EPSG:5042

Coordinate Reference System: 	EPSG:9057 (WGS 84) or another reference system  listed above

Datum:	WGS 84 defined by NGA

Projection: 	None / UTM / UPS

Horizontal Units:	Decimal degrees / Easting and northing

Coordinate Reference System Registry: 	EPSG Geodetic Parameter Registry

Date type (according to ISO 19115-1):  	002 - publication

Responsible party:  	International Association of Oil and Gas Producers (IOGP)

Producers of S-104 data must use the same CRS/projection as the underlying S-102 dataset and should endeavour to use the same realisation. (Reference system information encoded in datasets must be such that application software can automatically match reference system information encoded in different data products, especially S-101/S-102/S-104.)

## Vertical reference system

For positioning and navigation applications, it is desirable to accurately plot depths, bathymetry, elevations and terrain on nautical charts and maps using one or more vertical reference systems. To do so, a vertical datum is defined and serves as a reference surface for vertical positions. Vertical datums come in three categories: 1) those based on Mean Sea Level (MSL); 2) tidal datums; and 3) 3-D datums (ellipsoid) which are realised through space-based systems such as GPS. Vertical datums can be regional (geoid, tidal, chart) or global (ellipsoid) in nature. The vertical axis of a vertical reference system is defined upwards (away from the Earth’s centre) from its origin (EPSG code 6499) or downwards (EPSG code 6498) and is perpendicular to the horizontal surface where the observations or measurements are taken. As an example, a positive value for the level of water above the vertical datum in a vertical reference system with upward orientation (EPSG code 6499) means that the water level is above the vertical reference surface. For nautical charts, depths and tides are measured relative to a chart datum, defined for a certain epoch, such as Lowest Astronomical Tide (LAT) or Mean Lower Low Water (MLLW) or any chart datum stated in the IHO resolution 3/1919 as amended.

Coordinate Reference System: 	Vertical component of a 3-D reference system

Datum:	Chart, tidal, geoid, ellipsoid (WGS 84)

Projection: 	None

Horizontal Units:	metres

Coordinate Reference System Registry: 	EPSG Geodetic Parameter Registry

Date type (according to ISO 19115-1):  	002 - publication

Responsible party (vertical datums):  	National hydrographic and geodetic agencies

The vertical coordinate system is defined by three components. The first component defines the positive vertical direction (either an upward height or a downward depth). The second refers to the base or origin (that is, the zero value) of the vertical coordinate; if the base is a tidal datum, the specific datum is defined from either the S-100 list of vertical datums (for example LAT, MLLW, MSL, etc). Finally the specific datum number from the appropriate list is given. The components are summarised in Table 5-1.

Table 5-1 – Attributes describing the vertical coordinate system

| Name | Remarks |
| --- | --- |
| Vertical Coordinate System | EPSG Code; Allowed Values - 6499 (Height – Metres – Orientation Up) |
| Vertical Datum Reference | 1 – S-100 vertical datum |
| Vertical Datum | This is a value from the S100_VerticalAndSoundingDatum enumeration extended by adding “National Low Water Datum” recently defined in the IHO GI Registry |
| Vertical Datum Epoch | The period / epoch when the vertical datum was computed |

For navigation, the vertical datum must be consistent with the CRS in the underlying S-101 and S-102 datasets. Conflicts such as differing datums or cases where a dataset uses multiple datums should be resolved by discussion with the producer of the dataset(s) causing the conflict. (Splitting one or more of S-104 or another product into different datasets may be considered as a pathway to resolving such issues.)

## Temporal reference system

The temporal reference system is the Gregorian calendar for date and UTC for time. Time is measured by reference to TM_Calendar dates and TM_Clock time in accordance with ISO 19108:2002, Temporal Schema Clause 5.4.4. A date variable will have the following 8-character format (ISO 8601): yyyymmdd. A time variable will have the following 7-character format: hhmmssZ. A date-time variable will have the following 16-character format: yyyymmddThhmmssZ.

# Data Quality

## Introduction

Quality of water level data for navigation consists of quality of the observed/predicted/forecast data, quality of the positional data and quality of the time stamp.

### Data quality indication within datasets

Data quality may be indicated within datasets in the form of a single uncertainty value applicable to an entire grid or in the data values record as uncertainty values at individual grid points. Clauses 10 (Data Product Format) and 12.3 (Carrier Metadata) describe the encoding of quality indicators within datasets.

### Data quality elements and data quality measures (informative)

Data quality allows users and user systems to assess fitness for use of the provided data. Data quality measures and the associated evaluation are reported as metadata of a data product. This metadata improves interoperability with other data products and provides usage by user groups that the data product was not originally intended for. The secondary users can make assessments of the data product usefulness in their application based on the reported data quality measures.

For S-104 the following Data Quality Elements have been included :

- Conformance to this Product Specification;

- Intended purpose of the data product;

- Completeness of the data product in terms of coverage;

- Logical Consistency;

- Positional Uncertainty and Accuracy;

- Thematic Accuracy;

- Temporal Quality;

- Aggregation measures;

- Validation checks or conformance checks including:

  - General tests for dataset integrity;

  - Specific tests for a specific data model.

### Description of quality elements (informative)

The description of data quality measures in this clause is based on S-97 Edition 1.1. While this clause describes data quality elements in general, not all of them may be applicable to S-104 data. Clause 6.1.4 indicates the applicability and scope of the data quality elements for this Product Specification.

#### Completeness

Completeness is defined as the presence and absence of features, their attributes and relationships. It consists of two Data Quality Elements:

- Commission – excess data present in a dataset;

- Omission – data absent from a dataset.

#### Logical consistency

Logical Consistency is defined as the degree of adherence to logical rules of data structure, attribution and relationships (data structure can be conceptual, logical or physical). If these logical rules are documented elsewhere (for example in a Product Specification) then the source should be referenced (for example in the data quality evaluation). It consists of four data quality elements:

- Conceptual consistency – adherence to rules of the conceptual schema;

- Domain consistency – adherence of values to the value domains;

- Format consistency – degree to which data is stored in accordance with the physical structure of the dataset;

- Topological consistency – correctness of the explicitly encoded topological characteristics of a dataset.

#### Positional Accuracy

Positional Accuracy is defined as the accuracy of the position of features within a spatial reference system. It consists of three data quality elements:

- Absolute or external accuracy – closeness of reported coordinate values to values accepted as or being true;

- Relative or internal accuracy – closeness of the relative positions of features in a dataset to their  respective relative positions accepted as or being true;

- Gridded data positional accuracy – closeness of gridded data spatial position values to values  accepted as or being true.

#### Thematic accuracy

Thematic Accuracy is defined as the accuracy of quantitative attributes and the correctness of non-quantitative attributes and of the classifications of features and their relationships. It consists of three data quality elements:

- Classification correctness – comparison of the classes assigned to features or their attributes to a  Universe of Discourse (for example ground truth or reference data);

- Non-quantitative attribute correctness – measure of whether a non-quantitative attribute is  correct or incorrect;

- Quantitative attribute accuracy – closeness of the value of a quantitative attribute to a value  accepted as or known to be true.

The data quality information provided within datasets may include the following:

- Sigma confidence of predictions/model.

Quality of the observed data depends on the accuracy of the water level gauges and their processing techniques, and is normally available in field survey reports or quality controlled analyses. Quality of predicted/forecast data depends on quality, timeliness, and spatial coverage of the input data as well as the mathematical techniques.

#### Temporal quality

Temporal Quality is defined as the quality of the temporal attributes and temporal relationships of  features. It consists of three data quality elements:

- Accuracy of a time measurement – closeness of reported time measurements to values accepted as or known to be true;

- Temporal consistency – correctness of the order of events;

- Temporal validity – validity of data with respect to time.

Temporal accuracy for observational data is normally available in field survey reports or quality controlled analyses. Temporal accuracy for predicted/forecast data is normally described in technical reports.

#### Aggregation

The aggregated Data Quality result provides a result indicating if the dataset has passed conformance to the Data Product Specification. It consists of a single data quality element:

Aggregation Measures – a pass/fail indicator and a numeric ratio of the proportion of Product Specification requirements not passed.

### Applicable quality measures

Table 6-1 below indicates which of the data quality measures recommended in S-97 Part C have been identified as applicable to S-104. Columns 1-4 are taken as-is from S-97; the contents of column 5 are from S-97, annotated with whether the measure applies to S-104 and the scope, if it applies. Note that for attributes which allow fill values (see clause 10.2.2.2) the presence of a fill value is not counted as an error for the purposes of the data quality measures.

Table 6-1 – Quality measures applicable to S-104 (from S-97 Part C, clause 7)

| Data quality measure | Definition | DQ measure / description | Evaluation scope | S-104 applicability |
| --- | --- | --- | --- | --- |
| Completeness / Commission | Excess data present in a dataset, as described by the scope. | numberOfExcessItems / This data quality measure indicates the number of items in the dataset, that should not have been present in the dataset. | dataset / dataset series | Yes (dataset) |
| Completeness / Commission | Excess data present in a dataset, as described by the scope. | numberOfDuplicateFeatureInstances / This data quality measure indicates the total number of exact duplications of feature instances within the data. | dataset / dataset series | Yes (dataset) |
| Completeness / Omission | Data absent from the dataset, as described by the scope. | numberOfMissingItems / This data quality measure is an indicator that shows that a specific item is missing in the data. | dataset / dataset series / spatial object type | Yes (dataset) See clause 6.2 below |
| Logical Consistency / Conceptual Consistency | Adherence to the rules of a conceptual schema. | numberOfInvalidSurfaceOverlaps / This data quality measure is a count of the total number of erroneous overlaps within the data. Which surfaces may overlap and which must not is application dependent. Not all overlapping surfaces are necessarily erroneous. | spatial object / spatial object type | Yes (domain extent polygons only) |
| Logical Consistency / Domain Consistency | Adherence of the values to the value domains. | numberOfNonconformantItems / This data quality measure is a count of all items in the dataset that are not in conformance with their value domain. | spatial object / spatial object type | Yes (spatial object) |
| Logical Consistency / Format Consistency | Degree to which data is stored in accordance with the physical structure of the dataset, as described by the scope | physicalStructureConflictsNumber / This data quality measure is a count of all items in the dataset that are stored in conflict with the physical structure of the dataset. | dataset / dataset series | Yes (dataset) |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | rateOfFaultyPointCurveConnections / This data quality measure indicates the number of faulty link-node connections in relation to the number of supposed link-node connections. This data quality measure gives the erroneous point-curve connections in relation to the total number of point-curve connections. | spatial object / spatial object type | No (Applies only for PS with curves) |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfMissingConnectionsUndershoots / This data quality measure is a count of items in the dataset within the parameter tolerance that are mismatched due to undershoots. | spatial object / spatial object type | No (Applies only for PS with curves) |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfMissingConnectionsOvershoots / This data quality measure is a count of items in the dataset within the parameter tolerance that are mismatched due to overshoots. | spatial object / spatial object type | No (Applies only for PS with curves) |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfInvalidSlivers / This data quality measure is a count of all items in the dataset that are invalid sliver surfaces. A sliver is an unintended area that occurs when adjacent surfaces are not digitised properly. The borders of the adjacent surfaces may unintentionally gap or overlap to cause a topological error. | dataset / dataset series | Yes (domain extent polygons only) |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfInvalidSelfIntersects / This data quality measure is a count of all items in the dataset that illegally intersect with themselves. | spatial object / spatial object type | Yes (domain extent polygons only) |
| Logical Consistency / Topological Consistency | Correctness of the explicitly encoded topological characteristics of the dataset, as described by the scope. | numberOfInvalidSelfOverlap / This data quality measure is a count of all items in the dataset that illegally self-overlap. | spatial object / spatial object type | Yes (domain extent polygons only) |
| Positional Accuracy / Absolute or External Accuracy | Closeness of reported coordinative values to values accepted as or being true. | RMSError / Standard deviation, where the true value is not estimated from the observations but known a priori. | spatial object / spatial object type | No (S-104 allows only regular grids and does not estimate coordinate values) |
| Positional Accuracy / Vertical Position Accuracy | Closeness of reported coordinative values to values accepted as or being true. | linearMapAccuracy2Sigma / Half length of the interval defined by an upper and lower limit in which the true value lies with probability 95%. | spatial object / spatial object type | No (S-104 allows only 2D grids) |
| Positional Accuracy / Horizontal Position Accuracy | Closeness of reported coordinative values to values accepted as or being true. | linearMapAccuracy2Sigma / Half length of the interval defined by an upper and lower limit in which the true value lies with probability 95%. | spatial object / spatial object type | No (S-104 allows only regular grids) |
| Positional Accuracy / Gridded Data Position Accuracy | Closeness of reported coordinative values to values accepted as or being true. | RMSErrorPlanimetry / Radius of a circle around the given point, in which the true value lies with probability P. | spatial object / spatial object type | Yes (spatial object) |
| Temporal Quality / Temporal Consistency | Consistency with time. | chronologicalOrder / Correctness of ordered events or sequences, if reported. | dataset / dataset series / spatial object type | Yes (Applies to timestamps, time attributes and time intervals in spatial objects) |
| Thematic Accuracy / ThematicClassificationCorrectness | Comparison of the classes assigned to features or their attributes to a universe of discourse. | miscalculationRate / This data quality measure indicates the number of incorrectly classified features in relation to the number of features that are supposed to be there. [Adapted from ISO 19157] This is a RATE which is a ratio, and is expressed as a REAL number representing the rational fraction corresponding to the numerator and denominator of the ratio. For example, if there are 1 items that are classified incorrectly and there are 100 of the items in the dataset then the ratio is 1/100 and the reported rate = 0.01. | dataset / dataset series / spatial object type | Yes (dataset) |
| Thematic Accuracy / Quantitative Attribute Accuracy | Accuracy of a quantitative attribute | One of attributeValueUncertaintyMean, attributeValueUncertainty68.3, attributeValueUncertainty90, attributeValueUncertainty95, attributeValueUncertainty99, or attributeValueUncertainty99.8 / This data quality measure indicates the attribute value of uncertainty where half the length of the interval defined by an upper and lower limit in which the true value for the quantitative attribute lies with a probability of 50%, 68.3%, 90%, 95%, 99%, or 99.8% respectively. | dataset / dataset series / spatial object type | Yes (dataset / spatial object) |
| Aggregation Measures / AggregationMeasures | In a data Product Specification, several requirements are set up for a product to conform to the Specification. | DataProductSpecificationPassed / This data quality measure is a boolean indicating that all requirements in the referred data Product Specification are fulfilled. | dataset / dataset series / spatial object type | Yes (dataset) |
| Aggregation Measures / AggregationMeasures | In a data Product Specification, several requirements are set up for a product to conform to the Specification. | DataProductSpecificationFailRate / This data quality measure is a number indicating the number of data Product Specification requirements that are not fulfilled by the current product/dataset in relation to the total number of data Product Specification requirements. | dataset / dataset series / spatial object type | Yes (dataset) |

## Additional components of data quality

A time series is complete when there is a value or a null indicator at every time in the series. A water level coverage data set is complete when the grid or point set coverage value matrix contains height value or fill (missing) value for every vertex point defined in the grid, and when all of the mandatory associated metadata is provided. See S-158:104 (Validation Checks – Water Level Information for Surface Navigation) for related checks.

## Assessment of data quality

The prescribed precision (see Annex A – Data Classification and Encoding Guide) of water level (0.01 m) is close to the perceived accuracy of the data.

Important factors in the quality of water level information for navigation consists of the quality of:

- The observed data;

- The predicted/forecast data;

- The positional data; and

- The time stamp.

Factors determining the accuracy of the data are shown in Table 6-2. Information of the quality of the components of the data is normally available in field survey reports, QC analyses, or other technical reports.

Table 6-2 - Data types and accuracy factors

| Type of Data | Factors Influencing Accuracy | Method of evaluation |
| --- | --- | --- |
| Observed water level | Accuracy of the sensors Processing techniques Tide Gauge Initialisation Error (2σ RMSE) | Sensor’s manual Sensor’s manual Van de Casteele test |
| Predicted/forecast Water level | Quality of input data Timeliness of input data Mathematical modelling techniques Accuracy of harmonic constants | Van de Casteele test Bodnard test Harmonic analysis report / Hydrodynamic model report Harmonic analysis report |
| Horizontal Position | Accuracy of geolocation techniques Model grid accuracy | Hydrodynamic model report Hydrodynamic model report |
| Vertical Position | Accuracy of vertical datum | Quality control analysis |
| Time stamp | Sensor accuracy Data time tagging accuracy | Sensor’s manual Sensor’s manual |

Data quality measures for the entire data set are described in Table 12-2. These include horizontalPositionUncertainty, verticalUncertainty, and timeUncertainty. The additional data quality measure for uncertainty in waterLevelHeight is described in clause 10.2.2.4.

## Validation checks

Validation checks are intended for production systems designed to produce S-104 Water Level Information datasets. Validation checks apply to either datasets (HDF5 dataset files) or exchange sets. Validation checks for S-104 datasets and exchange sets are defined in two locations:

- General validation checks for all S-100-based product specifications intended for use on navigation systems are defined in S-158:100 (Validation Checks – Universal Hydrographic Data Model).

- Product-specific validation checks are defined in S-158:104 (Validation Checks – Water Level Information for Surface Navigation).

In addition, there are cross-product compatibility checks intended to verify suitability of combinations of products for use together on ECDIS. These checks will be defined in S-158:98 (Validation Checks – Interoperability).

Validation checks can be administered at any time during the production phase. They can also be applied downstream in the distribution and end user systems to test the conformance of a dataset to the format rules specified in S-100 Part 10c and the S-104 Product Specification.

For example, checks will be made for: inclusion of mandated variables; variable values being within accepted ranges; inclusion of optional values when required; matches between number of array elements and array dimension specifications; timeliness of data; etc. Error severity may be, for example, that the dataset is unusable, that the dataset is of degraded utility but otherwise safe to use, or that the dataset has one or more small and inconsequential inconsistencies.

Fill values must be considered as allowed values for attributes which allow them (see clause 10.2.2.2), even though the fill value will be outside the allowed range in the Feature Catalogue.

Cross-product compatibility checks need to be administered to combinations of S-104 and S-102 or S-101 datasets. Their administration should be coordinated with producers of underlying S-101 or S-102 datasets.

# Data Capture and Classification

The water level product contains data processed from sensors or derived from the output from mathematical models. In most cases, the data collected by the Producing Authority must be translated, sub-setted, reorganised, or otherwise processed to restructure into a usable data format.

## Data sources for water levels

Water level data comes primarily from a few specific sources: observations; astronomical predictions; analyses; and forecast models. When such data are produced and quality-controlled by an approved Producing Authority (IHO Resolutions A6.3 & A6.9, S-62), they are suitable for inclusion in the water level data product.

NOTE: S-104 Edition 2.0.0 uses only data types which the water level adjustment algorithm described in S-98 can process.

Observational Data: Observational water level data comes initially from in situ sensors in the field (for example tide gauges deployed along a channel) and if feasible interpolated or extrapolated values where there are no observations, and are monitored by the data collecting authority. After data acquisition, the data are quality controlled and stored by the Producing Authority. Some of the observed data may be available for distribution within minutes of being collected and are described as being ‘in real time’. Other data may be days or years old, and are called historical data.

NOTE: Observational Data in S-104 Edition 2.0.0 must still conform to the regular grid format.

Astronomical Predictions: Astronomical predictions are produced when a sufficiently long time series of observed water level has been obtained and the data has been harmonically analysed by the Producing Authority to produce a set of amplitude and phase constants. The harmonic values can then be used to predict the astronomical component of the water level as a time series covering any desired time interval. Astronomical predictions can also be produced by other proven methods of tidal analysis. Data available for single stations or numerous stations must be arranged by the Producing Authority into a gridded field to conform to this edition of S-104.

Analysed and Hybrid Values: Analysed water level values may be derived from sea-surface topography, data assimilation, statistical correlations or other means. A hybrid method combines two of or more approaches.

Forecast Data: Hydrodynamic models numerically solve a set of fluid dynamic equations in two or three dimensions, and rely on observational data, including water levels and winds, to supply boundary conditions. Model grids may be either regular or irregular. Such models are often run several times per day. The forecast is a simulation made for many hours into the future using predicted winds, water levels, etc. The results are saved for a limited number of times, and are stored as arrays that derive from the model’s grid. These models and methods are developed, run and monitored by the Producing Authority.

These descriptions are summarised in Table 7-1.

Table 7-1 - Types of water level data, based on the source of the data

| Name | Description |
| --- | --- |
| Real-time observation | Observation no more than a few minutes old |
| Astronomical prediction | Value computed using harmonic analysis or other proven method of tidal analysis [IHO Res. 3/1919, as amended] |
| Analysis or hybrid method | Calculation by statistical or other indirect methods, or a combination of methods |
| Forecast | Gridded data from a two- or three-dimensional dynamic simulation of future conditions using predicted data for boundary forcing, via statistical method or combination |

### Determination of trend

When the average change of the water level over a defined period (time window) is greater than or equal to a threshold set by the Producing Authority it is considered “increasing”. When it is less than or equal to the negative of the threshold it is “decreasing”. When it is between the double-sided range (+/- threshold) set by the Producing Authority, it is “steady”.

The default time window over which the trend is calculated is 60 minutes. However, the 60 minutes window may not be ideal for all locations (for example, locations where there is no tide versus locations with strong tides) and producers may use a different window, in which case the window must be encoded in the trendInterval metadata attribute.

In areas of small water level range, for example Baltic Sea, use of “not available” is optional.

The parameters used by the Producing Authority in determining the trend are the threshold value and the time interval. These parameters can be encoded in the metadata attributes waterLevelTrendThreshold and trendInterval respectively. These attributes are allowed at the general metadata level and the Values group level. When encoded at the Values group level they apply only to the particular feature for which they are encoded and override the values encoded in the general metadata level.

Though the Producing Authority may determine the threshold and interval used for calculating trend, waterLevelTrendThreshold must be reported in metres per hour. The actual interval (in minutes) is reported in attribute trendInterval.

EXAMPLE: For a threshold of 0.3 metres over a 3-hour interval, waterLevelTrendThreshold is reported as 0.1 and trendInterval as 180.

This edition of S-104 does not specify the precise temporal relationship between intervals for calculating trend and the time points at which water levels are reported in the dataset.

## The production process

Nearly all available information on water level from the Producer must be reformatted to meet the standards of this Product Specification (Figure 10-1 – the S-104 format). This means (a) populating the carrier metadata block (clause 12.3) and values group attributes (Table 12-4) with the relevant metadata and (b) reorganizing the water level data when using the encoding rules (see clause 10).

### Metadata

Metadata is derivable from the information available from the approved Authority. The following variables will require additional processing:

- The bounding rectangle is computable from grid parameters. Note that the bounding box is encoded in both carrier metadata at the root group level and in the discovery metadata block (attribute dataCoverage) in the Exchange Catalogue, and must be the same in both places.

- Position uncertainties may be available from the approved Authority’s metadata.

- Water level uncertainty may be available from the prediction or forecast model, specification of the water level gauge or calculated from observations.

- If a previously issued data file is being cancelled or replaced, the replacedData and/or dataReplacement attributes in the Exchange Catalogue must be populated.

All mandatory metadata in carrier metadata (clause 12.3) must be populated with appropriate values. In cases where the attribute is mandatory but inapplicable, the appropriate fill or null value described in clause 12.3 must be used.

Similarly, when the Exchange Set is being compiled, all mandatory metadata or information fields in the discovery metadata and Exchange Catalogue (clauses 12.1 and 12.2) must be populated. In cases where the attribute is mandatory but inapplicable, or the value is unknown or not included in the relevant enumeration list, the appropriate fill or null value described must be used.

NOTE (informative): Running the S-100 level validation checks and product-specific validation checks should detect missing metadata, but as of May 2024 the checks are yet to be completely defined and automated, and visual checking of metadata may be necessary. The Tables in clauses 12.2 and 12.3 describe the mandatory requirements and allowed values.

### Water Level Data

Observational water level, tidal water level predictions and gridded forecast data must normally be reformatted to fit the S-104 Product Specification. The following may require additional calculations:

- For gridded data, if a land mask array is included, the mask value is substituted into the gridded values as appropriate (see A-2).

- Time stamps must be encoded as UTC.

### Validation

Dataset and Exchange Set validation tests must be passed before the Exchange Set is published.

For numeric attributes, the fill value will be outside the allowed range of values specified in the Feature Catalogue, if any. Similarly, for enumerations, the fill value will not be a member of the enumeration as listed in the Feature Catalogue. Validation checks for datasets must allow for the presence of fill values.

Validation must apply both the S-100 level validation checks defined in the S-100 validation specification (only those checks applicable to S-104 need be applied) and the product-specific validation checks provided in S-158:104.

### Digital Signatures

Digital signatures are required for datasets and exchange sets intended for use on ECDIS. S-100 Part 15 describes the required signature algorithm and procedure for creating signatures. S-100 Part 17 describes where signatures must be provided. Additional guidance common to all datasets and exchange sets intended for ECDIS is being developed by the IHO. In the absence of this common guidance, the following guidance applies to S-104 datasets and exchange sets:

- The signature algorithm must be as specified in S-100 Part 15.

- In discovery metadata, the S100_SE_SignatureOnData element should be used to encapsulate digital signatures for datasets, with the dataStatus attribute set to unencrypted or encrypted according to whether the signature is for an unencrypted or encrypted HDF5 file.

- All resources in the exchange set must be signed, including any catalogue(s) and support files.

- At least one signature is required for each resource (dataset, catalogue, or support file) in the exchange set (the ECDIS will ignore unsigned resources or resources for which signature verification fails).

- Additional signatures may optionally be provided, or added downstream in the distribution chain, as provided for in S-100 Parts 17 and 15.

## Guidance for chunking and compression (informative)

Chunking affects both dataset size and optimised data retrieval, the latter in the sense of how an ECDIS would most efficiently retrieve relevant chunks of a dataset when a user pans and zooms.

Product Specification developers may desire to assess typical profiles and volumes of data for their datasets and develop guidance for the use of chunking and compression in their data products. Common practice is provided below. Product teams should assess its applicability to their own products and use, omit, and adapt it accordingly.

The development of guidance on how to optimally and correctly do chunking and compression is ongoing; however, current best practice is:

- For gridded data with 2 dimensions, for example dataCodingFormat = 2 (regular grids), choosing roughly-square rectangular chunk sizes will result in better performance when reading subsets of the data, and will probably result in better compression (one reason being that because NoData areas tend to be clustered together geographically, geographically-tiled chunks will compress out all those repetitive values).

- Producers may use "auto-chunking", where this functionality is available (for example, in the production toolset’s HDF5 library). Auto-chunking will choose chunk sizes automatically.

- Choosing the right chunk sizes depends on the type of data and what the use of chunking is trying to accomplish. Auto chunking is more ideal for compression and is less ideal for time-critical access patterns.

Auto-chunking means different datasets may be chunked differently. Applications cannot expect a standardised chunk size and will have to handle whatever chunk sizes they encounter in datasets.

Data Producers should note experiences from preliminary testing:

- 2D arrays – Need to be chunked based on how the data is read. If applications need to hold the entire grid in memory, use no chunking; otherwise estimate a reasonable size for data extraction.  It is probably better to have the chunking set a little smaller than to make it too big, for I/O purposes.

- 1D arrays – Do not chunk unless they are enormous (for S-104 this is not an issue since clause 11.2.1 limits datasets to well below the size where chunking matters).

- Given the relatively small sizes of datasets for S-104 chunking will not be of great benefit in read performance for S-104.

Producers should determine the compression scheme that is optimal for their own use case, as needed.

## Datasets in a series

Datasets in a time series (for example, 4X daily, 1X daily, etc.)  may be distributed by any appropriate means, such as transfer to an accessible Internet service or via a licensed distribution channel.

Each release by the producer should be accompanied by an exchange catalogue and bear the appropriate producer digital signatures as specified in S-100 Part 17 and S-98.

Route monitoring applications require up-to-date water level information and periodic forecasts should be issued in a timely manner (meaning, a successor dataset should be released before the expiry of one full period after the starting date and time of its predecessor).

Multi-pack exchange sets containing multiple sequential datasets may also be prepared as determined necessary by the producer, for example, for uses other than route monitoring on ECDIS. For multi-packs a single exchange catalogue containing discovery metadata for all datasets should be prepared

## Data use purpose

### Datum requirements

Datasets intended for use in navigation must use the same CRS as the underlying S-102 and ENC. Particular care should be taken to ensure that the horizontal and vertical datum are the same as the underlying S-102 and ENC (with preference for S-101 over S-57). The epoch of realization should be included in this assessment.

NOTE: Conformant datums are a requirement for display on ECDIS, and water level adjustment as described in S-98 (see also Annex D).

### Spatial type

All datasets intended for use in navigation on ECDIS must be issued as regular grids.

### Suitability for navigation

Datasets may be marked for use in navigation if the Producer is able to consistently produce data of quality corresponding to the same zone of confidence as the underlying ENC. The Zones of Confidence are described in S-101 Annex A – Data Classification and Encoding Guide (attribute “category of zone of confidence in data” - CATZOC).

Alternatively, datasets may be marked for use in navigation if the Producer is able to consistently produce data along with their uncertainties.

Producers should note that combining S-104 data of lower accuracy with S-101 or S-102 data of higher accuracy may degrade the certainty of information available to the mariner.

### Use purpose metadata

Datasets not intended for navigation purposes must have the discovery metadata attribute notForNavigation in the corresponding S100_DatasetDiscoveryMetadata block set to true.

Datasets intended for navigation must have the discovery metadata attribute notForNavigation in the corresponding S100_DatasetDiscoveryMetadata block set to false.

## Compliance categories

Compliance categories are described in S-100 clause 4a-5.5. Datasets intended for use on ECDIS must meet the requirements for category4 and the compliance category must be encoded accordingly.

## Compliance with S-98

S-98 Edition 3.0.0 includes a specification for the use of S-104 data to be used with S-102 for adjusting depths values for ECDIS display and pick reports (S-98 Appendix D). The requirements for datasets to be compliant with each aspect of interoperability are described below. Compliance to this Edition of S-104 is a fundamental requirement and will not be explicitly listed.

### Requirements for visual interoperability

S-104 Edition 2.0.0 datasets are intended only for use with water level adjustment as described in S-98 and therefore this Product Specification does not specify requirements for visual interoperability.

### Requirements for harmonised user experience

S-104 Edition 2.0.0 datasets are intended only for use with water level adjustment as described in S-98 and therefore this Product Specification does not specify requirements for achieving a harmonised user experience other than those required for water level adjustment functionality (clause 7.7.3).

### Requirements for water level adjustment

- The horizontal and vertical datums must be the same as the horizontal and vertical datums of the underlying S-102 (Bathymetry) dataset(s). In case of conflict between datums used in S-101 and S-102, the respective data producers and the TWCWG Chair should be consulted.

- Requirements pertaining to maximum and minimum resolution of S-104 grids in relation to the underlying S-101 or S-102 datasets must be met (clause 7.10).

- There must be no spatial overlap between S-104 datasets created by the same producer and with overlapping scale ranges in discovery metadata, with the exception of datasets in the same temporal series, which must have the same spatial extent.

- Temporal overlap for datasets with intersecting scale ranges is permitted only for datasets which are members of the same temporal series, when a forecast for a specific period is followed by a forecast for a later period.

- Other checks for cross-compatibility of S-101/S-102 and S-104 datasets should also be satisfied. These checks will be defined in S-158:98 (Validation Checks – Interoperability).

## Datasets with multiple vertical datums

In places where different regions of a quadrilateral grid coverage have water level information based on different vertical datums, the regions with different vertical datums must be encoded in the same HDF5 datafile as different WaterLevel feature instances.

If the data are referenced to a local vertical datum, the name and epoch of the datum should be specified. If the dataset contains areas where the datum is calculated from different epochs and the magnitude of these differences exceeds 10 cm, the epoch of both datums should be specified. Epochal differences should be included in the uncertainty.

The default vertical datum is specified in the root group (see Table 12-1). Feature instance groups with a vertical datum that differs from the default vertical datum also encode the attributes verticalDatum and verticalDatumReference on the feature instance group (Table 12-3). Each feature instance, including any instances using the default vertical datum, must have its in-coverage extent (region covered by the vertical datum) coded using a domainExtent.polygon polygon (see Table 10-2 and S-100 Table 10c-11).  Data for grid points outside the domain extent must consist of fill values. Points coinciding with the boundary of the domain extent polygon are considered to be within the polygon.

There is no requirement for boundaries of domain extent polygons to coincide with grid cell boundaries.

All WaterLevel instances in a dataset containing multiple vertical datums must use the same grid extents and grid parameters.

Domain extent polygons for vertical datums must conform to the rules for Level 3a geometry in S-100 Part 7. In addition, the format for domain extent polygons defined in S-100 Part 10c does not permit encoding domain extent polygons as multi-polygons or with holes, and interoperability with other data products is also desirable. Consequently, the following requirements apply to domain extent polygons:

- The sequence of polygon vertex coordinates must define a closed loop boundary curve beginning and ending with the same point (see S-100 clause 7-4.3.2).

- The boundary curve must not self-intersect (see S-100 clause 7-4.3.2).

- Exterior boundaries must be in the clockwise direction, that is, the vertices must be ordered so that the interior is to the right of the curve (see S-100 clause 7-4.3.2).

- If a vertical datum area is naturally a multi-polygon, each separate polygonal patch must be encoded as a separate feature instance with a distinct domain extent polygon.

- If a vertical datum area has holes where other vertical datums are used, the enclosing area may be cut and/or partitioned into separate simple polygons so as to avoid the need for interior boundaries.

- Land areas which will be always populated with fill values may be allocated to datum polygons as determined by the producer. It is suggested that allocation of land areas to domain extent polygons can be such as to simplify domain extent polygons as well as avoid unnecessary holes for land areas, provided that grid points on the land areas allocated to a domain extent polygon are always populated with fill values.

- Domain extent polygons must not extend beyond the geographic extent of the grid.

- Teams producing S-101, S-102, and S-104 should coordinate to ensure that domain extent polygons in S-102 and S-104 datasets match sounding datum features in the S-101 dataset in areas where two sounding datum features adjoin in the S-101 dataset.

NOTE: In this edition of S-104, domain extent polygons are not intended for portrayal on ECDIS.

## Construction of coverages

### Grid cell structure

S-104 provides two alternative approaches to grid cell structure:

- Producers may use the S-100 Part 8 and ISO 19123 convention that grid data are nominally situated exactly at the grid points defined by the grid coordinates. This convention makes the grid points the “sample points”, representing data over a neighborhood extending a half-cell in each direction (S-100 Part 8 clause 8.2.5.8). If this convention is followed, the attribute dataOffsetCode (clause 12.3.2) should not be encoded.

- Producers may construct grids where the “sample points” are located at the centres of grid cells, in which case dataOffsetCode must be encoded with value “5: Barycenter (centroid) of cell” (clause 12.3.2).

Note that a grid with 100x100 cells will have a nominal total of 101x101xnumberOfTimes value records in the first case and 100x100xnumberOfTimes value records in the second. See clause 10.2.2.7 for the rules specifying the dimensions of the values array for each convention.

## Scale ranges

### Maximum, minimum, and optimum display scales in discovery metadata

Suggested scale ranges for datasets are listed in Table 7-2 below, generally corresponding to the values of attribute navigationPurpose in dataset discovery metadata. Data Producers should decide what ranges are suitable for their data and are not required to use the ranges in this table. As a rule of thumb, scales may be aligned with scales specified for underlying S-102 datasets.

The ranges are selected from the list of allowed values for S-101; if a later edition of S-101 changes the values, corresponding adjustments should be made in S-104.

Table 7-2 Suggested dataset scale ranges

| Navigation purpose | maximumDisplayScale | optimumDisplayScale | minimumDisplayScale |
| --- | --- | --- | --- |
| port | 1 000 | 4 000 | 12 000 |
| transit | 12 000 | 45 000 | 90 000 |
| overview | 90 000 | 180 000 | 350 000 |
| overview | 350 000 | 700 000 | 1 500 000 |
| overview | 1 500 000 | 3 500 000 | 10 000 000 |

Production of datasets at scales smaller than 1:350000 should be considered in the light of their potential utilisation, for navigation or other uses. For example, S-104 water level information will not be used for route monitoring or route planning at oceanic depths nor can S-104 datasets be used for water level adjustment on ECDIS in regions where no S-102 bathymetric dataset is produced.

### Grid resolutions

The grid resolution for S-104 data must not be higher than the resolution for the underlying S-102 dataset (that is, S-104 grid cells must not be smaller than S-102 grid cells). If there are multiple underlying S-102 datasets in different scale ranges, this rule applies to S-104 and S-102 datasets with intersecting scale ranges.

## Bounding polygon guidelines

Bounding polygons for grid features should be the same as the spatial extent of the grid, except that large regions uniformly and permanently populated with fill values (such as large land areas) may be excluded. The boundaries are left to the discretion of the data producer, but overly-detailed boundaries and small interior holes are discouraged.

A boundingPolygon is restricted to a single GML Polygon with one exterior and 0 or more interiors expressed as Linear Rings using SRS EPSG:4326. The exterior and optional interiors shall be composed of a closed sequence of >=4 coordinate positions expressed individually or as a list (posList). The GML polygon shall have a valid GML identifier.

Bounding polygons for underlying S-102 datasets may be used as a starting point for S-104 bounding polygons.

If the extents and grid parameters are identical in the S-102 and S-104 grids, the S-102 bounding polygon(s) may be taken as a starting point and adjusted to enclose all S-104 grid data point neighborhoods populated with non-fill values.

If the S-102 and S-104 grids cover different extents but the S-104 grid lines coincide with grid lines of underlying S-102 grids, spatial unions of adjoining S-102 bounding polygons may be created, adjusted to include populated S-104 grid data point neighborhoods, and then clipped by the S-104 grid extents.

If there is no S-102 data underlying all or part of the S-104 grid, convex hulls of grid data point neighborhoods populated with non-fill values may be used to generate the exterior boundaries of bounding polygons. Alternatively, Sounding Datum features or skin of the earth water features in an underlying ENC (Depth Area and Dredged Area) may be used as a starting point, with Unsurveyed Area features also included to the extent that S-104 has populated grid points overlying an Unsurveyed Area.

In all cases, contiguous regions of adjacent grid data point neighborhoods populated with fill values may be used to generate interior boundaries of bounding polygons. Only significant contiguous regions (such as contiguous regions where S-102 data defines an interior boundary or with more than 10 S-104 adjacent data values) need be used to create interior boundaries.

Since S-104 is a time-series product, areas that are populated with non-fill values at any time point must be included, even if they dry or otherwise have fill values at other times.

Bounding polygons must include all populated grid data point neighborhoods and should remain the same for successive datasets for the same area as long as the grid parameters and extent remain the same. Bounding polygons should therefore be pre-computed over a sufficient period covering multiple successive datasets (at least 1 year is recommended).

For dataOffsetCode = 1 (nearest neighbor) the data point neighborhood is the neighborhood of each grid point (grid lines intersections), since data points coincide with grid points, clipped by the grid extent. For dataOffsetCode = 5 (barycenter) the data point neighborhood is the grid cell.

# Maintenance

## Overview of dataset maintenance

Maintenance and Update Frequency: Water level is always moving, so more-or-less- continual revision or updating of the data is essential. For real-time observations, new values are periodically collected (for example every 6 minutes). For a forecast, the entire field of water levels is created one or more times per day. New issues of real-time observations or forecasts should be considered updates.

Table 8-1 summarises this information.

Table 8-1 – Typical update/revision intervals and related information for S-104 products produced by a single Producer.

| Data Types | Update Interval | Number Of Spatial Locations | Number Of Time Values Per Location |
| --- | --- | --- | --- |
| Astronomical Predictions | 1 year | Depends on grid extent and resolution | 52560 (10 minute data) or 8,760 (hourly data) |
| Model Forecasts | 6 hr | 100,000 to 1,000,000 | 1 to 24 |
| Real-time Observations | 0.1 hr | Depends on grid extent and resolution | 1 to 240 |

NOTE: Population of the resource maintenance information in metadata will indicate to the ECDIS when new data can be expected to be available.

Data Source: Data is produced by the Producing Authority through the collection of observed values, predicting astronomical tides, or running analysis or forecast. This data is typically quality-controlled and reformatted to conform to file size limitations and the S-104 Standard encoding.

Production Process: S-104 datasets, including the metadata and the coverages for water level, are updated by replacement of the entire data product. Producers routinely collect observational data and maintain an analysis and/or forecast capability. When new data becomes available (often several times per day), the data is reformatted and made available for dissemination.

### Update of harmonic constants (informative)

Water level harmonic constant data are updated much less often than predictions, typically on an annual basis. Harmonic constants change their values if the environment changes (typically the bathymetry). Since this rarely happens an update of the harmonic constants has to be made only in rare occasions. If a long time series of level data is available, a statistical analysis of the harmonic constants can be made in order to use their standard deviations to decide if an update is really necessary, for example if their differences exceed three times the standard deviations.

Water level harmonic constant data should be updated only if their values differ from previous ones by a given amount (for example three times their standard deviations).

Since this Edition of S-104 does not include harmonic constants in datasets, updates to harmonic constants will affect S-104 datasets only as and when updated harmonic constants are used in generating S-104 datasets. Producers may use the comment attribute of dataset discovery blocks in the Exchange Catalogue to indicate which version of harmonic constants was used for the dataset.

## Metadata related to dataset maintenance

### Elements used in S-104

S-100 metadata related to maintenance that may be used in S-104 metadata consists of metadata elements specifying:

- The purpose (of issuing the dataset);

- The Edition number of the dataset;

- When its successor will be available;

- The issue date and time of the dataset; and

- The date of the metadata record for the dataset.

Some types of S-104 datasets use only some of these elements. Clauses 8.2.2 ff. provide guidance for selecting the values of these elements for the corresponding discovery metadata blocks in the Exchange Catalogue.

### New datasets

#### Classification as new datasets

S-100 Part 17 (Table S100_Purpose) defines a new dataset as a “Brand new dataset” with a remark clarifying that “No data has previously been produced for this area”. The factors that should be considered in determining whether a dataset should be classified as a new dataset are:

- Whether any S-104 water level datasets are currently being produced for the region.

- Whether a new type of water level information (Table 7-1) is being made available. For example, if real-time observations are made available for a region where only astronomical predictions were formerly issued, the real-time dataset should be considered a new dataset.

- Changes to spatiotemporal representations:

  - Changes in the grid spacing for gridded data (without changing the grid extent)  should not be considered a new dataset.

  - Minor changes to the time interval in time series data should not be considered a new dataset. For example, changing from six-hourly to three-hourly forecasts need not be considered a new dataset.

  - Minor adjustments to spatial extent such as a small adjustment to a grid’s boundaries should not be considered new datasets.

  - Significant adjustments to spatial extent or time interval should be considered for classification as a new dataset.

  - The determination of whether an adjustment to spatial extent or time series interval is minor or significant is left to the Producer.

  - Changes to vertical datum areas without changes to the extent of the grid as a whole should not be considered a new dataset.

- Additional factors: The effect on the end user, change of designation (“not for navigation” vs. “for navigation”), change of navigation purpose, effects on data distribution and data management on ECDIS.

- Local factors, such as the S-104 cell scheme used by the Producer.

In case of doubt, Producers are encouraged to seek advice from the TWCWG.

#### Metadata for new datasets

Dataset discovery metadata (clause 12.2) for a new dataset must be encoded as follows:

- editionNumber in dataset discovery metadata (clause 12.2.4) must be 1.

- purpose in dataset discovery metadata must be newDataset.

### New Editions

#### Classification as New Edition

S-100 Part 17, clause 17-4.5 (S100_Purpose) states that a New Edition “Includes new information which has not been previously distributed by updates”.

New Editions of S-104 datasets are issued either when part or all the dataset is erroneous and must be replaced, or when better data become available. In either case, the dataset is replaced as a whole.

New Editions are also issued when feature instances in the dataset are introduced, revised, or deleted (including introduction, revision, or deletion of domain extent polygons) without changing the spatial and temporal extents of the dataset as a whole.

New Editions are to be used for successor datasets (for example, when a forecast for a specific period is followed by a forecast for a later period). This also applies if the successor dataset consists of feature instances or domain extent polygons different from its predecessor.

#### Metadata for new editions

For a New Edition, set:

- purpose = newEdition

- edition number: increment by 1. New Editions can increment by more than 1 and may or may not be sequential. A higher edition number constitutes a newer dataset.

### Cancellations

#### Classification as cancellation

S-100 Part 17, clause 17-4.5 (S100_Purpose) states that a cancellation “Indicates the dataset or Catalogue should no longer be used and can be deleted”.

S-104 datasets are cancelled only when a dataset or data sequence (such as a sequence of forecasts) is terminated. This might happen for various reasons, for example if Producers reorganise their S-104 cell scheming or replace one type of water level information by another.

Cancellation of a dataset in S-104 is interpreted consistently with S-100 Part 17, with additional conditions arising from the time-dependent nature of water level datasets. Cancellation of an S-104 dataset must be treated as described below:

- Water level information in the cancelled dataset for times beginning and after the effective date and time of cancellation must not be used. The effective date and time are the issue date and time in the discovery metadata for the cancellation.

- Water level information in the cancelled dataset for times preceding the effective date/time of cancellation may be used only in the absence of an uncancelled dataset covering the area and time in question.

- Cancellation of a dataset that is part of a sequence also cancels the sequence. The sequence should be treated as terminated - there can be no successors to the cancelled dataset in the same sequence.

- There may be a successor sequence that starts with a new dataset. If there is, the fields dataReplacement and replacedData should be populated accordingly in the cancellation record.

NOTE: Populating dataReplacement and replacedData.provides the end-user system with sufficient information to supersede a cancelled dataset with its replacement and avoid spurious errors for apparently overlapping datasets.

- If a replacement dataset is being issued, producers should consider whether there will be a temporal discontinuity between the cancelled and replacement datasets. A temporal discontinuity means that water level adjustment cannot be calculated during the time gap. Maintaining temporal continuity requires the following:

  - the replacement dataset be available when the cancellation takes effect, and

  - data records in the replacement dataset begin no later than the issue date and time of the cancellation.

- If a sequence is being cancelled, cancellations should be issued simultaneously for all datasets in the sequence whose temporal extents overlap the time the cancellation is issued. (This ensures that cancelling a forecast dataset (for example) does not leave active predecessor forecasts which extend after the time of cancellation.)

- The retention, archiving or removal of cancelled datasets or sequences from the system must be according to the common principles for retention, archiving or removal set forth in S-98 or other applicable documents.

Cancellations should not be issued for time-expired forecast or prediction datasets unless the relevant dataset series is being terminated. If the series is being terminated a cancellation must be issued so the transfer/distribution system is informed whether or not it should expect or attempt to obtain successors or replacement datasets.

#### Metadata for cancellation

S-104 uses the fileless cancellation method described in S-100 5.2.1 Part 17 clause 17-4.4.1:

Fileless cancellation may be achieved by using a dataset metadata entry with the filename and original digital signature specifying the resource to be cancelled, and with all other mandatory metadata fields also set to the same values as the original, with the exception of the issueDate, which must be set to the issue date of the fileless cancellation itself.

The “dataset metadata entry” means the S100_DatasetDiscoveryMetadata block in exchange catalogues. For a cancellation, set:

- fileName = fileName of the cancelled dataset

- digitalSignatureValue = (same as that of the cancelled dataset)

- purpose = cancellation

- editionNumber = (same as that of the cancelled dataset)

- issueDate and issueTime = the issue date and time of the cancellation

- replacedData = true if and only if the cancelled dataset or sequence is replaced by another dataset/sequence; otherwise false. This attribute must be populated for a cancellation.

- dataReplacement = fileName of the replacement dataset (if and only if the cancelled dataset/sequence is replaced by another dataset/sequence). This attribute must be populated when replacedData=true.

- all other mandatory attributes to the same values as in the discovery metadata block for the dataset being cancelled.

NOTE (informative): fileName means the fileName entry in S100_DatasetDiscoveryMetadata, which may not be identical to the dataset file name (for example, fileName may be a URI that includes the data file name as a component – see S-100 Part 17).

#### Production of a cancellation

S-104 uses only the fileless cancellation method described in S-100 Part 17. In order to cancel a dataset, the cancelling authority (generally the producer of the original dataset) must:

- Prepare an exchange catalogue with an S100_DatasetDiscoveryMetadata block with field values as described in clause 8.2.4.2).

- If a sequence of datasets is being terminated, ensure that the exchange catalogue also contains dataset discovery metadata blocks cancelling any predecessor datasets in the same sequence whose temporal extents include the expected issue time for the cancellation.

- Complete other parts of the exchange catalogue as required by clause 12.2 (for example, provide discovery metadata for a replacement dataset if such is included in the same exchange set).

- Sign and distribute the exchange catalogue in a normally structured exchange set. Do not include HDF files for the cancelled datasets in the exchange set.

### Other S100_Purpose values (informative)

S-104 does not use the reissue, update and delta values of the S100_Purpose enumeration.

S-100 Part 17, clause 17-4.5 (S100_Purpose) states that a re-issue “Includes all the updates applied to the original dataset up to the date of the re-issue. A re-issue does not contain any new information additional to that previously issued by updates”. Since S-104 does not include a format for dataset updates, S-104 datasets are not reissued. Corrections to datasets, if required, should be addressed by cancellation of the old dataset accompanied by a New Edition with the same name and an incremented Edition number.

S-100 Part 17, clause 17-4.5 (S100_Purpose) states that an update is for “Changing some information in an existing dataset”. S-104 does not provide for replacing part of a dataset; instead, if changing information is necessary, the whole dataset is replaced.

### Maintenance of support files

The Edition number is 1 for the first issue of a support file for a particular dataset. In the event that the file is updated or replaced (for example for a correction) the Edition number is incremented by 1.

The Edition number for language packs changes if and only if the language pack file is updated.

### Encoding update frequency

The encoding of information about when the next update to a dataset will be released (cf. clause 12.2.4 - resourceMaintenance) is mandatory for datasets that are updated or replaced monthly or more frequently.

The provision of this information for datasets that contain information updated less frequently than monthly (such as astronomical predictions when these are issued annually) is left to the discretion of the Producer, with the expectation that if it is not encoded in discovery metadata the expected release dates will be communicated to distributors and end-users by other means.

For datasets that are continually updated on a fixed cycle, timing information should be encoded as the interval from the issue date and time of the current dataset (that is, using the userDefinedMaintenanceFrequency sub-attribute - cf. S-100 Part 17, clause 17-4.9).

Data Producers or metadata compilers must consult S-100 clause 17-4.9 for the rules on encoding information about release timing.

# Portrayal

The display of gridded water level data is controlled by the underlying ENC or bathymetry product because S-104 data is used to adjust depth and uncertainty values in those products. This edition of S-104 therefore does not provide a portrayal catalogue or describe symbology. Further, the only interoperability functionalities described by S-98 which apply to S-104 are those pertaining to water level adjustment. These functionalities are described in S-98 Appendix D.

NOTE 1: Specification of the algorithm for adjustment of bathymetry data or ENC feature data is outside the scope of this Product Specification. It is described in S-98.

NOTE 2: There are no specifications for the display of a water surface from gridded data as a distinct layer in this Product Specification Edition.

Pick reports on S-104 data are encouraged to display the following information in addition to adjusted data values:

Data dynamicity (Table 12-3)

Water level trend at the current time (current simulated time, if the system is doing a time-varying simulation or animation).

# Data Product Format (Encoding)

## Introduction

The water level data products must be encoded using one of the listed formats. The structure of the data product is discussed in clause 10.2. There is only one format allowed to encode data.

Format:		HDF5 for water level height and trend data

Character Set:	MD_CharacterSetCode (ISO19115-1) must be utf8

Specification:	S-100 profile of HDF5

## HDF5 product structure for time series and gridded data

The key idea at the core of the structure is this: The organisation of the information is substantially the same for each of the various types of data, but the information itself will be interpreted differently.

### Data type definition

HDF5 will be used for all water level data types.

Format Name:	HDF5

Character Set:	MD_CharacterSetCode (ISO 19115-1). All strings must be UTF-8.

Specification:	S-100 profile of HDF5

The S-100 HDF5 format is designed to be flexible enough to apply to coverage data in various forms, including: (a) data at one or more times for one or more individual, fixed stations, organised by time or station; (b) regularly-gridded data for one or more times; (c) ungeorectified gridded data for one or more times; and (d) TIN data. Since each type of data is structured differently, the type of data must be identified by the variable dataCodingFormat. Since S-104 contains only one format (regularly-gridded data), only one of the dataCodingFormat values allowed by S-100 is used in S-104, as shown in Table 10-1. (The letter in parentheses in the second column references the types listed earlier in this paragraph.)

Table 10-1 – S-104 data type and value of the variable dataCodingFormat (see S-100 Edition 5.2.1, Clause 10c-10.1)

| dataCodingFormat | Type of Data |
| --- | --- |
| 2 | Regularly-gridded data at one or more times - type (b) |

For the use of HDF5, the following key concepts (S-100 Part 10c, clause 10c-5.1) are important:

- File - a contiguous string of bytes in a computer store (memory, disk, etc), and the bytes represent zero or more objects of the model;

- Group - a collection of objects (including groups);

- Dataset - a multidimensional array of data elements with attributes and other metadata;

- Dataspace - a description of the dimensions of a multidimensional array;

- Datatype - a description of a specific class of data element including its storage layout as a pattern of bits;

- Attribute - a named data value associated with a group, dataset, or named datatype;

- Property List - a collection of parameters (some permanent and some transient) controlling options in the library;

- Link - the way objects are connected.

In addition, a dataset may have one, two, or more dimensions, and each element in the dataset may be a compound. That is, each element may itself be an array of possibly different datatypes (float, integer, string, etc).

For all data types, the product structure in HDF5 includes: (a) a metadata block; which is followed by (b) one or more Groups which contain the actual water level data. The water level information is saved in arrays that hold either gridded data or a time series.

### Product structure

The structure of the data product follows the form given in S-100 Part 10c – HDF5 Data Model and File Format. The general structure is given in Figure 10-1 and is the same as the generic structure described in S-100 with elements not used in S-104 omitted.

Figure 10-1 - Outline of the data file structure for S-104 data files, showing the realisation of S-104 structure from the generic structure described in S-100 (see S-100 Part 10c, Figure 10c-7). Note that there are four levels from top to bottom

In Figure 10-1 there are four levels:

Level 1: At the top level lies the Root Group, and it contains the Root Metadata (Table 12-1) and two subsidiary groups. The Root Metadata applies to all S-100 type products.

Level 2: The next Level contains the Feature Information Group and the Feature Container Group. The Feature Information Group contains two datasets: the featureCode, which has the name of the S-100 feature (here WaterLevel); and the feature information dataset (WaterLevel) which contains a compound array with eight parameters for each S-100 feature attribute (height, trend, and time). The Feature Container Group contains the Feature Type Metadata (Table 12-2) and one or more Feature Instance Groups. The Feature Type Metadata is common to all water level products.

Level 3: This contains one or more Feature Instances. A feature instance is, for example, a time series of gridded data for a single region; or a time series of astronomical predictions for a set of stations.

Feature instances in the same data file must not overlap unless they use different vertical datums. See clause 7.8 for details about datasets with multiple vertical datums. This prohibition applies even if the overlapping feature instances contain different types of data (astronomical predictions, forecasts, etc).

Features for different scale ranges must be in separate data files.

Level 4: This contains the actual data for the feature. S-104 uses only the Values Group. The Positioning group in S-100 is not needed for regular grids and is therefore not used in this edition of S-104.

The basic structure of the S-104 data product is shown in Table 10-2. Levels refer to HDF5 structuring. (C.f. S-100 Part 10c, Figure 10c-9). Naming in each box below header line is as follows: Generic name; S-100 or S-104 name; and (HDF5 type) group, attribute or attribute list, or dataset.

Table 10-2 – Overview of an S-104 data product

| LEVEL 1 (ROOT) CONTENT | LEVEL 2 CONTENT | LEVEL 3 CONTENT | LEVEL 4 CONTENT |
| --- | --- | --- | --- |
| General Metadata (see Table 12-1) (h5_attribute) |  |  |  |
| Feature Codes Group_F (h5_group) | Feature Type Name WaterLevel (h5_dataset) |  |  |
|  | Feature Type Codes featureCode (h5_dataset) |  |  |
| Feature Type WaterLevel (h5_group) | Feature Type Metadata (see Table 12-2) (h5_attribute) |  |  |
|  | Horz. & vert. Axis Names axisNames (h5_dataset) |  |  |
|  | First Feature Instance WaterLevel.01 (h5_group) | Feature Instance Metadata (see Table 12-3) (h5_attribute) |  |
|  |  | Uncertainty Data uncertainty (h5_dataset) |  |
|  |  | Domain extent polygon domainExtent.polygon (h5_dataset) (If and only if the feature covers an area with a vertical datum different from the root group. Either this or the bounding box attribute in Table 12-3 must be populated.) |  |
|  |  | First data group Group_001 (h5_group) | Time Attribute timePoint (h5_attribute) |
|  |  |  | Height+trend Array values (h5_dataset) |
|  |  | Second data group Group_002 (h5_group) | Time Attribute timePoint (h5_attribute) |
|  |  |  | Height+trend Array values (h5_dataset) |
|  |  | Third data group Group_003 (h5_group) | Time Attribute timePoint (h5_attribute) |
|  |  |  | Height+trend Array values (h5_dataset) |
|  | Second Feature Instance WaterLevel.02 (h5_group) | Feature Instance Metadata, etc., as for first instance |  |

The following clauses explain entries in Table 10-2 in more detail.

#### Root group

The Root group contains the Feature Codes group, the Feature Type group, and the simple attributes shown in Table 12-1.

#### Feature type codes (Group_F)

This group specifies the S-100 feature to which the data applies. The group has no attributes and consists of two components:

featureCode – a dataset with the name(s) of the S-100 feature(s) contained in the data product. For S-104, the dataset has a single element, the string “WaterLevel”.

WaterLevel – this is a dataset with the name contained in the featureCode dataset. The dataset contains a one-dimensional compound array of length 3 (one for each of the three water level attributes: height, trend, and uncertainty). Each of the three elements of string values has 8 values, as shown in Table 10-3.

NOTE: Values provided in Table 10-3 are required.

Table 10-3 – Contents of the one-dimensional compound array (length = 3, compound elements = 8) WaterLevel. All values are strings

| N | Name | Explanation | Attribute 1 | Attribute 2 | Attribute 3 |
| --- | --- | --- | --- | --- | --- |
| 1 | code | Camel Case Name | waterLevelHeight | waterLevelTrend | uncertainty |
| 2 | name | Plain text | Water Level Height | Water Level Trend | Uncertainty |
| 3 | uom.name | Units of Measurement | Metre |  | Metre |
| 4 | fillValue | Denotes missing data | -9999.00 | 0 | -1.00 |
| 5 | datatype | HDF5 datatype | H5T_FLOAT | H5T_ENUM | H5T_FLOAT |
| 6 | lower | Lower bound on attribute | -99.99 |  | 0.00 |
| 7 | upper | Upper bound on attribute | 99.99 |  | 99.99 |
| 8 | closure | Open or Closed data interval. See S100_IntervalType in S-100 Part 1 | closedInterval |  | closedInterval |

The values in this array must be consistent with the corresponding entries in the Feature Catalogue. Optional attributes (here, only uncertainty) are encoded in Group_F only if they are used in feature instances (see S-100 Edition 5.2.1 clause 10c-9.5). Attributes populated entirely with fill values are still considered as being used in the dataset. If encoded in Group_F, they must be present (populated with the fill value, if necessary) in all feature instances in this dataset.

NOTE: The decimal representation for floating point attributes is only for compatibility with the S-100 Feature Catalogue model.

#### Type group (WaterLevel)

This group contains a dataset called axisNames and one or more instances of the single feature WaterLevel. A single instance may contain a gridded forecast at multiple hours, or a set of time series predictions or observations. This group has the simple attributes shown in Table 12-2. For S-104, axisNames consists of two elements, the strings ‘longitude’ and ‘latitude’ (EPSG:4326) or ‘easting’ and ‘northing’ for UTM/UPS. The contents of the axisNames array must correspond to the names of axes for the coordinate system specified in the metadata, either “latitude” and “longitude” or “easting” and “northing” depending on whether the CRS is EPSG 4326 or a UTM/UPS CRS.

#### Instance group (WaterLevel.nn)

This group contains a single instance of the feature (see clause 10.2.2.3). The groups are numbered from 01 to 99. This group has the simple attributes shown in Table 12-3, as well as the (water level, trend, and time) values groups, the (conditional) positioning group, and a dataset called ‘uncertainty’.

Uncertainty Dataset – The (optional) uncertainty data is contained in a compound HDF5 dataset named ‘uncertainty’. There is a name and an uncertainty value for water level height, which is waterLevelHeight. The units of height uncertainty are metres. The default, denoting a missing value, is -1.0. The values in the uncertainty dataset are overridden for individual grid points by the uncertainty component of the values record when it is populated by a non-fill value.

Domain extent polygon – The domain extent polygon delimits the spatial extent of the domain of the coverage. It is encoded if and only if the feature covers an area with a vertical datum different from the root group. The encoding of domain extent polygons is described in S-100 Table 10c-11 and is reproduced and elaborated below.

Datatype: Array (1-d): i=0, P – HDF5 dataset of type Compound (Float, Float).

Components: <longitude, latitude> or <X, Y> (coordinates of bounding polygon vertices as a closed ring; that is, the first and last elements will contain the same values). Axis names, units and field sizes for the components should be the same as for the attributes which encode the grid location parameters (gridOriginLatitude and gridOriginLongitude in the feature instance group – see Table 12-3).

The domain extent polygon must conform to the requirements for domain extent polygons described in clause 7.8.

#### Value groups (Group_nnn)

These groups each contain an attribute (the date-time stamp), and the compound data arrays containing water level height and trend, and optionally uncertainty. These groups have the simple attributes shown in Table 12-4. These components are explained below.

Date-Time Stamp - The date-time stamp is an attribute named timePoint with a single (string) value. For gridded data the time stamp is the time of validity for all points in the grid.

Value Arrays - The height, trend and local uncertainty values (waterLevelHeight, waterLevelTrend and uncertainty) are stored in arrays named values, with a prescribed number of rows (numROWS) and, if two-dimensional, columns (numCOLS).

For a regular grid (dataCodingFormat = 2), the height, trend and (local) uncertainty values will be for each point in the grid, the data array values is two-dimensional, and the time for all points in the grid is given by the date-time stamp.

#### Conditional geography group (Positioning)

S-100 defines a group named Positioning which contains all the locations (geographical coordinates) that have associated data values. This group has no attributes.

The Positioning group is not used in S-104 datasets because S-104 uses only the regular grid coverage type which does not require explicit encoding of grid point locations.

For dataCodingFormat = 2 (regular grid), location data for grid points can be computed from the grid origin and number of grid points in each dimension, which are encoded as HDF5 attributes. The attribute numPOS is not needed since the grid data is stored as a two-dimensional array with the number of rows and columns given by the numbers of grid points in each dimension. See S-100 Part 10c, clause 10c-9.3 for more information.

Table 10-4 – Values of numPOS for the group Positioning

| Data Coding Format | Data Type | Location Data | Array Size:‌ Value of numPOS |
| --- | --- | --- | --- |
| 2 | Regular grid | (Not applicable) | (Not applicable) |

#### Summary of generalised dimensions

To summarise, for regular grids numPOS is inapplicable and X and Y positions of individual grid points are not stored. There are only data Groups containing water level data, which are stored in two-dimensional arrays of size numROWS by numCOLS. The total number of data Groups is numGRP.

The four variables that determine the array sizes (numROWS, numCOLS. numPOS, and numGRP) are given in Table 10-5.

Table 10-5 – The array dimensions used in the data product

| Data Coding Format | Data Type | Positioning | Data Values |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | numPOS | numCOLS | numROWS | numGRP |
| 2 | Regular Grid | (not used) | numPointsLongitudinal | numPointsLatitudinal | numberOfTimes |

NOTE: The values of numCOLS and numROWS must be adjusted down by 1 if data points are at cell centres (dataOffsetCode = 5) in order to avoid overrunning the last row and column of the grid extent.

#### Mandatory naming conventions

The following group and dataset names are mandatory in S-100: ‘Group_F’, ‘featureCode’, and (for S-104) ‘WaterLevel’, ‘axisNames’, ‘Positioning’, (for S-104) ‘WaterLevel.nn’, and ‘Group_nnn’ (n is an integer from 0 to 9). Attribute names shown in clause 12.3 are also mandatory.

#### Summary of product structure

For regularly gridded data, the water level array is two dimensional, with dimensions numPointsLongitudinal and numPointsLatitudinal. These attributes are part of feature instance metadata described in Table 12-3 and S-100 Part 10c, Table 10c-12. By knowing the grid origin and the grid spacings, the position of every point in the grid can be computed by simple formulae.

The remaining groups each contain a title, a date-time value (attribute timePoint), and the water level array. The title can be used to identify each individual station with time-series data. For dataCodingFormat = 2, the date-time is for the entire grid. The water level array is two dimensional, with a number of columns (numCOLS) and rows (numROWS). For a grid, the water value will be for each point in the grid.

The format allows features to be encoded only with uniform time intervals.

- For uniform time intervals, the time interval is encoded as an attribute of the Values group. In this case, the date-time of individual records is omitted from the water level array.

The groups are numbered 1, 2, etc, up to the maximum number of groups, numGRP. For regular grids (dataCodingFormat = 2), the number of groups is the number of time records.

The overall structure of the water level data product is created by assembling the data and metadata. The product structure is compliant with the HDF5 data architecture, which allows multi-dimensional arrays of data to be grouped with metadata. The format of the data product (cf. Figure 10-1) described above is portrayed in Figure 10-2. The Carrier Metadata is discussed in clause 12.3 (Tables 12-3 – 12-5), and the Values group attributes are discussed in clause 12.3 (Table 12-4).

NOTE: The name of each Group is the ‘Group_nnn‘, where nnn is numbered from 1 to numGRP.

|  |  |  | HDF5 Dataset |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | File Metadata (Table 12-3) |  |  |  |
|  |  |  | Group: WaterLevel |  |  |  |
|  |  |  | Feature Type Metadata (Table 12-4) |  |  |  |
|  |  |  | Group: WaterLevel.01 |  |  |  |
|  |  |  | Feature Instance Metadata (Table 12-5) |  |  |  |
|  |  |  | uncertainty, domain extent (optional – Table 10-2) |  |  |  |
|  |  |  | Group: Group_001 |  |  |  |
|  |  |  | Values Group attributes (Table 12-6) |  |  |  |
|  |  |  | Valid Date-Time1 |  |  |  |
|  |  |  | Water Level + Trend Array (i=0,numCOLS-1, j=0,numROWS-1) |  |  |  |
|  |  |  | Group: Group_002 |  |  |  |
|  |  |  | Values Group attributes |  |  |  |
|  |  |  | Valid Date-Time2 |  |  |  |
|  |  |  | Water Level + Trend Array (i=0,numCOLS-1, j=0,numROWS-1) |  |  |  |
|  |  |  | Group: Group_nnn |  |  |  |
|  |  |  | Values Group attributes |  |  |  |
|  |  |  | Valid Date-TimenumGRP |  |  |  |
|  |  |  | Water Level + Trend Array (i=0,numCOLS-1, j=0,numROWS-1) |  |  |  |

Figure 10-2 - Schematic of the S-104 HDF5 data product structure. The four parameters numPOS, numCOLS, numROWS, and numGRP are explained in Table 10-5.

Valid Date-Time1,2,...numGRP have specific meanings and encodings for dataCodingFormat = 2  (see Table 12-6).

#### Digital Certification Block

Information here is used to certify the validity or integrity of the data.

This Edition does not provide for inclusion of certificates or digital signatures within the HDF5 file. When necessary, certificates and digital signatures must be provided for the HDF5 file as a whole, using the mechanisms described in S-100 Parts 15 and 17.

#### Feature Identifiers

Individual instances of features within a dataset are identified by the name of the instance group, for example, WaterLevel.01, WaterLevel.02, etc. Unique feature identifiers are constructed by combining the file name of the HDF5 dataset with the name of the instance group, separated by a “:” (colon).

EXAMPLE: 104US00_CHES_TYPE1:WaterLevel.01 identifies the feature instance coded in the WaterLevel.01 instance group in the file named 104US00_CHES_TYPE1.h5.

## Datatypes for coordinates

This edition of S-104 does not include coverage spatial types which require grid point coordinates to be explicitly encoded.

Coordinates in embedded metadata must conform to the datatypes specified in clause 12.3.

Coordinates in discovery metadata (the exchange catalogue) must conform to the datatypes specified in clause 12.2 and S-100 Part 17.

## Axis names and ordering

Axis names are case-sensitive and must be encoded in lower-case. The permitted combinations are:

- “latitude” and “longitude” for ellipsoidal coordinate systems

- “easting” and “northing” for UTM/UPS

The order of axis names in the axisNames array should be latitude followed by longitude (as in EPSG 6422) or easting followed by northing (as in EPSG 4400).

NOTE: The order in axisNames and the order in sequencingRule.scanDirection (Table 12-2) are independent; the order in the former corresponds to the order within each coordinate tuple while the order in the latter describes the storage order of the values array. Note that the regular grid format does not encode coordinate tuples for individual grid nodes.

# Data Product Delivery

## Introduction

This clause describes how the water level data product is to be packaged by the Producer.

Due to the cost of transmitting data via the internet, it is desirable to limit file size and updating frequency whenever possible. In addition, smaller grid sizes are more efficient for loading since applications can automatically load the small cells that are needed quickly, and then load other nearby cells when needed based on ship movement or changes to the view (panning or zooming). Considerations here are the size of each transfer, the total volume of data transferred over time (the latter particularly applies to datasets which are issued daily or more frequently, such as forecasts), and application loading efficiency. Maximum size limits for S-104 datasets intended for ECDIS have therefore been empirically established. The size limits are:

- Each S-104 HDF5 data file, as created by the Producer, must be limited to a maximum of 15,000,000 value records. This limit is calculated as numPointsLatitudinal X numPointsLongitudinal X numberOfTimes ≤ 15,000,000. Value records partially or fully populated with fill values are included in the count.

EXAMPLE: An S-104 data file with numPointsLatitudinal=167, numPointsLongitudinal=141,  numberOfTimes=180 is counted as containing 167 X 141 X 180 = 4,238,460 value records and satisfies the size limitation.

- In the case of multiple vertical datums, the size limit applies to each datum feature separately. That is, a dataset with two vertical datums will have two (coincident) grid features, each of which is limited to a maximum of 15,000,000 value records calculated as previously described.

- The “cell scheming” (geographic extents covered) for datasets, especially datasets which are issued frequently (for example, daily or more frequent forecasts) should be determined so as to reduce the transfer of unnecessary data (information not needed for route planning or monitoring within reasonable time windows). It is recommended that cell scheming and grid density take into account the navigation purposes defined in S100_NavigationPurpose, reproduced below:

  - port – For port and near shore operations

  - transit – For coast and planning purposes

  - overview – For ocean crossing and planning purposes.

S-100 Part 15, clause 15-5.2 allows one data compression scheme: Zip (note that this may not provide a significant reduction due to internal compression applied within the HDF file). In addition, the file may be encrypted.

Updating of files typically means issuing a new forecast, or disseminating the latest observed water level for a specific geographic region. This may occur several times per day. Therefore, all files must contain a date-time of issuance of the product. Because of the potentially high frequency (that is, hourly or less) availability of new datasets, the ECDIS system may need to check for new data at a similar frequency. The “resource maintenance” information in external metadata and “delivery interval” in internal metadata should therefore be populated whenever possible.

## HDF5 dataset packaging

The HDF5-formatted datasets are packaged with metadata and an Exchange Catalogue, and then combined into an Exchange Set. HDF5 files for time series or gridded water level data may require internet transmission, since they change several or more times a day.

### Exchange Sets

Datasets, or data products, produced by the Producer consist of packages (such as ZIP archives or files organised within a file system folder/directory structure), containing both the Exchange Catalogue and one or more data products (of possibly different S-100 types), with each product covering a specific geographic region and specific period of time (Figure 11-1). The Exchange Catalogue lists the products and contains the discovery metadata. Support files may also be included in the package.

The name of the Exchange Set must be derived from the catalogue identifier, which in turn must be globally unique. See clause 12.2.2 for the rules determining how the catalogue identifier is constructed. An extension appropriate to the packaging method must be suffixed. For example:

- If the catalogue identifier is 104ABCDXYZ and the Exchange set is packaged as a Zip file, the name of the Zip file must be 104ABCDXYZ.zip or 104ABCDXYZ.ZIP.

- If the catalogue identifier is 104ABCDXYZ and the Exchange Set is distributed as a folder on compact disc media, the folder name must be 104ABCDXYZ.

|  | Exchange Set |  |
| --- | --- | --- |
|  | Exchange Catalogue |  |
|  | Metadata (includes list of files in Exchange Set) |  |
|  | Other Catalogues |  |
|  | Auxiliary files (Feature and Portrayal Catalogue, etc.) |  |
|  | Data Products |  |
|  | Data Product No. 1 |  |
|  | Data Product No. 2 |  |
|  | Data Product No. 3 |  |
|  | Data Product No. 4 |  |
|  | Etc |  |
|  | Support Files Language packs, dictionaries |  |

Figure 11-1 – Schematic diagram of the Exchange Set.

The size of datasets (HDF5 data files) can vary widely, depending on the data. Exchange files may be compressed using the DEFLATE compression algorithm commonly used in ZIP archives (cf. S-100 Part 15, clause 15-5.2). Doing so can reduce file size by 80% or more.

#### Exchange Set structure

The structure of an S-104 Exchange Set must be according to the structure described below, which is based on S-100 Part 17, clause 17-4.2. The S-104 Exchange Set structure is depicted in Figure 11-2.

- All content must be placed inside a top root folder named S100_ROOT. This is the only top level root folder in an Exchange Set containing only S-100 products.

- The S100_ROOT folder must contain a subfolder for S-104 which holds content specific to S-104.

- An S-104 Exchange Set must contain an Exchange Set Catalogue, CATALOG.XML, its digital signature CATALOG.SIGN and may contain any number of S-104 conformant dataset files and Catalogue files.

- The S-104 subfolder must contain subfolders for the component dataset files (DATASET_FILES) and Catalogues (CATALOGUES) as required:

  - The DATASET_FILES subfolder is required if and only if the Exchange Set contains an S-104 HDF5 dataset.

  - The CATALOGUES subfolder is required if and only if the Exchange Set contains a Feature Catalogue. (This Edition of S-104 does not include Interoperability or Portrayal Catalogues.)

- The DATASET_FILES folder must contain a subfolder named according to the Producer Code.

- Individual data files must be placed under the Producer subfolder, either directly in the Producer folder, or within a lower-level subfolder hierarchy. Individual data files may be optionally placed in their own subfolders or grouped with other data files.

- An Exchange Set may carry a Feature Catalogue, which should also be placed in the CATALOGUES folder.

- Portrayal Catalogues are not included in exchange sets because this edition of S-104 in intended only for use with water level adjustment on ECDIS and does not provide for S-104 Portrayal Catalogues.

- Except for the signature of the Exchange Catalogue file (CATALOG.XML), which is in the CATALOG.SIGN file, all digital signatures are included within their corresponding resource metadata records in CATALOG.XML.

- Dataset and Catalogue file and/or folder names should be such as to avoid inadvertent overwriting of files.

- Digital signatures are required for Exchange Sets and datasets intended for navigation on ECDIS. All resources included within an Exchange Set intended for navigation, including support files and catalogues, must be signed (S-100 Part 17).

- It is not necessary for an Exchange Set to contain more than one build of a Feature Catalogue for the same version of a Product Specification.

- Inclusion of the dictionary of enumerations in any particular Exchange Set is optional, since it will be the same for all datasets from all Producers. For similar reasons, inclusion of the Feature Catalogue in any particular Exchange Set is optional. Producers may distribute dummy Exchange Sets containing only the Feature Catalogue and enumerations dictionary, when any of them is updated or when a new version of the Product Specification is released. Validation checks should ensure that these files are present on the system if they are not included in any particular Exchange Set.

Figure 11-2 – Typical Exchange Set structure

General guidelines for Exchange Set structure are included in S-100 Part 17.

Note that the names and locations of files are coded within the CATALOG.XML files, and therefore files and folders should not be renamed or relocated by Producers or end-user systems unless these references can be updated. Feature Catalogues can be relocated to a common system location if their internal structure is maintained.

### Exchange Catalogue

The Exchange Catalogue which is in XML format acts as the table of contents for the Exchange Set. The catalogue file of the Exchange Set must be named CATALOG.XML (as specified in S-100 Part 17); no other file in the Exchange Set may have the same name.  The contents of the Exchange Catalogue are described in clause 12.

The Exchange Catalogue Schemas for S-104 are the same as for S-100 and may be obtained from the IHO S-100 Schema server: https://schemas.s100dev.net. The S-104 Exchange Catalogue uses an additional product-specific constraints file implementing product-specific restrictions, which is also available from the same site. Use of the additional product-specific constraints file is optional; developers may implement the constraints using any convenient method.

### Dataset file naming

The dataset file contains both metadata and one or more sets of height and trend arrays (see clause 10 – Data Product Format). The dataset name must begin with the three-character Product Specification number, followed by the four-character Producer Code (CCCC). Thus water level files begin with the seven-character string ‘104CCCC’.

The characters between this string and the extension are nominally unrestricted in S-100 and S-97. However, S-104 restricts the “unrestricted” characters as follows:

- Alphabetic characters in the “Latin alphabet; that is, A-Z and a-z;

- Numeric characters; that is, the characters 0-9;

- The hyphen and underscore characters (“-“ and “_”).

The “unrestricted” characters may be used to denote geographical region, source of the data, version numbers, and/or any other relevant information. Characters may be lower or upper case. It is not recommended to use unique values such as date/time references as the unrestricted characters need to remain identical between each edition of a dataset.

The filename extension for HDF5 (.h5) must be used to denote the file format.

The total length of the file name shall be no more than 64 characters, including the extension.

EXAMPLE: 104US00_CHES_TYPE1.h5 for observational data (see clause 12.3.5, Table 12-10) produced by NOAA for Chesapeake Bay (CHES).

Each Producer should adopt a naming scheme that is consistent across its entire S-104 product line. While the example above is hypothetical, it illustrates how the principles of this clause can be applied by Producers.

#### Dataset MRN (informative)

The dataset file name may be mapped to an MRN as follows:

urn:mrn:iho:s104:<ver>:<cccc>:<region>:<type>:<dtg>

where:

- The first part “urn:mrn:iho:s104” is common to all dataset URNs for S-104;

- The Product Specification version is represented by the “<ver>” part, for example, 2:0:0 for Edition 2.0.0;

- <cccc> represents the 4-character Producer Code;

- <region> represents the geographical region;

- <type> represents the data coding format, for example “type2” for regular grids;

- <dtg> represents the date/time component in the name.

This is an interim rule pending definition of an “S-100-wide” rule for MRNs and will be superseded by the “S-100-wide” rule when it is published.

### Support files

Only the following types of support files are allowed in S-104:

- Optional ‘language packs’ for Feature Catalogues. Each language pack contains a translation of the Feature Catalogue into a specified language.

- Dictionary resource files listing the allowed values and codes of enumerations. There will generally be a single dictionary file for each version of the Product Specification (corrections, if any, will be issued through the usual mechanism for corrections). Inclusion of the dictionary resource file in Exchange Sets is optional, since the Internet location is standardised and manufacturers are permitted to obtain it by other means and install it in an application-specific location.

### Support file naming

#### General

Support file names are subject to the same naming rules as dataset file names (clause 11.2.3), except that the extension is determined by the support file format.

This clause covers names of language packs and enumeration dictionaries, which are the only support files allowed in this Edition of S-104. Producers who discover a need for other types of support files should conform to the general rule above and consult the TWCWG as necessary.

#### Names of language packs

If a language pack created by a data Producer for the S-104 Feature Catalogue is included, it must have the standard 7-character “104CCCC” prefix and the same base name as the standard IHO-issued Feature Catalogue with the 3-letter ISO 639-2/T language code suffixed. The language codes must be exactly those in the S-100 codelist for languages (S100_MD_LanguageCode, which can be found in the S-100 Schema distribution). The file extension must be “.XML”.

NOTE: A language pack issued by the IHO for the IHO Feature Catalogue will use the IHO Producer Code.

EXAMPLE: The language pack for Italian translations issued by the data Producer with code “IT01” of the Feature Catalogue named 104_1_1_0_FC.XML is named 104IT01_104_1_1_0_FC_ita.XML.

#### Names of enumeration dictionaries

Enumeration dictionaries are supplied by IHO as part of this Product Specification and should not be renamed.

NOTE (informative): Substitute or extended enumeration dictionaries may be developed if translations are needed. Producers who desire to provide translations of enumeration dictionaries with S-104 Exchange Sets should consult with the TWCWG.

# Metadata

## Introduction

For information exchange, there are several categories of metadata required:

- Metadata about the overall Exchange Set and Catalogue;

- Discovery metadata about each of the datasets contained in the Catalogue; and

- Discovery metadata about the support files that make up the package.

The discovery metadata classes have numerous attributes which enable important information about the datasets and accompanying support files to be examined without the need to process the data, for example, decrypt, decompress, load, etc.

Feature Catalogues can be included in the Exchange Set in support of the datasets. If included, discovery metadata about the Catalogues must also be provided.

NOTE: S-104 datasets do not reference support files. The only support files allowed in the Exchange Set are “language packs” for Feature Catalogues and enumeration dictionaries, and these are not referenced from within the HDF5 datasets.

Discovery metadata for each HDF5 dataset is given in an XML block within the Exchange Set Catalogue file, and can be accessed without opening the HDF5 file. In addition to discovery metadata, S-104 also provides for carrier metadata that is embedded within the HDF5 file, which provides information needed to process and display the data. Discovery metadata is described in clause 12.2; carrier metadata in clause 12.3.

This clause defines the mandatory and optional metadata needed for S-104. In some cases (if provided by the Producer or Exchange Set packager) the metadata may be repeated in a language other than English. See S-100 Part 17, clauses 17-4.6 – 17-4.8 for guidance on encoding of metadata in languages other than English.

### Realisation of Exchange Set components and metadata classes (informative)

The realization of S-104 Exchange Set components and metadata classes from ISO 19115-1 and ISO 19115-3 is the same as in S-100 Part 17, depicted in Figure 12-1. The Figure depicts, from left to right:

- The relevant ISO data exchange structural classes;

- The relevant ISO metadata classes for metadata for exchange;

- S-100 structure classes representing the S-100/S-104 Exchange Set components;

- The relevant S-100/S-104 Exchange Set metadata classes.

Note that the only support files in S-104 are language packs or enumeration dictionaries, represented by S100_SupportFile. The corresponding metadata blocks are represented by S100_SupportFileDiscoveryMetadata elements.

Figure 12-1 – Realisation of the Exchange Set classes (S-100 Part 17, Figure 17-1 with relationships not used in S-104 omitted)

### Exchange Set components and related metadata

Figure 12-2 depicts the relationships of Exchange Set “core” elements (datasets and Feature/Portrayal Catalogues) and Exchange Set metadata. This Figure is derived from S-100 Part 17, Figure 17-2. Relationships not applicable to S-104 have been omitted (for example, the link between datasets and support files in S-100 Figure 17-2, because S-104 datasets do not reference support files). Note also that the link between S100_Dataset and S100_CatalogueMetadata is implicit by means of the S-104 version to which the Feature Catalogue, Portrayal Catalogue and dataset conform, which must have the same Edition and revision components.

Figure 12-2 – Components and associated metadata for the S-104 Exchange Set (S-100 Part 17, Figure 17-2 with relationships not used by S-104 omitted)

The rules governing the presence and roles of the exchange set components depicted in Figure 12-2 are given below.

- Every Exchange Set must contain an Exchange Catalogue, represented by S100_ExchangeCatalogue in Figure 12-2.

- Dataset discovery metadata (S100_DatasetDiscoveryMetadata) must be provided in the Exchange Catalogue for each S-104 dataset in the Exchange Set.

- Catalogue metadata (S100_CatalogueDiscoveryMetadata) must be provided in the Exchange Catalogue for any Feature and Portrayal Catalogues included in the Exchange Set.

- The only support files allowed are language packs and enumeration dictionaries (both represented by S100_SupportFile). Their inclusion in Exchange Sets is optional.

- Producers must not include ISO metadata files to convey information for ECDIS application processing, since processing these files is not an ECDIS requirement. All information necessary for ECDIS processing must be in CATALOG.XML

- Language packs are described in S-100 Part 18 and provide translations of Feature Catalogues.

- If a language pack is included, a support file discovery metadata block (S100_SupportFileDiscoveryMetadata) describing the file must be included in the Exchange Catalogue.

- A signature file for the Exchange Catalogue must also be included in the Exchange Set (S100_CatalogueSignature).

Since S-104 does not add product-specific metadata attributes, the S-100 metadata classes and Schema are used in S-104 Exchange Sets without extension. The constraints S-104 impose on generic S-100 metadata are described in the documentation Tables in clause 12.2.

NOTE: The distribution package implements the additional S-104 constraints on metadata attributes (and many of the S-100 constraints) as Schematron rules in files available from the IHO Schema server. Implementers may substitute any implementation method to apply or check constraints instead of using Schematron-capable processing software.

The tangible representations of the structure classes in Figure 12-2 within actual Exchange Sets are the digital files or folders containing the Exchange Set, dataset(s), Catalogue(s), and support files. The tangible representations of their roles as depicted in Figure 12-2 are the inclusion of the respective components within the Exchange Set. Documentation tables for the structure classes are not provided since the Exchange Set structure is described in clause 11.2.1.

The metadata classes in Figure 12-2 are represented by XML files or XML blocks and are documented in clause 12.2.

## Discovery metadata

An outline of the overall concept of an S-104 Exchange Set for the interchange of geospatial data and its relevant metadata is explained in clauses 11.2.1 and 12.1.1. The place of metadata in the Exchange Set is summarised in clause 12.1.2.

Figure 12-3 depicts the structure of the Exchange Catalogue and its component discovery metadata blocks. The structure is the same as in S-100 Part 17.

Figure 12-3 – Relationship between Exchange Catalogue, discovery metadata, and dataset (from S-100 Part 17, Figure 17-6)

The detailed structure of the S-104 Exchange Catalogue is depicted in Figure 12-4. This Figure is derived from S-100 Part 17, Figure 17-7, with the following restrictions:

- Elements that are optional in the generic S-100 catalogue model but not used in S-104 are not shown; for example, the updateNumber and updateApplicationDate attributes in the dataset discovery class are not used in S-104.

- Constraints that are specific to S-104 are summarised in a diagram note. Details about constraints are provided in the documentation tables following the diagram.

In S-104 Edition 2.0.0 the only catalogues allowed are Feature Catalogues.

The language used for the metadata is English.

Time reference for all data will be UTC.

All water level values to be given in metres (up to two decimal places for real values).

More detailed information about the various classes and textual descriptions of the constraints are in the Tables in clauses 12.2.1 – 12.2.30 following Figure 12-4. Differences from generic S-100 metadata are emphasized for developer convenience in bold text.

Page intentionally left blank

Figure 12-4 – Details of Exchange Set Catalogue classes. Based on S-100 Part 17, Figure 17-7

### S100_ExchangeCatalogue

Each Exchange Set has a single S100_ExchangeCatalogue which is an XML file that contains meta information for the data in the Exchange Set. S-104 restricts the S-100 class as described in the Remarks column.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_ExchangeCatalogue | An Exchange Catalogue contains the discovery metadata about the exchange datasets and support files | - | - | The optional S-100 attributes identifier, contact, and productSpecification are mandatory in S-104 |
| Attribute | identifier | Uniquely identifies this Exchange Catalogue | 1 | S100_ExchangeCatalogueIdentifier | Mandatory in S-104 |
| Attribute | contact | Details about the issuer of this Exchange Catalogue | 1 | S100_CataloguePointOfContact | Mandatory in S-104 |
| Attribute | productSpecification | Details about the Product Specifications used for the datasets contained in the Exchange Catalogue | 1..* | S100_ProductSpecification | Mandatory in S-104 |
| Attribute | defaultLocale | Default language and character set used for all metadata records in this Exchange Catalogue | 0..1 | PT_Locale | Default is English and UTF-8 |
| Attribute | otherLocale | Other languages and character sets used for the localized metadata records in this Exchange Catalogue | 0..* | PT_Locale | Required if any localized entries are present in the Exchange Catalogue |
| Attribute | exchangeCatalogueDescription | Description of what the Exchange Catalogue contains | 0..1 | CharacterString |  |
| Attribute | exchangeCatalogueComment | Any additional Information | 0..1 | CharacterString |  |
| Attribute | certificates | Signed public key certificates referred to by digital signatures in the Exchange Set | 0..* | S100_SE_CertificateContainerType | Content defined in S-100 Part 15. All certificates used, except the SA root certificate (installed separately by the implementing system) shall be included |
| Attribute | dataServerIdentifier | Identifies the data server for the permit | 0..1 | CharacterString |  |
| Role | datasetDiscoveryMetadata | Exchange Catalogues may include or reference discovery metadata for the datasets in the Exchange Set | 0..* | Aggregation S100_DatasetDiscoveryMetadata |  |
| Role | catalogueDiscoveryMetadata | Metadata for Catalogue | 0..* | Aggregation S100_CatalogueDiscoveryMetadata | Metadata for the Feature, Portrayal and Interoperability Catalogues, if any |
| Role | supportFileDiscoveryMetadata | Exchange Catalogues may include or reference discovery metadata for the support files in the Exchange Set | 0..* | Aggregation S100_SupportFileDiscoveryMetadata | The only support files allowed in S-104 are enumeration dictionaries and language packs for Feature Catalogues |

### S100_ExchangeCatalogueIdentifier

S-104 uses S100_ExchangeCatalogueIdentifier without modification.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_ExchangeCatalogueIdentifier | An Exchange Catalogue contains the discovery metadata about the exchange datasets and support files | - | - | The concatenation of identifier and dateTime form the unique name |
| Attribute | identifier | Uniquely identifies this Exchange Catalogue | 1 | CharacterString | See Note 1 for the naming convention |
| Attribute | dateTime | Creation date and time of the Exchange Catalogue, including time zone | 1 | DateTime | Format: yyyy-mm-ddThh:mm:ssZ |

NOTE 1: Use the file name component of the dataset according to the convention in clause 11.2.3. For example, if the dataset file is named 104ABCDXYZ.h5 the metadata identifier should be 104ABCDXYZ. In the event of an Exchange Set containing multiple datasets, use the name of the dataset of largest extent with a “+N” suffix (without quotes), where N is the number of additional datasets in the Exchange Set. If the Exchange Set contains only Feature and/or Portrayal Catalogues, use 104ABCD+N where “ABCD” is the 4-character code of the producer of the Feature or Portrayal Catalogue.

### S100_CataloguePointofContact

S-104 uses S100_CataloguePointOfContact without modification.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_CataloguePointOfContact | Contact details of the issuer of this Exchange Catalogue | - | - | - |
| Attribute | organization | The organization distributing this Exchange Catalogue | 1 | CharacterString | This could be an individual producer, value added reseller, etc |
| Attribute | phone | The phone number of the Organization | 0..1 | CI_Telephone |  |
| Attribute | address | The address of the Organization | 0..1 | CI_Address |  |

### S100_DatasetDiscoveryMetadata

Data in the Discovery Metadata are used to identify the relevance of the dataset to the particular application. S-104 restricts the multiplicity and contents of S100_DatasetDiscoveryMetadata as described in the Remarks column.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_DatasetDiscoveryMetadata | Metadata about the individual datasets in the Exchange Catalogue | - | - | The optional S-100 attributes updateNumber, updateApplicationDate, otherLocale, and referenceID are not used in S-104 The optional S-100 attributes datasetID dataCoverage, editionNumber, producerCode and boundingBox are mandatory in S-104 |
| Attribute | fileName | Dataset file name | 1 | URI | See S-100 Part 1, clause 1-4.6 |
| Attribute | description | Short description giving the area or location covered by the dataset | 0..1 | CharacterString | For example a harbour or port name, between two named locations etc |
| Attribute | datasetID | Dataset ID expressed as a Maritime Resource Name | 1 | URN | The URN must be an MRN Made mandatory in S-104 See clause 11.2.3.1 |
| Attribute | compressionFlag | Indicates if the resource is compressed | 1 | Boolean | true indicates a compressed dataset resource false indicates an uncompressed dataset resource |
| Attribute | dataProtection | Indicates if the data is encrypted | 1 | Boolean | true indicates an encrypted dataset resource false indicates an unencrypted dataset resources |
| Attribute | protectionScheme | Specification of method used for data protection | 0..1 | S100_ProtectionScheme | In S-100 the only allowed value is “S100p15” |
| Attribute | digitalSignatureReference | Specifies the algorithm used to compute digitalSignatureValue | 1 | S100_SE_DigitalSignatureReference (see S-100 Part 15) |  |
| Attribute | digitalSignatureValue | Value derived from the digital signature | 1..* | S100_SE_DigitalSignature (see S-100 Part 15) | The value resulting from application of digitalSignatureReference Implemented as the digital signature format specified in Part 15 At least one S100_SE_SignatureOnData is required |
| Attribute | copyright | Indicates if the dataset is copyrighted | 1 | Boolean | true indicates the resource is copyrighted false Indicates the resource is not copyrighted |
| Attribute | classification | Indicates the security classification of the dataset | 0..1 | MD_SecurityConstraints> MD_ClassificationCode (codelist) | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret 6. sensitive but unclassified 7. for official use only 8. protected 9. limited distribution Recommended for datasets intended for use on ECDIS (with expected classification as “unclassified”) |
| Attribute | purpose | The purpose for which the dataset has been issued | 0..1 | S100_Purpose |  |
| Attribute | notForNavigation | Indicates the dataset is not intended to be used for navigation | 1 | Boolean | true indicates the dataset is not intended to be used for navigation false indicates the dataset is intended to be used for navigation |
| Attribute | specificUsage | The use for which the dataset is intended | 0..1 | MD_USAGE>specificUsage (character string) | Information about specific usage(s) for which the dataset is intended |
| Attribute | editionNumber | The Edition number of the dataset | 1 | Integer | Mandatory in S-104 See clause 8.2 |
| Attribute | issueDate | Date on which the data was made available by the Data Producer | 1 | Date |  |
| Attribute | issueTime | Time of day at which the data was made available by the Data Producer | 0..1 | Time | Mandatory when the interval between datasets is shorter than 1 day, such as 6-hourly forecasts |
| Attribute | boundingBox | The extent of the dataset limits | 1 | EX_GeographicBoundingBox | Mandatory in S-104 |
| Attribute | temporalExtent | Specification of the temporal extent of the dataset | 0..1 | S100_TemporalExtent | The temporal extent is encoded as the date/time of the earliest and latest data records (in coverage datasets) or date/time ranges (in vector datasets) If there is more than one feature in a dataset, the earliest and latest time values of records in all features are used, which means the earliest and latest values may be from different features If date/time information for a feature is not encoded in the dataset, it is treated for the purposes of this attribute as extending indefinitely in the appropriate direction on the time axis, limited by the issue date/time or the cancellation or supersession of the dataset This attribute is encoded if and only if at least one of the start and end of the temporal extent is known |
| Attribute | productSpecification | The product specification used to create this dataset | 1 | S100_ProductSpecification |  |
| Attribute | producingAgency | Agency responsible for producing the data | 1 | CI_ResponsibleParty>CI_Organisation | See S-100 Part 17 |
| Attribute | producerCode | The official IHO Producer Code from S-62 | 1 | CharacterString | Recommended for datasets intended for use on ECDIS. Mandatory in S-104. |
| Attribute | encodingFormat | The encoding format of the dataset | 1 | S100_EncodingFormat | Must be HDF5 |
| Attribute | dataCoverage | Area covered by the dataset | 1..* | S100_DataCoverage | Mandatory in S-104 |
| Attribute | comment | Any additional information | 0..1 | CharacterString |  |
| Attribute | defaultLocale | Default language and character set used in the dataset | 0..1 | PT_Locale |  |
| Attribute | metadataPointOfContact | Point of contact for metadata | 0..1 | CI_Responsibility > CI_Individual or CI_Responsibility > CI_Organisation | Only if metadataPointOfContact is different from producingAgency |
| Attribute | metadataDateStamp | Date stamp for metadata | 0..1 | Date | May or may not be the issue date |
| Attribute | replacedData | Indicates if a cancelled dataset is replaced by another data file(s) | 0..1 | Boolean | See Note Mandatory when purpose = cancellation |
| Attribute | dataReplacement | Dataset name | 0..* | CharacterString | A dataset may be replaced by 1 or more datasets See Note Mandatory when replacedData = true |
| Attribute | navigationPurpose | Classification of intended navigation purpose (for Catalogue indexing purposes) | 0..3 | S100_NavigationPurpose | Mandatory when notForNavigation = false |
| Role | resourceMaintenance | Information about the frequency of resource updates, and the scope of those updates | 0..1 | MD_MaintenanceInformation | S-100 restricts the multiplicity to 0..1 and adds specific restrictions on the ISO 19115 structure and content. See clause MD_MaintenanceInformation in S-100 Part 17 Format: PnYnMnDTnHnMnS (XML built-in type for ISO 8601 duration). See S-100 Part 17, clause 17-4.9 for encoding guidance If present, the duration must match the duration encoded in embedded metadata (Table 12-1) |

NOTE: replacedData and dataReplacement: The intended use of the attributes replacedData and dataReplacement could be, for example, to provide a mechanism for service providers to build automation when providing replacement data sets to customers within existing subscription periods.

### S100_NavigationPurpose

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_NavigationPurpose | The Navigational Purpose of the dataset | - |  |
| Value | port | For port and near shore operations | 1 |  |
| Value | transit | For coast and planning purposes | 2 |  |
| Value | overview | For ocean crossing and planning purposes | 3 |  |

### S100_DataCoverage

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_DataCoverage | A spatial extent where data is provided; and the display scale information for the provided data | - | - | The S-100 attribute temporalExtent is not used. |
| Attribute | boundingPolygon | A polygon which defines the actual data limit | 1 | EX_BoundingPolygon | See the Notes below this Table |
| Attribute | optimumDisplayScale | The scale with which the data is optimally displayed | 0..1 | Integer | Example: A scale of 1:25000 is encoded as 25000 Guidance for display scales will be provided in a future revision (see Note 6) |
| Attribute | maximumDisplayScale | The maximum scale with which the data is displayed | 0..1 | Integer | Guidance for display scales will be provided in a future revision (see Note 6) |
| Attribute | minimumDisplayScale | The minimum scale with which the data is displayed | 0..1 | Integer | Guidance for display scales will be provided in a future revision (see Note 6) |
| Attribute | approximateGridResolution | The resolution of gridded or georeferenced data (in metres) | 1..2 | Real | A single value may be provided when all axes have a common resolution For multiple value provision, use axis order as specified in dataset May be approximate for ungeorectified data For example, for 5 metre resolution, the value 5 must be encoded Mandatory for S-104 |

NOTE 1: If there are multiple grid features in the dataset, each feature should have a separate dataCoverage attribute in dataset discovery metadata, except that the coverages for intersecting or adjacent features with the same grid resolution may be combined at Producer discretion.

NOTE 2: Bounding polygons for grid features should be the same as the spatial extent of the grid, except that large regions uniformly and permanently populated with fill values (such as large land areas) may be excluded. The boundaries are left to the discretion of the data producer, but overly-detailed boundaries and small interior holes are discouraged.

NOTE 3: A boundingPolygon is restricted to a single GML Polygon with one exterior and 0 or more interiors expressed as Linear Rings using SRS EPSG:4326. The exterior and optional interiors shall be composed of a closed sequence of >=4 coordinate positions expressed individually or as a list (posList). The GML polygon shall have a valid GML identifier.

NOTE 4: For approximateGridResolution, if the grid cell size varies over the extent of the grid, an approximated value based on model parameters or production metadata should be used.

NOTE 5: More detailed guidance for bounding polygons will be provided in a future edition. In the interim, it is suggested that data producers use polygons for underlying S-102 datasets as a starting point.

NOTE 6: More detailed guidance about display scales will be provided in a future edition. In the interim, display scales are left to the discretion of the producer; it is suggested they should be matched to display scales in the corresponding S-102 dataset(s).

### S100_Purpose

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_Purpose | The purpose of the dataset | - | See clause 8.2. The S-100 values update, reissue and delta are not used |
| Value | newDataset | Brand new dataset | 1 | No data has previously been produced for this area |
| Value | newEdition | New Edition of the dataset or Catalogue | 2 | Includes new information which has not been previously distributed by updates |
| Value | cancellation | Dataset or Catalogue that has been cancelled | 5 | Indicates the dataset or Catalogue should no longer be used and can be deleted |

### S100_TemporalExtent

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_TemporalExtent | Temporal extent | -- |  | At least one of the timeInstantBegin and timeInstantEnd attributes must be populated; if both are known, both must be populated. The absence of either begin or end indicates indefinite validity in the corresponding direction, limited by the issue date/time or the cancellation or supersession of the dataset |
| Attribute | timeInstantBegin | The instant at which the temporal extent begins | 0..1 | DateTime |  |
| Attribute | timeInstantEnd | The instant at which the temporal extent ends | 0..1 | DateTime |  |

NOTES:

- In case of overlap in temporal extent between predecessor and successor datasets, the successor dataset prevails. For example, water level or weather forecast datasets may have a temporal extent of N days or hours, but be replaced by new forecast at N – X.

- Precedence and succession can be determined from information in dataset discovery metadata (in particular, issue date, time and temporal extent).

### S100_EncodingFormat

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_EncodingFormat | Encoding format | - | Only the HDF5 format is used in S-104 |
| Value | HDF5 | The HDF5 data format as defined in Part 10c | 3 |  |

### S100_ProductSpecification

S-104 uses S100_ProductSpecification without modification.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_ProductSpecification | The Product Specification contains the information needed to build the specified product | - | - | The optional S-100 attributes name, version and compliancyCategory are mandatory in S-104 |
| Attribute | name | The name of the Product Specification used to create the datasets | 1 | CharacterString | The name in the Product Specification Register, in the IHO Geospatial Information (GI) Registry. For S-104, this is “Water Level Information for Surface Navigation” Mandatory in S-104 |
| Attribute | version | The version number of the Product Specification | 1 | CharacterString | For example 2.0.0 for S-104 Edition 2.0.0 Mandatory in S-104 |
| Attribute | date | The version date of the Product Specification | 0..1 | Date | From the Product Specification Register of the IHO GI Registry. For interim drafts use the version date in Product Specification Metadata |
| Attribute | productIdentifier | Machine readable unique identifier of a product type | 1 | CharacterString (Restricted to Product ID values from the IHO Product Specification Register, in the IHO GI Registry) | For S-104 this must be the string “S-104” (without quotes) |
| Attribute | number | The number used to lookup the product in the Product Specification Register of the IHO GI Registry | 1 | Integer | From the Product Specification Register in the IHO GI Registry Encode as “0” until this Edition is added to the GI Registry and receives a Registry number. Do not use the number of any other Edition |
| Attribute | compliancyCategory | The level of compliance of the Product Specification to S-100 | 1 | S100_CompliancyCategory | See S-100 Part 4a, clause 4a-5.5 and clause 7.6 in this Product Specification Mandatory in S-104 |

### S100_CompliancyCategory

S-104 uses only Category 4 as defined in S-100 Part 4a, clause 4a-5.5.

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_CompliancyCategory |  | - | S-104 2.0.0 is compliant with category4 |
| Value | category4 | IHO S-100 and IMO harmonized display compliant | 4 |  |

### S100_ProtectionScheme

S-104 uses S100_ProtectionScheme without modification.

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_ProtectionScheme | Data protection schemes | - | - |
| Value | S100p15 | IHO S-100 Part 15 | - | See S-100 Part 15 |

### S100_SupportFileDiscoveryMetadata

The only support files in S-104 are enumeration dictionaries and language packs for Feature Catalogues.

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_SupportFileDiscoveryMetadata | Metadata about the individual support files in the Exchange Catalogue | - | - | S-104 does not use otherDataTypeDescription |
| Attribute | fileName | Name of the support file | 1 | URI | See S-100 Part 1, clause 1-4.6 and clause 11.2.5 in this Product Specification |
| Attribute | revisionStatus | The purpose for which the support file has been issued | 1 | S100_SupportFileRevisionStatus | For example new, replacement, etc |
| Attribute | editionNumber | The Edition number of the support file | 1 | Integer | See clause 8.2.6 |
| Attribute | issueDate | Date on which the data was made available by the Data Producer | 0..1 | Date | Date on which the support file was made available by its Producer |
| Attribute | supportFileSpecification | The Specification used to create this file | 0..1 | S100_SupportFileSpecification |  |
| Attribute | dataType | The format of the support file | 1 | S100_SupportFileFormat |  |
| Attribute | comment | Optional comment | 0..1 | CharacterString |  |
| Attribute | compressionFlag | Indicates if the resource is compressed | 1 | Boolean | true indicates a compressed resource false indicates an uncompressed resource |
| Attribute | digitalSignatureReference | Specifies the algorithm used to compute digitalSignatureValue | 1 | S100_SE_DigitalSignatureReference (see S-100 Part 15) | (Type corrected to conform to S-100 Part 15) |
| Attribute | digitalSignatureValue | Value derived from the digital signature | 1..* | S100_SE_DigitalSignature (see S-100 Part 15) | The value resulting from application of digitalSignatureReference Implemented as the digital signature format specified in S-100 Part 15 |
| Attribute | defaultLocale | Default language and character set used in the support file | 0..1 | PT_Locale | In absence of defaultLocale the language is English in UTF-8 A support file is expected to use only one as locale. Additional support files can be created for other locales |
| Attribute | supportedResource | Identifier of the resource supported by this support file | 0..* | CharacterString | Conventions for identifiers are still to be developed in S-100. S-100 allows file URI, digital signature or cryptographic hash checksums to be used. In the interim, S-104 language packs will reference the Feature Catalogue. For enumeration dictionaries, use the Product Specification identifier and version in URI form |
| Attribute | resourcePurpose | The purpose of the supporting resource | 0..1 | S100_ResourcePurpose | Identifies how the supporting resource is used |

### S100_SupportFileFormat

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_SupportFileFormat | The format used for the support file | - | S-104 uses only XML; language packs and enumeration dictionaries are all XML files |
| Value | XML | Extensible Markup Language | 4 |  |

### S100_SupportFileRevisionStatus

S-104 uses S100_SupportFileRevisionStatus without modification.

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_SupportFileRevisionStatus | The reason for inclusion of the support file in this Exchange Set | - | - |
| Value | new | A file which is new | 1 | Signifies a new file |
| Value | replacement | A file which replaces an existing file | 2 | Signifies a replacement for a file of the same name |
| Value | deletion | Deletes an existing file | 3 | Signifies deletion of a file of that name |

### S100_SupportFileSpecification

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_SupportFileSpecification | The Standard or Specification to which a support file conforms | - | - | - |
| Attribute | name | The name of the Specification used to create the support file | 1 | CharacterString | S-100 for language packs and enumeration dictionary |
| Attribute | version | The version number of the Specification | 0..1 | CharacterString | Use the applicable edition of the Standard in the name attribute For example, “5.0.0” for language packs conforming to S-100 Edition 5.0.0 |
| Attribute | date | The version date of the Specification | 0..1 | Date | Omit or use the publication date in the GI Registry |

### S100_ResourcePurpose

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_ResourcePurpose | Defines the purpose of the supporting resource | - | S-104 allows only language packs and enumeration dictionaries as support files and the allowed values of the S-100 enumeration are restricted accordingly |
| Value | languagePack | A Language pack | 3 |  |
| Value | other | A type of resource not otherwise described | 100 | For an enumeration dictionary, which supports all datasets for a particular version of the Product Specification |

### S100_CatalogueDiscoveryMetadata

S-104 uses S100_CatalogueDiscoveryMetadata without modification. This class is used to provide metadata about Feature Catalogues.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_CatalogueDiscoveryMetadata | Class for S-100 Catalogue metadata | - | - | - |
| Attribute | fileName | The name for the Catalogue | 1 | URI | See S-100 Part 1, clause 1-4.6 |
| Attribute | purpose | The purpose for which the Catalogue has been issued | 0..1 | S100_Purpose (codelist) | The values must be one of the following: 2 new edition 5 cancellation Default is new edition |
| Attribute | editionNumber | The Edition number of the Catalogue | 1 | Integer | Initially set to 1 for a given productSpecification.number Increased by 1 for each subsequent New Edition Uniquely identifies the version of the Catalogue |
| Attribute | scope | Subject domain of the Catalogue | 1 | S100_CatalogueScope |  |
| Attribute | versionNumber | The version identifier of the Catalogue | 1 | CharacterString | Human readable version identifier |
| Attribute | issueDate | The issue date of the Catalogue | 1 | Date |  |
| Attribute | productSpecification | The Product Specification used to create this file | 1 | S100_ProductSpecification |  |
| Attribute | digitalSignatureReference | Specifies the algorithm used to compute digitalSignatureValue | 1 | S100_SE_DigitalSignatureReference (see S-100 Part 15) |  |
| Attribute | digitalSignatureValue | Value derived from the digital signature | 1..* | S100_SE_DigitalSignature (see S-100 Part 15) | The value resulting from application of digitalSignatureReference Implemented as the digital signature format specified in S-100 Part 15 |
| Attribute | compressionFlag | Indicates if the resource is compressed | 1 | Boolean | true indicates a compressed resource false indicates an uncompressed resource |
| Attribute | defaultLocale | Default language and character set used in the Catalogue | 0..1 | PT_Locale | In absence of defaultLocale the language is English in UTF-8 |
| Attribute | otherLocale | Other languages and character sets used in the Catalogue | 0..* | PT_Locale |  |

### S100_CatalogueScope

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_CatalogueScope | The scope of the Catalogue | - | S-104 exchange sets do not contain Interoperability or portrayal Catalogues and the corresponding values are removed |
| Value | featureCatalogue | S-100 Feature Catalogue | 1 |  |

### MD_MaintenanceInformation

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | MD_MaintenanceInformation | Information about the scope and frequency of updating | - | - | S-100 restricts the ISO 19115-class to: - Prohibit maintenanceScope, maintenanceNote, and contact attributes; - Define restrictions on maintenanceAndUpdate‌Frequency, maintenanceDate, and userDefinedMaintenance‌Frequency attributes |
| Attribute | maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the resource after the initial resource is completed | 0..1 | MD_MaintenanceFrequencyCode (codelist) | Must be populated if userDefinedMaintenanceFrequency is not present, otherwise optional. See Table MD_Maintenance‌Frequency‌Code in this clause for values allowed in S-100 metadata |
| Attribute | maintenanceDate | Date information associated with maintenance of the resource | 0..1 | CI_Date | Exactly one of maintenanceDate and userDefinedMaintenanceFrequency must be populated Allowed value for dateType: nextUpdate |
| Attribute | userDefinedMaintenanceFrequency | Maintenance period other than those defined | 0..1 | TM_PeriodDuration | Exactly one of maintenanceDate and userDefinedMaintenanceFrequency must be populated Only positive durations allowed |

### MD_MaintenanceFrequencyCode

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | MD_MaintenanceFrequencyCode | Frequency with which modifications and deletions are made to the data after it is first produced | - | S-100 is restricted to only the following values from the ISO 19115-1 codelist. The conditions for the use of a particular value are described in its Remarks |
| Value | asNeeded | Resource is updated as deemed necessary | 1 | Use only for datasets which normally use a regular interval for update or supersession, but will have the next update issued at an interval different from the usual Allowed if and only if userDefinedMaintenanceFrequency is not populated |
| Value | irregular | Resource is updated in intervals that are uneven in duration | 2 | Use only for datasets which do not use a regular schedule for update or supersession Allowed if and only if userDefinedMaintenanceFrequency is not populated |

### PT_Locale

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | PT_Locale | Description of a locale | - | - | From ISO 19115-1 |
| Attribute | language | Designation of the locale language | 1 | LanguageCode | ISO 639-2/T 3-letter language codes |
| Attribute | country | Designation of the specific country of the locale language | 0..1 | CountryCode | ISO 3166-2 2-letter country codes |
| Attribute | characterEncoding | Designation of the character set to be used to encode the textual value of the locale | 1 | MD_CharacterSetCode | UTF-8 is used in S-100 |

LanguageCode, CountryCode and MD_CharacterSetCode are codelists which are defined in resource files within the S-100 XML schemas package and described in the documentation for the S-100 XML Schemas.

### S100_SE_CertificateContainer

S-104 uses S100_SE_CertificateContainer without modification.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_SE_CertificateContainer | A set of signed public key certificates | - | - | Used in S-100 Part 17 Exchange Catalogues |
| Attribute | schemeAdministrator | The Scheme Administrator identity | 0..1 | CharacterString | The identity of the Scheme Administrator is contained in the “id” attribute of the schemeAdminstrator element. The Scheme Adminstrator certificate is NOT included in catalogue metadata as it is independently verified by the implementing system |
| Attribute | certificate | A signed public key certificate | 1..* | Base 64 encoded Character String | Conforms to X.509 encoding. Contains a digitally signed identifier of an entity |

### S100_SE_DigitalSignatureReference

S-104 uses only the ECDSA-384-SHA2 value of S100_SE_DigitalSignatureReference, in conformity with the restriction in S-100 Part 15, clauses 15-8.7 and 15-8.11.7.

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_SE_DigitalSignatureReference | Algorithm used to compute the digital signature | - | Only ECDSA is currently used in implementations of S-100 for file based transfer of data to ECDIS. Other values are included for interoperability with other implementations by external standards. See S-100 Part 15, clause 15-8.4 |
| Value | ECDSA-384-SHA2 |  | 8 | 384 bits ECDSA: SHA2-384 |

### S100_SE_DigitalSignature

S-104 conforms to S-100 Part 15, clause 15-8-11.4, which states: “The class S100_SE_DigitalSignature is realized as one of either S100_SE_SignatureOnData (a digital signature of a particular identified resource) or an additional digital signature defined using the [same class] which is either a S100_SE_SignatureOnData or S100_SE_SignatureOnSignature element as described in clause 15-8.8.  S-100 Part 17 metadata thus allows for multiple digital signatures, a single mandatory S100_SE_SignatureOnData and any number of additional signatures, either of the data or other signatures.” (In S-100, this class is not documented separately.)

S-104 uses the class S100_SE_DigitalSignature without modification; however, in S-100 exchange catalogues it is implemented by one of its subclasses S100_SE_SignatureOnData or S100_SE_SignatureOnSignature.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_SE_DigitalSignature |  | - | Base64 encoded digital signature value | See S-100 Part 15, clause 15-8 Abstract class substituted by one of its subclasses. |
| Attribute | id | Identifier of the digital signature | 1 | CharacterString | Every signature entry has a unique identifier |
| Attribute | certificateRef | Signed Public Key | 1 | CharacterString | Identifier of the certificate against which the digital signature validates |

### S100_SE_SignatureOnData

S-104 uses S100_SE_SignatureOnData without modification.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_SE_SignatureOnData |  | - | Base64 encoded digital signature value | See S-100 Part 15, clause 15-8 Subclass of S100_SE_DigitalSignature |
| Attribute | id | Identifier of the digital signature | 1 | CharacterString | Every signature entry has a unique identifier (Inherited attribute) |
| Attribute | certificateRef | Signed Public Key | 1 | CharacterString | Identifier of the certificate against which the digital signature validates (Inherited attribute) |
| Attribute | dataStatus | The digital signature | 1 | DataStatus |  |

### S100_SE_SignatureOnSignature

S-104 uses S100_SE_SignatureOnSignature without modification.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_SE_SignatureOnSignature |  | - | Base64 encoded digital signature value | See S-100 Part 15, clause 15-8 Subclass of S100_SE_DigitalSignature |
| Attribute | id | Identifier of the digital signature | 1 | CharacterString | Every signature entry has a unique identifier (Inherited attribute) |
| Attribute | certificateRef | Signed Public Key | 1 | CharacterString | Identifier of the certificate against which the digital signature validates (Inherited attribute) |
| Attribute | signatureref | The digital signature referenced | 1 |  |  |

### DataStatus

S-104 uses the S-100 enumeration DataStatus defined in S-100 Part 15 without modification.

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | DataStatus | The state of data when a digital signature is created | - |  |
| Value | unencrypted | The data is unencrypted and uncompressed | - | For example, supporting resources |
| Value | encrypted | The data is compressed and encrypted | - | For example, copy protected datasets |
| Value | compressed. | The data is compressed only | - | For example, archives of multiple resources |

### EX_GeographicBoundingBox

From ISO 19115-1.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | EX_GeographicBoundingBox | Geographic position of the dataset | - | - | Defined in ISO 19115-1: Geographic position of the resource |
| Attribute | westBoundLongitude | Western-most coordinate of the limit of the dataset extent, expressed in longitude in decimal degrees (positive east) | 1 | Real | Arc degrees |
| Attribute | eastBoundLongitude | Eastern-most coordinate of the limit of the dataset extent, expressed in longitude in decimal degrees (positive east) | 1 | Real | Arc degrees |
| Attribute | southBoundLatitude | Southern-most coordinate of the limit of the dataset extent, expressed in latitude in decimal degrees (positive north) | 1 | Real | Arc degrees |
| Attribute | northBoundLatitude | Northern-most, coordinate of the limit of the dataset extent expressed in latitude in decimal degrees (positive north) | 1 | Real | Arc degrees |

NOTE (from ISO 19115-1): This is only an approximate reference so specifying the Coordinate Reference System is unnecessary and need only be provided with a precision of up to two decimal places.

### EX_BoundingPolygon

From ISO 19115-1.

| Role Name | Name | Description | Mult | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | EX_BoundingPolygon | Boundary enclosing the dataset, expressed as the closed set of (x,y) coordinates of the polygon (last point replicates first point) | - | - | Defined in ISO 19115-1: enclosing geometric object which locates the resource, expressed as a set of (x,y) coordinate(s) |
| Attribute | polygon | Sets of points defining the bounding polygon | 1 | GM_Object | Must be a GML polygon with one exterior and 0 or more interiors expressed as Linear Rings using SRS EPSG:4326 (See S-100 Part 17) |

NOTE (from ISO 19115-1): If a polygon is used it should be closed (last point replicates first point).

Page intentionally left blank

## Carrier Metadata

The metadata for the S-104 product is divided in three sections, corresponding to the General Metadata (Table 12-1), the Feature Type Metadata (Table 12-2), and the Feature Instance Metadata (Table 12-3 and Table 12-4). The Instance Metadata is subdivided into metadata attached to the instance as a whole (Table 12-3) and metadata attached to individual values groups (Table 12-4). Since these values do not reside in the Metadata blocks in the Exchange Catalogue, but are in the HDF5 files, they are referred to as Carrier Metadata. The Carrier Metadata consists of the data and parameters needed to read and interpret the information in the Water Level product even if the other S-104 Metadata files are unavailable.

Note that in Tables 12-1 – 12-4, some of the metadata variables have restrictions on their core values (that is, whether they are optional or mandatory, the specific values allowed, etc) that are not imposed in S-100. These are grouped under the heading ‘Additional restrictions on core metadata for S-104’.

Mandatory attributes in a section of a Table that is designated for one or more specified dataCodingFormat values are mandatory only for the specified dataCodingFormat value(s).

It is suggested for any enumeration in S-104, to use unsigned integer types (preferably standard integer type H5T_STD_U8LE) for the base type of the numeric code when creating the enumeration.

Figures 12-5 through 12-9 depict the carrier metadata at each level of the structural hierarchy in an HDF5 dataset. The elements (groups and metadata) defined in S-100 are distinguished from those defined in S-104 by prefix and shade. Figure 12.5 is a summary diagram depicting all levels of the structural and their associated metadata components for all the coverage types used in S-104. Figures 12-6 –12-9 show the details for each structural level and each coverage type.

The same information as in Figures 12-6 through 12-9 is depicted in Annex B (Figures B-6 – B-7) but organised by coverage instead of levels in the HDF5 structural hierarchy.

The maximum length of all string HDF5 attributes is 300 characters.

An attribute defined at a lower level supersedes the same attribute defined at a higher level.

Figure 12.5 – Carrier metadata for the S-104 HDF5 group hierarchy

Figure 12-6 – General metadata – Carrier metadata for the root group

Figure 12-7 – Feature Type metadata – Carrier metadata for the Feature Container group

Figure 12-8 – Feature Instance metadata – Carrier metadata for the Feature Instance group

Figure 12-9 – Feature Instance metadata – Carrier metadata for the Values group in each Feature Instance group

For all carrier metadata, latitude and longitude values are precise to 10-7 degrees except where noted. All times are in UTC format.

All enumeration attributes in carrier metadata must be implemented as HDF5 enumerations. The base type for all enumeration attributes in the following tables must be 8-bit unsigned integer in the HDF5 standard integer type H5T_STD_U8LE.

Integer types are signed integers unless designated as “unsigned”.

Strings must use UTF-8 character encoding. String padding is not specified in this edition of the Product Specification due to the diversity of API framework treatment of padding.

### General metadata - details

Table 12-1 – General metadata, related to the entire HDF5 file (see S-100 Part 10c, Table 10c-6). All times are in UTC format

| No | Name | Camel Case | Mult | Data Type | Remarks and/or Units |
| --- | --- | --- | --- | --- | --- |
| 1 | Product Specification number and version | productSpecification | 1 | String | This must be encoded as ‘INT.IHO.S-104.X.Y’, with X representing the Edition number and Y the revision number. See Note 6 |
| 2 | Date of data product issue | issueDate | 1 | String | Date must be consistent with issueDate in discovery metadata |
| 3 | Horizontal Coordinate Reference System | horizontalCRS | 1 | Integer 32-bit | EPSG code (cf. clause 5.1) or -1 if user defined EXAMPLE 1: 4326 (for WGS84) EXAMPLE 2: EPSG:9057 is WGS 84 (G1762) realization with valid epoch 2005.0 |
| 4 | Bounding box | westBoundLongitude | 1 | Float 32-bit | Area encompassing all feature instances Units are Decimal Degrees in the EPSG 4326 CS. In accordance with ISO 19115-1 these coordinates need be accurate only to two decimal places |
| 5 |  | eastBoundLongitude | 1 | Float 32-bit |  |
| 6 |  | southBoundLatitude | 1 | Float 32-bit |  |
| 7 |  | northBoundLatitude | 1 | Float 32-bit |  |
| 8 | Geographic location of the resource (by description) | geographicIdentifier | 0..1 | String | Description, or location code from list agreed by data producers (In S-100: EX_Extent > EX_GeographicDescription.geographicIdentifier > MD_Identifier.code) |
| 9 | Name of the horizontal CRS | nameOfHorizontalCRS | 0..1 | String | Mandatory if horizontalCRS = -1 |
| 10 | Type of the horizontal CRS | typeOfHorizontalCRS | 0..1 | Enumeration | Mandatory if horizontalCRS = -1 See Table 12-5 |
| 11 | Horizontal coordinate system | horizontalCS | 0..1 | Integer 32-bit | Mandatory if horizontalCRS = -1 Allowed values if typeOfHorizontalCRS = 1 (Geodetic CRS 2D): - 6422 (Lat, Lon – degree) Allowed values if typeOfHorizontalCRS = 2 (Projected CRS): - 4400 (Easting, Northing – metres) - 4500 (Northing, Easting – metres) |
| 12 | Horizontal datum | horizontalDatum | 0..1 | Integer 32-bit | Mandatory if horizontalCRS = -1 EPSG code or -1 if user defined |
| 13 | Name of horizontal datum | nameOfHorizontalDatum | 0..1 | String | Mandatory if horizontalDatum = -1 |
| 14 | Prime meridian | primeMeridian | 0..1 | Integer 32-bit | Mandatory if horizontalDatum = -1; EPSG Code |
| 15 | Spheroid | spheroid | 0..1 | Integer 32-bit | Mandatory if horizontalDatum = -1; EPSG Code |
| 16 | Projection method | projectionMethod | 0..1 | Integer 32-bit | Mandatory if typeOfHorizontalCRS = 2; EPSG Code, see Table 12-7 |
| 17 | Projection parameter 1 | projectionParameter1 | 0..1 | Float 64-bit | Only if projectionMethod is used. See Table 12-7 |
| 18 | Projection parameter 2 | projectionParameter2 | 0..1 | Float 64-bit | Only if projectionMethod is used. See Table 12-7 |
| 19 | Projection parameter 3 | projectionParameter3 | 0..1 | Float 64-bit | Only if projectionMethod is used. See Table 12-7 |
| 20 | Projection parameter 4 | projectionParameter4 | 0..1 | Float 64-bit | Only if projectionMethod is used. See Table 12-7 |
| 21 | Projection parameter 5 | projectionParameter5 | 0..1 | Float 64-bit | Only if projectionMethod is used. See Table 12-7 |
| 22 | False northing | falseNorthing | 0..1 | Float 64-bit | Only if projectionMethod is used. To be applied to the coordinates at axis Northing. [m] |
| 23 | False easting | falseEasting | 0..1 | Float 64-bit | Only if projectionMethod is used. To be applied to the coordinates at axis Easting. [m] |
| 24 | Epoch of realization | epoch | 0..1 | String | Code denoting the epoch of the geodetic datum used by the CRS. For example, 2005.0 for the G1762 realization of the geodetic datum for WGS84. Must match epoch denoted by horizontalCRS. |
| Additional metadata for S-104 |  |  |  |  |  |
| 25 | Water level trend threshold | waterLevelTrendThreshold | 1 | Float 32-bit | Critical value used to determine steady water level trend. Units are metres/hour (m/hr). For example, 0.2. See clause 7.1.1 and Annex A (DCEG) |
| 26 | Dataset delivery interval | datasetDeliveryInterval | 0..1 | String | The expected time interval between availability of successive datasets for time-varying data. Must be formatted as PnYnMnDTnHnMnS (ISO 8601 duration). See Note 8 |
| 27 | Trend Interval | trendInterval | 0..1 | Integer 32-bit unsigned | The interval over which trend at a particular time is calculated Unit: minutes |
| 28 | Vertical datum epoch | verticalDatumEpoch | 0..1 | String | The period / epoch when the vertical datum was computed |
| Additional restrictions on core general metadata for S-104 |  |  |  |  |  |
| 29 | Time of data product issue | issueTime | 1 | String | Mandatory for S-104. S-100 Time format. All times are in UTC. For example 123000Z |
| 30 | Vertical coordinate system | verticalCS | 1 | Integer 32-bit | Mandatory for S-104 EPSG Code; Allowed Value 6499 (Height– Metres–Orientation Up) |
| 31 | Vertical coordinate base | verticalCoordinateBase | 1 | Enumeration | Mandatory in S-104 The only allowed value is verticalDatum (see S-100 Table 10c-22) |
| 32 | Vertical datum reference | verticalDatumReference | 1 | Enumeration | Mandatory beginning S-104 Edition 1.1 1: S-100 vertical datum |
| 33 | Vertical datum | verticalDatum | 1 | Integer 32-bit | Mandatory beginning S-104 Edition 1.1 This is one of the standard values from Table 12-8 |

NOTE 1: If the CRS is user defined only the following coordinate systems are supported:

  - Geodetic CS (Latitude, Longitude) – Degrees; and

  - Cartesian CS (Northing, Easting or Easting, Northing) – Metres.

NOTE 2: For the horizontal Datum all EPSG predefined Datums are allowed or any combination of predefined Prime Meridians or predefined Spheroids.

NOTE 3: The projection methods are limited to those given in Table 12-7.

NOTE 4: If the horizontal CRS is defined by the EPSG code, the defined CRS should not use any other elements than the one allowed for user defined CRSs; (for example, no projection method that is not in the Table).

NOTE 5: The bounding box is the data set bounding box; the coverage data feature instances may or may not cover the entire bounding box. If there is only a single coverage feature, its extent may or may not be the same as the data set.

NOTE 6: Beginning S-100 Edition 5.0.0, class S100_ProductSpecification (S-100 Part 17) contains a productIdentifier field whose value must be the Product ID value from the IHO Product Specification Register in the IHO Geospatial Information Registry. Attribute productSpecification in Table 12-1 must use exactly the same value.

NOTE 7: S-104 does not use seaSurface and seaFloor datums. which makes attribute verticalCoordinateBase redundant. It is included in order to ensure compliance with generic validation checks for attribute verticalDatum.

NOTE 8: Dataset delivery interval is encoded only if the dataset is part of a sequence delivered at known intervals (for example, daily, weekly, or 6-hourly forecasts). S-100 Part 17, clause 17-4.9 contains detailed guidance for encoding the discovery metadata equivalent of this attribute (userDefinedMaintenanceFrequency) and the same guidelines apply to encoding this attribute. If this attribute and its discovery metadata equivalent are both encoded (in the HDF5 dataset and discovery metadata block respectively), the durations encoded by them must be the same. Intervals greater than monthly may be encoded at Producer discretion.

### Feature Type metadata - details

Table 12-2 – Feature Type metadata, pertaining to the WaterLevel feature type (see S-100 Part 10c, Table 10c-10)

| No | Name | Camel Case | Mult | Data Type | Remarks and/or Units |
| --- | --- | --- | --- | --- | --- |
| 1 | Data organization index (Used to read the data. See Table 10-1) | dataCodingFormat | 1 | Enumeration | See Table 12-9 . The allowed values are: 2: Regularly-gridded arrays This Product Specification allows the use of only value 2 from S-100 |
| 2 | Dimension | dimension | 1 | Integer, 8-bit unsigned | The (spatial) dimension of the feature instances. For water levels, use 2 This is the number of coordinate axes, not the rank of the HDF5 arrays storing coordinates or values |
| 3 | Common Point Rule | commonPointRule | 1 | Enumeration | The procedure used for evaluating the coverage at a position that falls on the boundary or in an area of overlap between geometric objects 1: average 2: low 3: high 4: all (recommended) |
| 4 | Horizontal position uncertainty | horizontalPositionUncertainty | 1 | Float 32-bit | -1.0 (unknown) or positive value (m) |
| 5 | Vertical position uncertainty | verticalUncertainty | 1 | Float 32-bit | -1.0 (unknown) or positive value (m) |
| 6 | Time uncertainty | timeUncertainty | 0..1 | Float 32-bit | -1.0 (unknown) or positive value (s) |
| 7 | Number of feature instances | numInstances | 1 | Integer 32-bit unsigned |  |
| Additional metadata for S-104 |  |  |  |  |  |
| 8 | Methodology | methodWaterLevelProduct | 0..1 | String | Brief description of tide gauge type, forecast method or model, etc |
| 9 | Minimum water level height in dataset | minDatasetHeight | 1 | Float 32-bit | Height in verticalCS in Table 12-1 Use the same precision as the corresponding attribute in the values record |
| 10 | Maximum water level height in dataset | maxDatasetHeight | 1 | Float 32-bit | Height in verticalCS in Table 12-1 Use the same precision as the corresponding attribute in the values record |
| dataCodingFormat = 2 (regular Grid) |  |  |  |  |  |
| 11 | Sequencing Rule | sequencingRule.type | 1 | Enumeration | Method to be used to assign values from the sequence of values to the grid coordinates. Components: type: Enumeration CV_SequenceType Must be 1 (for ‘linear’) |
| 12 |  | sequencingRule.scanDirection | 1 | String | scanDirection: String <axisNames entry> (comma-separated). For example “latitude,longitude” implies column-major storage. See S-100 Figure 8-6. For regular grids, “longitude,latitude” or “easting,northing” are suggested as corresponding to intuitive grid point order (+ve eastward/northward). The order in axisNames and the order in sequencingRule.scanDirection are independent; scanDirection describes the storage order of the values array. |
| 13 | Interpolation Type | interpolationType | 1 | Enumeration | Interpolation method recommended for evaluation of the S100_GridCoverage Values: Only nearestneighbor is allowed from the values in the S-100 Part 8 enumeration S100_CV_InterpolationMethod. Note: This is not a recommendation for interpolation to be used by the data producer, but rather by an application reading S-104 data |
| 14 | Offset of data point in cell | dataOffsetCode | 0..1 | Enumeration | Mandatory if data points are at grid cell centres. See S-100 clauses 10c-9.6 and 8-5.2.8. The allowed values in S-104 are: 1: XMin, YMin (“Lower left”) 5: Barycenter (centroid) of cell The default is “Lower left” and this attribute may be omitted if data points are at cell lower-left corners. Other cell corners are not allowed. |

### Feature Instance metadata - details

Table 12-3 – Feature Instance metadata, pertaining to the feature instance (see S-100 Part 10c, Table 10c-12). All times are in UTC format

| No | Name | Camel Case | Mult | Data Type | Remarks and/or Units |
| --- | --- | --- | --- | --- | --- |
| 1 | Bounding box | westBoundLongitude | 0..1 | Float 32-bit | Area of grid The unit must conform to the CRS used for the dataset (for example, degrees for the geographic 2D CRS EPSG 4326; and metres for the UTM zone projected CRS EPSG 32710). Either this or the domainExtent dataset must be populated. If the grid extents for different feature instances overlap, the domain extent polygon must be provided instead. |
| 2 |  | eastBoundLongitude | 0..1 | Float 32-bit |  |
| 3 |  | southBoundLatitude | 0..1 | Float 32-bit |  |
| 4 |  | northBoundLatitude | 0..1 | Float 32-bit |  |
| 5 | Number of time records | numberOfTimes | 1 | Integer 32-bit unsigned | The total number of time records. Mandatory in S-104. For water level adjustment to be non-trivial, there should be multiple time records (an acceptable minimum is undetermined at this time). |
| 6 | Time interval | timeRecordInterval | 0..1 | Integer 16-bit unsigned | The interval between time records. Units: Seconds. Recommended if the interval between successive values is uniform. |
| 7 | Valid time of earliest value | dateTimeOfFirstRecord | 1 | String | DateTime format. First record in the Instance. All times are in UTC Mandatory in S-104 |
| 8 | Valid time of latest value | dateTimeOfLastRecord | 1 | String | DateTime format Mandatory in S-104. |
| 9 | Number of values groups | numGRP | 1 | Integer 32-bit unsigned | Number of Values Groups. For dataCodingFormat = 2 equals the number of time points given by numberOfTimes |
| Additional metadata for S-104 |  |  |  |  |  |
| 10 | Data dynamicity | dataDynamicity | 1 | Enumeration | See Table 12-10. The allowed values are: 1: Observation 2: Astronomical prediction 3: Analysis or hybrid method 5: Hydrodynamic model forecast |
| 11 | Vertical datum epoch | verticalDatumEpoch | 0..1 | String | The period / epoch when the vertical datum was computed |
| dataCodingFormat = 2 (regular Grid) |  |  |  |  |  |
| 12 | Longitude of grid origin | gridOriginLongitude | 1 | Float-Double (64-bit) | As per CRS |
| 13 | Latitude of grid origin | gridOriginLatitude | 1 | Float-Double (64-bit) | As per CRS |
| 14 | Grid spacing, longitudinal | gridSpacingLongitudinal | 1 | Float-Double (64-bit) | As per CRS |
| 15 | Grid spacing, latitudinal | gridSpacingLatitudinal | 1 | Float-Double (64-bit) | As per CRS |
| 16 | Number of points, longitudinal | numPointsLongitudinal | 1 | Integer 32-bit unsigned | numCOLS |
| 17 | Number of points, latitudinal | numPointsLatitudinal | 1 | Integer 32-bit unsigned | numROWS |
| 18 | Start sequence | startSequence | 1 | String | For example, “0,0” (without quotes) for lower left. For upper left, “0,n”, where n is the value of numROWS-1. First value represents first axis in sequencingRule.scanDirection |
| 19 | Vertical datum reference | verticalDatumReference | 0..1 | Enumeration | Mandatory if and only if the vertical datum of this instance differs from the datum in the root group, omitted if it is the same. 1: S-100 vertical datum |
| 20 | Vertical datum | verticalDatum | 0..1 | Integer 32-bit | Mandatory if and only if the vertical datum of this instance differs from that in the root group, omitted if it is the same. This is one of the standard values from Table 12-8 |

### Values Group attributes - details

An expanded new metadata block is required for the Values Groups (Table 12-4) to add attributes for specifying the parameters used for calculating trend values.

Table 12-4 – Values Group attributes (see S-100 Part 10c, Table 10c-19). All times are in UTC format

| No | Name | Camel Case | Mult | Data Type | Remarks and/or Units |
| --- | --- | --- | --- | --- | --- |
| 1 | Time stamp | timePoint | 1 | String | DateTime. All times are in UTC. See Clause 10.2.2.5 |
| 2 | Water level trend threshold | waterLevelTrendThreshold | 0..1 | Float 32-bit | Critical value used to determine steady water level trend. Units are metres/hour (m/hr). For example, 0.2. See Annex A (DCEG) Supersedes waterLevelTrendThreshold defined at the root group level |
| 3 | Trend Interval | trendInterval | 0..1 | Integer 32-bit unsigned | The interval over which trend at a particular time is calculated Unit: minutes. Default: 60 minutes Supersedes trendInterval defined at the root group level |

### Additional enumerations used in carrier metadata

Table 12-5 – Type of the horizontal CRS

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | typeOfHorizontalCRS | Codes for describing the type of the two-dimensional horizontal CRS | - |  |
| Literal | geodeticCRS2D | Two-dimensional geodetic CRS | 1 |  |
| Literal | projectedCRS | Projected CRS | 2 |  |

Table 12-6 – Vertical datum reference

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | verticalDatumReference |  | - |  |
| Literal | s100VerticalDatum | The vertical datum is one of those listed in S100_VerticalAndSoundingDatum | 1 |  |

Table 12-7 – Projection methods and their parameters

| Name | EPSG Code | Parameter 1 | Parameter 2 | Parameter 3 | Parameter 4 | Parameter 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Mercator | 9805 | Latitude of 1st standard parallel | Longitude of natural origin | - | - | - |
| Transverse Mercator | 9807 | Latitude of natural origin | Longitude of natural origin | Scale factor at natural origin | - | - |
| Oblique Mercator | 9815 | Latitude of projection centre | Longitude of projection centre | Azimuth of initial line | Angle from Rectified to Skew Grid | Scale factor on initial line |
| Hotline Oblique Mercator | 9812 | Latitude of projection centre | Longitude of projection centre | Azimuth of initial line | Angle from Rectified to Skew Grid | Scale factor on initial line |
| Lambert Conic Conformal (1SP) | 9801 | Latitude of natural origin | Longitude of natural origin | Scale factor at natural origin | - | - |
| Lambert Conic Conformal (2SP) | 9802 | Latitude of false origin | Longitude of false origin | Latitude of 1st standard parallel | Latitude of 2nd standard parallel | - |
| Oblique Stereographic | 9809 | Latitude of natural origin | Longitude of natural origin | Scale factor at natural origin | - | - |
| Polar Stereographic | 9810 | Latitude of natural origin | Longitude of natural origin | Scale factor at natural origin | - | - |
| Krovak Oblique Conic Conformal | 9819 | Latitude of projection centre | Longitude of projection centre | Azimuth of initial line | Latitude of pseudo standard parallel | Scale factor on pseudo standard parallel |
| American Polyconic | 9818 | Latitude of natural origin | Longitude of natural origin | - | - | - |
| Albers Equal Area | 9822 | Latitude of false origin | Longitude of false origin | Latitude of 1st standard parallel5 | Latitude of 2nd standard parallel6 | - |
| Lambert Azimuthal Equal Area | 9820 | Latitude of natural origin | Longitude of natural origin | - | - | - |

Table 12-8 – S100_VerticalAndSoundingDatum

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| S100_Codelist | S100_VerticalAndSoundingDatum | Allowable vertical and sounding datums | - | S-104 allows only the standard values of this codelist, which makes it effectively an enumeration for S-104 purposes. The values seaFloor, seaSurface, and hydrographicZero are not used in S-104. |
| Value | meanLowWaterSprings | The average height of the low waters of spring tides. This level is used as a tidal datum in some areas. | 1 | (MLWS) |
| Value | meanLowerLowWaterSprings | The average height of lower low water springs at a place. | 2 | - |
| Value | meanSeaLevel | The average height of the surface of the sea at a tide station for all stages of the tide over a 19-year period, usually determined from hourly height readings measured from a fixed predetermined reference level. | 3 | (MSL) |
| Value | lowestLowWater | An arbitrary level conforming to the lowest tide observed at a place, or somewhat lower. | 4 | - |
| Value | meanLowWater | The average height of all low waters at a place over a 19-year period. | 5 | (MLW) |
| Value | lowestLowWaterSprings | An arbitrary level conforming to the lowest water level observed at a place at spring tides during a period of time shorter than 19 years. | 6 | - |
| Value | approximateMeanLowWaterSprings | An arbitrary level, usually within 0.3m from that of Mean Low Water Springs (MLWS). | 7 | - |
| Value | indianSpringLowWater | An arbitrary tidal datum approximating the level of the mean of the lower low water at spring tides. It was first used in waters surrounding India. | 8 | - |
| Value | lowWaterSprings | An arbitrary level, approximating that of mean low water springs (MLWS). | 9 | - |
| Value | approximateLowestAstronomicalTide | An arbitrary level, usually within 0.3m from that of Lowest Astronomical Tide (LAT). | 10 | - |
| Value | nearlyLowestLowWater | An arbitrary level approximating the lowest water level observed at a place, usually equivalent to the Indian Spring Low Water (ISLW). | 11 | - |
| Value | meanLowerLowWater | The average height of the lower low waters at a place over a 19-year period. | 12 | (MLLW) |
| Value | lowWater | The lowest level reached at a place by the water surface in one oscillation. | 13 | (LW) |
| Value | approximateMeanLowWater | An arbitrary level, usually within 0.3m from that of Mean Low Water (MLW). | 14 | - |
| Value | approximateMeanLowerLowWater | An arbitrary level, usually within 0.3m from that of Mean Lower Low Water (MLLW). | 15 | - |
| Value | meanHighWater | The average height of all high waters at a place over a 19-year period. | 16 | (MHW) |
| Value | meanHighWaterSprings | The average height of the high waters of spring tides. | 17 | (MHWS) |
| Value | highWater | The highest level reached at a place by the water surface in one oscillation. | 18 | (HW) |
| Value | approximateMeanSeaLevel | An arbitrary level, usually within 0.3m from that of Mean Sea Level (MSL). | 19 | - |
| Value | highWaterSprings | An arbitrary level, approximating that of mean high water springs (MHWS). | 20 | - |
| Value | meanHigherHighWater | The average height of higher high waters at a place over a 19-year period. | 21 | (MHHW) |
| Value | equinoctialSpringLowWater | The level of low water springs near the time of an equinox. | 22 | - |
| Value | lowestAstronomicalTide | The lowest tide level which can be predicted to occur under average meteorological conditions and under any combination of astronomical conditions. | 23 | (LAT) |
| Value | localDatum | An arbitrary datum defined by a local harbour authority, from which levels and tidal heights are measured by this authority. | 24 | - |
| Value | internationalGreatLakesDatum1985 | A vertical reference system with its zero based on the mean water level at Rimouski/Pointe-au-Père, Quebec, over the period 1970 to 1988. | 25 | - |
| Value | meanWaterLevel | The average of all hourly water levels over the available period of record. | 26 | - |
| Value | lowerLowWaterLargeTide | The average of the lowest low waters, one from each of 19 years of observations. | 27 | - |
| Value | higherHighWaterLargeTide | The average of the highest high waters, one from each of 19 years of observations. | 28 | - |
| Value | nearlyHighestHighWater | An arbitrary level approximating the highest water level observed at a place, usually equivalent to the high water springs. | 29 | - |
| Value | highestAstronomicalTide | The highest tidal level which can be predicted to occur under average meteorological conditions and under any combination of astronomical conditions. | 30 | (HAT) |
| Value | balticSeaChartDatum2000 | Baltic Sea Chart Datum 2000 | 44 | - |
| Value | internationalGreatLakesDatum2020 | The 2020 update to the International Great Lakes Datum, the official reference system used to measure water level heights in the Great Lakes, connecting channels, and the St Lawrence River system | 46 | Unlike the previous two IGLDs, this datum update will use a geoid-based vertical datum that will be accessible using global navigation satellite systems (GNSS) such as the Global Positioning System (GPS) |
| Value | nationalLowWaterDatum | A low water datum defined by a National authority, from which levels, clearances and tidal heights are measured by this authority. | 50 | - |

The descriptions are reproduced verbatim from the IHO GI Registry as of August 2026. Note that the list of permitted datums is constrained by the need to match datums permitted by S-102.

Table 12-9 – S100_HDF_DataCodingFormat

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S100_HDF_‌DataCodingFormat | Data coding formats for S-100 HDF5 data | - | S-104 does not use movingPlatform, irregularGrid, or variableCellSize data coding formats |
| Value | regularGrid | Data at grid points forming a regular grid with constant cell spacing | 2 | Regular grids are commonly composed of perpendicularly crossing lines of equal spacing on each dimension, creating square or rectangular cells |

Table 12.10 – S104_DataDynamicity

| Item | Name | Description | Code | Remarks |
| --- | --- | --- | --- | --- |
| Enumeration | S104_DataDynamicity | Classification of data according to the relationship between the time of its collection, generation, or calculation of generation parameters, in relation to the time of publication of the dataset | - |  |
| Value | observation | Values from in-situ sensor(s); may be quality controlled and stored after collection | 1 | Only real-time observations See also Notes 1 and 2 below |
| Value | astronomicalPrediction | Values computed using harmonic analysis or other proven method of tidal analysis | 2 | IHO Resolution 3/1919, as amended |
| Value | analysisOrHybrid | Values calculated by statistical or other indirect methods, or a combination of methods | 3 | A hybrid method combines two or more approaches |
| Value | hydrodynamicForecast | Values calculated from a two- or three-dimensional dynamic simulation of future conditions using predicted data for boundary forcing, via statistical method or combination | 5 | A forecast is a simulation made for many hours into the future using predicted winds, water levels, etc |

NOTE 1: The time period covered by the observations should be encoded in the metadata attribute temporalExtent.

NOTE 2: Sensors (for example tide gauges deployed along a channel) are monitored by the data collecting Authority. After data acquisition, the data are quality controlled and stored by the Producing Authority.

See clause 7.1 for detailed descriptions of the types of water level data based on the time-dependence of the source.

## Language

The language used for the Discovery Metadata and the Carrier Metadata is English. Producers may add translations of Feature Catalogues into additional languages using ‘language packs’ (cf. S-100 Part 18 and Part 11, clauses 11.2.4 – 11.2.5).

Page intentionally left blank

# ANNEX A – Data Classification and Encoding Guide

- A-1	Features

Water Level (WaterLevel)

| IHO Definition: FEATURE: WATER LEVEL: The vertical position of a water surface |  |  |  |  |
| --- | --- | --- | --- | --- |
| S-104 Geo Feature: Water Level |  |  |  |  |
| Primitives: coverage |  |  |  |  |
| S-104 Attribute | Allowable Encoding Value | Type | HDF5 File Type | Multiplicity |
| Water Level Height | Must be in decimal metres, maximum resolution of 0.01 metres | RE | H5T_IEEE_F32LE | 1 |
| Water Level Trend | 1 : Decreasing 2 : Increasing 3 : Steady | EN | HDF5 enumeration type with 8-bit unsigned integer as base type | 1 |
| Uncertainty | Must be in decimal metres, maximum resolution of 0.01 metres | RE | H5T_IEEE_F32LE | 0..1 |

Remarks:

- S-104 Attribute Water Level Time (waterLevelTime) is included in the Feature Catalogue for ECDIS use only and must not be present in the HDF5 dataset. In ECDIS, S-104 data is available to the user only via query (pick report), and the time of S-104 data used in water level adjustment is critical information for the user. Water Level Time in the Feature Catalogue provides a space for the ECDIS to auto-generate an attribute to contain timepoint information and be made available to the user via pick report.

- A-2	Feature Attributes

The number of attributes for Water Level is three: water level height, water level trend, and uncertainty. Encoding remarks are included with each attribute specification, below.

- Water Level Height (waterLevelHeight)

Water Level Height: The height of a water surface relative to a vertical datum

Unit: Metre (m)

Maximum Resolution: 0.01 m

Format: xxx.xx

Example: 10.54

Remarks: - Land mask or missing value is denoted by a unique number as specified in the metadata. - The height is relative to some vertical datum, which is defined in the metadata. - 0.01 metres equals 0.3937 inches (1 cm).

- Water Level Trend (waterLevelTrend)

Water Level Trend: The tendency of water level to change in a particular direction.

1 : Decreasing (decreasing) 2 : Increasing (increasing) 3 : Steady (steady) Unit: none (enumeration) Resolution: N/A (enumeration) Format: x Example: 3 (Steady) Remarks: - To determine category, use metadata variable waterLevelTrendThreshold (See Table 12-1): - Decreasing: trend <= -waterLevelTrendThreshold - Increasing: trend >= +waterLevelTrendThreshold - Steady: -waterLevelTrendThreshold < trend < +waterLevelTrendThreshold Where a value is not known, the fill value must be populated, which is 0 (Unknown). The fill value may be used in non-tidal or similar regions. - Native integer type H5T_STD_U8LE should be used for the base type of the numeric code (1, 2, or 3 here) when creating the enumeration.

- Uncertainty (uncertainty)

Uncertainty: Estimate characterising the range of values within which the true value of a measurement is expected to lie as defined within a particular confidence level. It is expressed as a positive value.

Unit: Metre (m)

Resolution: 0.01 metre

Format: xx.xx

Example: 0.02

Remarks: - Represents a +/- value defining the possible range of associated water level height, expressed as a positive number. For example, 0.02 represents +/- 0.02 m uncertainty in water level height. - Represents the uncertainty at a particular grid point and may be omitted if the uncertainty is the same at all grid points. - If the water level height for a particular values record is the fill value, the uncertainty in that record must be the uncertainty fill value.

# ANNEX B – S-104 Comprehensive Model Including Application Schema and Carrier Metadata (UML Diagrams)

Figure B-1 depicts the various components of the S-104 model. The Meta-features and Exchange Set Metadata components are empty because S-104 does not define any meta-features or extend S-100 Exchange Catalogue classes. Figure B-2 depicts the derivation of the S-104 packages from various S-100 components.

Figure B-1 – S-104 Model components

Figure B-2 – Derivations from S-100

Figure B-3 depicts the coverage types used in S-104 and their realizations from the conceptual coverages in S-100 Part 8 and ISO 19123. This is a more detailed version of Figure 4-3. Note that the realizations are not directly from the S-100 Part 8 and ISO 19123 classes, but via corresponding notional classes for the HDF5 implementations of the various data coding formats (not included in this diagram). For example, S104_RegularGrid is a notional extension of a notional S-100 class Part10c::S100_HDF_RegularGrid which encapsulates the encoding of dataCodingFormat 2 in HDF5. The notional classes are omitted to reduce diagram clutter.

The implementation of most attributes in the S-100 Part 8 model by S-100 Part 10c closely follows the names and types of the attributes. Certain attributes in the S-100 Part 8 and ISO 19123 models are simplified in S-100 Part 10c HDF5 implementation, as follows:

- HDF5 Regular Grid (data coding formats 2) implements S100_IF_Grid:

  - The attribute origin is implemented in the form of two HDF5 attributes, gridOriginLatitude and gridOriginLongitude.

  - The attribute offsetVectors is implemented in the form of two HDF5 attributes, gridSpacingLongitudinal and gridSpacingLatitudinal.

The rangeType attribute common to all coverage types is implemented implicitly in the S-100 Feature Catalogue’s binding of attributes to a feature and in the name/datatype information in feature information datasets in the feature information group (S-100 Part 10c, Table 10c-8).

Figure B-3 Spatial Types – Coverages with Realizations from S-100 Part 8 and ISO 19123

Figure B-4 depicts the domain features. The WaterLevel feature and its attributes realise the values record in the S-100 Part 8 and ISO 19123 coverages models.

Figure B-4 – Domain Objects

Figure B-5 depicts the external catalogue metadata classes (the same information as Figure 12-4).

Figure B-6 depicts the same information as Figures 12-6 through 12-9, organised by coverage type instead of structural level. Different levels in the HDF5 structure (root, feature type, feature instance, and value) are indicated by backgrounds of different colours.

Figure B-5 – Exchange Set class details

Figure B-6 – All carrier metadata for coverage type Regular Grid (data coding format 2)

# ANNEX C – Feature Catalogue

C-1 	Meta Feature Types

None.

C-2 	Geo Feature Types

C-2.1 	Water Level

Definition: The vertical position of a water surface.

CamelCase: WaterLevel

Alias:

Super type:

Feature use type: geographic

Primitive: coverage

Remarks: Water Level Time is included in the Feature Catalogue for ECDIS use only and must not be present in the HDF5 dataset.

Attribute Bindings:

| S-104 Attribute | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- |
| Water Level Height |  | RE | 1,1 |
| Water Level Trend | 1 : Decreasing 2 : Increasing 3 : Steady | EN | 1,1 |
| Uncertainty |  | RE | 0,1 |

C-3 	Cartographic Feature Types

None.

C-4 	Information Types

None.

C-5 	Simple Attributes

C-5.1 	Water Level Height

Definition: The height of a water surface relative to a vertical datum.

CamelCase: waterLevelHeight

Alias:

Value type: real

Remarks: No remarks.

Unit of measure name: Metre    definition: The basic unit of length in the International System of Units (SI) system.    symbol: m

Quantity specification: length

Constraints:

| String Length | Text Pattern | Range |  | Precision |
| --- | --- | --- | --- | --- |
| (not specified) | (none) | lowerBound | -99.99 | 2 |
|  |  | upperBound | 99.99 |  |
|  |  | closure | closedInterval |  |

C-5.2 	Water Level Trend

Definition: The tendency of water level to change in a particular direction.

CamelCase: waterLevelTrend

Alias:

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Decreasing | Becoming smaller in magnitude. |
| 2 | Increasing | Becoming larger in magnitude. |
| 3 | Steady | Constant. |

C-5.3 	Uncertainty

Definition: Estimate characterising the range of values within which the true value of a measurement is expected to lie as defined within a particular confidence level. It is expressed as a positive value.

CamelCase: uncertainty

Alias:

Value type: real

Remarks: Represents a +/- value defining the possible range of associated depth, expressed as a positive number.

Unit of measure name: Metre    definition: The basic unit of length in the International System of Units (SI) system.   symbol: m

Quantity specification: length

Constraints:

| String Length | Text Pattern | Range |  | Precision |
| --- | --- | --- | --- | --- |
| (not specified) | (none) | lowerBound | 0.00 | 2 |
|  |  | upperBound | 99.99 |  |
|  |  | closure | closedInterval |  |

C.5.4. Water Level Time

Definition: The time of the water level height, expressed in Date-time format as specified by ISO 8601.

CamelCase: waterLevelTime

Alias:

Value type: dateTime

Remarks:

- Unit: Years, months, days, hours, minutes, seconds; Resolution: 1 second; Format: YYYYMMDDTHHMMSSZ, where Y is year, M is month, D is day, H is hour, M is minute, and S is second; Example: 19850412T101530Z denotes 10 hours, 15 minutes, and 30 seconds on 12 April 1985.

- This attribute is included in the feature catalogue for ECDIS use only and must not be encoded in datasets.

Constraints:

| String Length | Text Pattern | Range | Precision |
| --- | --- | --- | --- |
| (not specified) | (((((19)¦(20))\d{2})¦(21([0-4]\d)))(1[0-2]¦0[1-9])(3[01]¦0[1-9]¦[12][0-9])T(2[0-3]¦[01][0-9]):?([0-5][0-9]):?([0-5][0-9])Z)¦(21500101T000000Z) |  |  |

C-6 	Complex Attributes

None.

C-7 	Roles

None.

C-8 	Information Associations

None.

C-9 	Feature Associations

None.

C-10 	Feature Catalogue XML

The feature catalogue is provided as a separate XML file and can be downloaded from the IHO GI Registry.

Page intentionally left blank

# ANNEX D – Use Cases

## D-1	German Water Level Data and Forecast

### D-1.1	Summary

| Name: | German S-104 Water Level Data and Forecast. (Courtesy: BSH - Bundesamt für Seeschifffahrt und Hydrographie). |
| --- | --- |
| Description: | The usage of S-104 format for data delivery and the operational regular data transfer in near real time allows several applications. The optimised water level forecast builds the basis for safe navigation, especially in areas with strong tides, as the inner German Bight. The S-104 data may be used for route planning, allowing to adapt to changing water depth. Optimised water level forecasting leads to better route monitoring as well as route planning and a more efficient loading of ships. Furthermore, the potential data transfer on ships helps for direct navigation of a vessel. It will allow a more efficient usage of narrow shipping channel or areas with high traffic density. The main users are shipping companies and administration, pilots, and organisations for harbour and waterway management. Further applications are possible in fields of civil protection, protection of marine environment, support of offshore and coastal activities, as well as coastal engineering. |
| Potential Actors: | Navigators, Marine Pilots, Shipping Companies, Vessel Control Centres, Ship and Harbour Managements, Port Authorities, BSH (Bundesamt für Seeschifffahrt und Hydrographie), WSV (Wasserstraßen- und Schifffahrtsverwaltung des Bundes). |
| Potential Applications: | - Route monitoring. - Route planning and forecast of safety corridor. - Other applications as civil protection, protection of marine environment, support of offshore and coastal activities, as well as coastal engineering. |
| Data Requirements: | - High quality water level forecast (with adequate spatial and temporal resolution). - Astronomical prediction. - Near real time observational data. |
| Technical Aspects and Post- Processing: | - Different kinds of water level data are stored in one file. Portrayal of real time observation, optimised water level forecast and astronomic prediction with the help of a WMS. - Transformation of all kinds of data. - Route planning with respect to forecasted water level. |

### D-1.2	Additional details

### D-1.2.1	Types of data used to create S-104 test datasets

- Operational Numerical Forecast Model Output

  - Hydrodynamic model forecast for Elbe Estuary / German Bight

  - 90m x 90m grid spacing / 900m x 900m grid spacing

  - Forecast interval: 48h, 15 Minutes time step

  - 1155 x 728 grid points / 1030 x 1761 grid points

  - Update: 2 times daily

- Observation

  - Discrete observations from gauge stations

  - Time spacing: 1 minute

  - Update: near real time

- Astronomical prediction

  - Forecast for discrete locations

  - Long range prediction

  - Update: yearly

- Model output statistics [MOS]

  - Statistical method for optimizing numerical model forecast

  - Available for some locations

  - Forecast up to 7 days

  - Time spacing: 15 Minutes

  - Update: every 15 Minutes

### D-1.2.2	Data processing

Numerical model output and model output statistics (MOS) at several stations have been combined for an optimised areal water level forecast. The method uses the surface shape of a hydrodynamic model forecast and corrects it with high accurate MOS forecasts at specific locations. This leads to an overall optimised and frequently updated water level surface forecast.

### D-1.2.3	S-104 data transformation

The data described above, especially the post processed data, builds the basis for the creation of S-104 test data sets. The transformation of different formats are achieved by a tuple of tools:

- S-104 regular grid converter

  - Input: Hydrodynamic model forecast (MOS-corrected)

  - Output: S-104 conformant HDF5 file

- S-104 station data converter

  - Input: Observation, Astronomic Prediction, MOS-Data, Numerical Model data (MOS-corrected) for discrete locations

  - Output: single S-104 HDF5 file

### D-1.2.4	Results

Figure D-1 shows one application of S-104 datasets; a WMS to display the S-104 data for the use case pick report. The request pops up a figure displaying the data described above. The green line represents the observation, blue the astronomical prediction and black represents the hydrodynamic model forecast optimised with MOS. The purple line shows the difference between prediction and hydrodynamic model forecast.

Figure D-1 – Pick report for S-104 test data

As a practical example, S-104 datasets were produced as a suitable format to deliver different kinds of water level data in one file. A regular data transfer via FTP has been established and the S-104 data has been integrated in a WMS for a highly frequented region. The “pick report” portrayal has been implemented.  It has been confirmed by consensus of a user workshop that the display of water level data as a pick report like specified in S-104 is good practice.

## D-2	Depth Adjustment in ECDIS

### D-2.1	Summary

| Name: | Depth adjustment in ECDIS. |
| --- | --- |
| Description: | S-104 data may be used for route planning, in combination with S-101 ENC and S-102 Bathymetric Surface data to provide navigation officers with dynamic water depth information for the purpose of route planning. This will allow efficient use of waterways with tidal or other dynamic variations of water levels, saving vessels transit time and fuel costs. The main users are commercial vessels and pilots. |
| Potential Actors: | Navigators, Marine Pilots, Hydrographic Offices. |
| Potential Applications: | - Route planning and assessments of safe depths. - Safety contours according to dynamic depths instead of the static contours currently provided by ENC data alone. |
| Data Requirements: | - High quality water level forecast (with adequate spatial and temporal resolution). - Astronomical prediction. - Near real time observational data. - S-102 bathymetry data (with adequate spatial resolution). - Availability of underlying ENC data. - Vertical datums in S-102 and S-104 data should match. |
| Technical Aspects and Post- Processing: | - S-104 data must be provided as a continuous coverage (data coding format 2 (regular grid), 3 (ungeorectified grid) or 7 (TIN)). - ECDIS must implement S-98 Annex C interoperability in order to integrate S-101, S-102, and S-104 data. - ECDIS must have functionality to implement route planning with respect to forecasted water levels. |

### D-2.2	Additional details

### D-2.2.1	Types of data

S-104 data, available as forecasted data in grid format covering the projected time of the transit and the planned route.

S-102 data, available in grid format covering the planned route.

Relevant vessel parameters (draught, squat) are available to the ECDIS.

### D-2.2.2	Processing

The ECDIS is used in route planning mode and the navigator plots the planned route or imports it from a S-421 dataset.

The navigator sets the projected start time of the transit and the route parameters such as speed over ground, cross-track deviation, etc.

The navigator selects a time interval based on the speed of the vessel and the time record interval between S-104 records that is encoded in the S-104 forecast data (see Table 12-3).

Adjustment zones are created by the ECDIS based on the route schedule, speed, and a buffer (calculated either as a multiple of cross-track deviation or a user-specified value). The extent of each adjustment zone depends on the speed and cross-track buffer.

Figure D-2 – Adjustment zones based on planned route (courtesy: Furuno Electric Company, Ltd)

The ECDIS computes an “integrated and dynamic safety contour” in the region covered by the combined adjustment zones, computed from bathymetry data adjusted by the dynamic water levels extracted from the S-104 record(s) corresponding to the time the vessel is transiting each adjustment zone.

Names of measures other than attributeValueUncertaintyMean are planned corrections by the DQWG to the names listed in S-100 Edition 5.2.0 Part 4c.

The display format for time is controlled by ECDIS application settings.

There will usually be multiple underlying ENCs with different scale ranges, which will ideally use the same CRS and vertical datums. If not, the ENC Producer(s) should be consulted about possible ENC update plans and the appropriate ENC to which water level information should conform.

This requirement allows applications to retrieve data value records for a geographic position by retrieving values from different WaterLevel instances in the same HDF5 file using the same array indexes for the values arrays. Except on datum jump boundaries, at most one of the retrieved value records will be populated with actual (non-fill) values.

This requirement means that boundaries of vertical datum areas must be clipped by the grid’s geographic boundaries when the datum area extends beyond a grid boundary. Depending on the relationship between shape of the datum area and grid boundaries, such clipping may result in a multi-polygon, which must then be split into separate WaterLevel feature instances.

The values for scale in this table and in discovery metadata are the denominator of the scale. For example “12000” means a scale of 1:12000.

Since cancellations cannot always be predicted, this requirement obviously cannot be put into effect until the cancellation arrives on the system.

This Product Specification does not mandate maintenance of temporal continuity between cancelled and replacement datasets. External factors such as production constraints, producers’ own data standards or ECDIS performance standards may be determinative and must be taken into account.

Producer Codes may be obtained from the IHO Producer Code Register in the IHO GI Registry. The four-character S-100 “Alpha” codes must be used.

Exceptions: (1) Producer Codes must use the same case as the IHO Producer Code Register. (2) A name component taken from an external Specification, must follow the rules in that Specification (for example, “20190703T00Z” for a time component in ISO 8601 basic format, not “20190703t00z”).

See the guidance on HDF5 datatypes (https://support.hdfgroup.org/archive/support/HDF5/Tutor/datatypes.html, retrieved 30 July 2026) for more information on the use of standard vs native types when creating a dataset and for memory operations (read/write).

The “procedure” referred to means only evaluate operations as defined in ISO 19123 (“accept a DirectPosition as input and return a set of Records of feature attribute values for that direct position”). Applications must apply their own evaluation methods to the resulting list, for example, “water level adjustment” in ECDIS (S-98 Annex C) should select the water level value that produces the shoalest depth at the position.

S-100 lists permitted numeric codes and descriptions in Part 10c (Table 10c-10) but does not define labels. Pending definition of an enumeration in S-100, the encoding should use the labels “LowerLeft” and “Barycenter” for codes 1 and 5 respectively. The labels “UpperRight”, “LowerRight”, “UpperLeft” may be used with codes 2, 3, and 4 to define the HDF5 enumeration data type but must not be encoded as values of the dataOffsetCode attribute. A validation check in S-158:104 will enforce this constraint.